***Sub Engineer***

1. Which one of the following mistakes/errors may be cumulative + or - :

a. bad ranging

b. bad straightening

c. erroneous length of chain

d. sag.

2. The angle of intersection of a contour and a ridge line, is

a. 30°

b. 45°

c. 60°

d. 90°.

3. For the construction of highway (or railway)

a. longitudinal sections are required

b. cross sections are required

c. both longitudinal and cross sections are required

d. none of these.

4. If h is the difference in level between end points separated by l, then the slope correction is  . The second term may be neglected if the value of h in a 20 m distance is less than

a.  m

b. 1 m

c. 2 m

d. 3 m

5. Cast steel is manufactured by

a. Cementation process

b. Crucible process

c. Bessemer process

d. Open hearth process.

6. Based on flow quality, the sequence of pipes is

a. A.C. pipes, G.I. pipes, C.I. pipes, PVC pipes

b. C.I. pipes, G.I. pipes, A.C. pipes, PVC pipes

c. C.I. pipes, G.I. pipes, PVC pipes, A.C. pipes

d. None of these.

7. In paints, the pigment is responsible for

a. colour

b. smoothness

c. glassy face

d. none of these.

8. The coefficient of hardness of stones used in road work should be greater than

a. 10

b. 12

c. 15

d. 17.

9. For calculating the allowable stress of long columns. The empirical formula , is known as

a. Straight line formula

b. Parabolic formula

c. Perry's formula

d. Rankine's formula.

10. Flat spiral springs

a. are supported at outer end

b. are wound by applying a torque

c. are used in clock-work mechanism

d. all the above.

11. If the width of a simply supported beam carrying an isolated load at its centre is doubled, the deflection of the beam at the centre is changed by

a. 4 times

b. 8 times

c. 1/2 times

d. 3 times.

12. If a rectangular beam measuring 10 x 18 x 400 cm carries a unformly distributed load such that the bending stress developed is 100 kg/cm2. The intensity of the load per metre length, is

a. 250 kg

b. 260 kg

c. 270 kg

d. 280 kg.

13. Hydraulic grade line

a. remains above the centre line of conduit

b. remains below the centre line of conduit

c. remains parallel to the centre line of conduit

d. may be above or below the centre line of conduit.

14. An open container filled with water is moved vertically upward with a uniform linear acceleration. The pressure at its bottom will be

a. greater than static pressure

b. equal to static pressure

c. lesser than static pressure

d. none of these.

15. In an inclined pipe, the pressure difference at its two ends is due to

a. exit head drop

b. frictional loss head

c. elevation head

d. all the above.

16. Equation of continuity of fluids is applicable only if

a. flow is compressive

b. flow is one dimensional

c. velocity is uniform over the cross-sections

d. all the above.

17. Failure of a slope occurs only when total shear force is

a. equal to total shearing strength

b. greater than total shearing strength

c. less than total shearing strength

d. none of these.

18. A sample of saturated soil has 30% water content and the specific gravity of soil grains is 2.6. The dry density of the soil mass in g/cm3, is

a. 1.47

b. 1.82

c. 1.91

d. none of these.

19. The angle of internal friction is maximum for

a. angular-grained dense sand

b. round-grained dense sand

c. round-grained loose sand

d. clays.

20. A partially saturated soil is classified as

a. one phase soil

b. two phase soil

c. three phase soil

d. four phase soil.

21. To ensure that the hogging bending moment at two points of suspension of a pile of length L equals the sagging moment at its centre, the distances of the points of suspension from either end, is

a. 0.107 L

b. 0.207 L

c. 0.307 L

d. 0.407 L

22. A circular slab subjected to external loading, deflects to form a

a. semi-hemisphere

b. ellipsoid

c. parabolloid

d. none of these.

23. Design of R.C.C. cantilever beams, is based on the resultant force at

a. fixed end

b. free end

c. mid span

d. mid span and fixed support.

24. If the permissible compressive and tensile stresses in a singly reinforced beam are 50 kg/cm2 and 1400 kg/cm2 respectively and the modular ratio is 18, the percentage area At of the steel required for an economic section, is

a. 0.596%

b. 0.696%

c. 0.796%

d. none of these.

25. Pick up the correct statement from the following :

a. the roof slabs of multi-storeyed buildings are constructed monolithically to carry the various floor loads

b. the beams of multi-storeyed buildings rest on girders and are the main load transferring members to the columns

c. the slab is spanned across the secondary beams provided between the main beams

d. All of these.

26. Pick up the correct statement from the following :

a. the first coat of stucco plaster is called scratch coat

b. the second coat of stucco plaster is called brown coat

c. the third coat of stucco plaster is called white coat

d. all the above.

27. Pick up the incorrect statement from the following:

a. The process of changing cement paste into hard mass, is known as setting of cement

b. The phenomenon by virtue of which cement does not allow transmission of sound, is known as soundness of cement

c. The heat generated during chemical reaction of cement with water, is known as heat of hydration

d. None of these.

28. Bulking of sand is

a. mixing of different sizes of sand particles

b. mixing of lime with sand

c. maximum water with sand

d. swelling of sand when wetted.

29. The transitional middle portion of a logistic curve follows

a. a geometric growth

b. a logarithmic growth

c. a first over curve

d. a constant rate.

30. Pick up the correct statement from the following :

a. The pH value of neutral water is 7

b. The maximum acidity is obtained when pH value is zero

c. The maximum alkalinity is obtained when pH value is 14.

d. All the above.

31. The intensity of rain is expressed in

a. cm/minute

b. cm/hour

c. cm/day

d. none of these.

32. Pick up the incorrect statement from the following :

a. The process of decomposing the organic matter under controlled anaerobic conditions, is called sludge digestion

b. Sludge digestion is carried out in sludge tank

c. The gases produced in sludge digestion process, contain 75% carbon dioxide

d. The gases produced in sludge digestion process, contain 75% methane.

33. When a canal is carried over a natural drainage, the structure provided, is known as

a. syphon

b. aqueduct

c. super passage

d. syphon-aqueduct.

34. Pick up the incorrect statement from the following:

a. In free flooding irrigation, water is admitted at one corner of a field and is allowed to spread over the entire area

b. In check method of irrigation, the field is divided into smaller compartments and water is admitted to each in turn

c. In furrow irrigation water is admitted between the rows of plants in the field

d. None of these.

35. The intensity of irrigation means

a. percentage of culturable commanded area to be irrigated annually

b. percentage of gross commanded area to be irrigated annually

c. percentage of the mean of culturable commanded area and the gross commanded area to be irrigated annually

d. total depth of water supplied by the number of waterings.

36. The field capacity of a soil depends upon

a. capillary tension in soil

b. porosity of soil

c. both (a) and (b)

d. neither (a) nor (b).

37. Width of a rotary round should be equal to

a. twice the width of narrowest radial road

b. width of the widest road

c. width of the widest road plus the width of one lane

d. width of the widest road plus 2 metres.

38. For night travel, the length of a valley curve should be such that the head light beam distance is the same as

a. stopping sight distance

b. overtaking sight distance

c. sum of (a) and (b)

d. difference of (a) and (b)

39. Pick up the correct statement from the following:

a. Curve length should be at least 150 metres for a deflection angle of 5 degress

b. For every degree decrease in the deflection angle, 30 metre length of curve to be increased

c. If the deflection angle is less than 1°, no curve is designed

d. All the above.

40. Pick up the correct statement from the following:

a. Detailed survey is carried out for a strip of land about 30 m at sharp curves

b. Levels are taken along the trace cut at an interval of 20 m

c. Contour interval is generally adopted at 2 metres vertical interval

d. All the above.

41. The volume is measured correct to the nearest

a. 0.01 cum

b. 0.02 cum

c. 0.03 cum

d. 0.04 cum

42. The measurement is not made in square metres in case of

a. D.P.C. (Damp proof course)

b. Form works

c. Concrete Jaffries

d. R.C. Chhajja.

43. The most reliable estiamte is

a. Detailed estimate

b. Preliminary estimate

c. Plinth area estimate

d. Cube rate estimate

44. Brick walls are measured in sq. m if the thickness of the wall is

a. 10 cm

b. 15 cm

c. 20 cm

d. None of these.

45. PERT analysis is based on

a. optimistic time

b. pessimistic time

c. most likely time

d. all the above.

46. The Overall in-charge of an organisation at the site responsible for the execution of the works, is

a. Engineer

b. Junior Engineer

c. Sub overseer

d. Assistant Engineer.

47. The main principle of an organisation, is

a. cohegency

b. effective control at all levels

c. delegation of authority

d. all the above.

48. The artificial activity which indicates that an activity following it, cannot be started unless the preceding activity is complete, is known as

a. event

b. free float

c. dummy

d. constant

49. Pick up the correct statement from the following:

a. The wind data of an air port is depicted in the form of a chart known as wind rose

b. The landing and take off of the air craft is made against the wind direction

c. The product of wind loading and power loading is called air craft rating

d. All the above.

50. Total correction for elevation, temperature and gradient for a runway should not be more than

a. 20%

b. 25%

c. 30%

d. 35%