***Sub Engineer***

1. One of the tacheometric constants is additive, the other constant, is

a. subtractive constant

b. multiplying constant

c. dividing constant

d. indicative constant.

2. The least count of a vernier scale is

a. sum of the smallest divisions of main and vernier scales

b. value of one division of the primary scale divided by total number of divisions of vernier scale

c. value of one division of vernier scale divided by total number of divisions of primary scale

d. none of these.

3. Probable systematic error in precise levelling as recommended by International Geodetic Association should not exceed (where k is in kilometers.)

a. ± 0.1 k mm

b. ± 0.2 k mm

c. ± 0.1 k

d. 0.2 k mm.

4. While measuring a chain line between two stations A and B intervened by a raised ground

a. vision gets obstructed

b. chaining gets obstructed

c. both vision and chaining get obstructed

d. all the above.

5. Pick up the correct statement from the following:

a. Attrition test is done to find out the rate of wear of stones which are used in road construction

b. Crushing test is done to find out the compressive strength of the stone

c. Impact test is done to determine toughness of a stone

d. All the above.

6. The frog of a brick is normally made on its

a. top face

b. bottom face

c. longer face

d. shorter side.

7. For the manufacture of plywood, veneers are placed so that grains of adjacent veneers

a. run at right angles

b. parallel

c. inclined at 45°

d. inclined at 60°.

8. Stainless steel resists corrosion due to

a. sulphur

b. vanadium

c. chromium

d. maganese.

9. A steel rod of sectional area 250 sq. mm connects two parallel walls 5 m apart. The nuts at the ends were tightened when the rod was heated to 100°C. If αsteel = 0.000012/C°, Esteel = 0.2 MN/mm2, the tensile force developed at a temperature of 50°C, is

a. 80 N/mm2

b. 100 N/mm2

c. 120 N/mm2

d. 150 N/mm2

10. Maximum principal stress theory for the failure of a material at elastic point, is known

a. St. Venant's theory

b. Rankine's theory

c. Haig's theory

d. Von Mises's theory.

11. If a rectangular beam measuring 10 x 18 x 400 cm carries a unformly distributed load such that the bending stress developed is 100 kg/cm2. The intensity of the load per metre length, is

a. 250 kg

b. 260 kg

c. 270 kg

d. 280 kg.

12. The ratio of the flexural strengths of two square beams one placed with its two sides horizontal and the other placed with one diagonal vertical, diagonal, is

a. 2

b. 2

c. 5

d. 7

13. An open container filled with water is moved vertically downward with a uniform linear acceleration. The pressure at its bottom will be

a. greater than static pressure

b. equal to static pressure

c. lesser than static pressure

d. none of these.

14. To measure very low pressure, we use

a. barometers

b. piezometers

c. manometers

d. differential manometers.

15. Unit of kinematic viscosity is

a. m2/sec

b. Newton sec/m2

c. Newton sec/m3

d. Kg sec/m2.

16. Pick up the correct statement from the following :

a. The fluids which follow  are known as Newtonian fluids

b. The fluids which do not follow the linear relationship between shear and rate of strain are known as non Newtonian fluids

c. The substances which flow after yield strains, are known as plastics

d. all the above.

17. For a homogeneous earth dam 50 m high having 2 m free broad, a flow net was constructed and the results were : Number of potential drops = 2.4 Number of flow channels = 0.4. If coefficiency of permeability of the dam mateiral is 3 x 10-3 cm3/sec, the discharge per metre length of dam, is

a. 12 x 10-5 m3/sec

b. 24 x 10-3 m3/sec

c. 6 x 10-5 m3/sec

d. 24 x 10-5 m3/sec

18. For shear strength, triaxial shear test is suitable because

a. precise measurement of the pore pressure and volume change during the test is possible

b. stress distribution on the failure plane, is uniform

c. state of stress within the specimen during any stage of the test as well as at failure, is completely determined

d. all the above.

19. Rankine's theory of active earth pressure assumes

a. ground surface is a plane which may be horizontal or inclined

b. back of the wall is vertical and smooth

c. wall yields about the base

d. all the above.

20. The liquidity index is defined as a ratio expressed as percentage of

a. plastic limit minus the natural water content, to its plasticity index

b. natural water content minus its plastic limit to its plasticity index

c. natural water content plus its plastic limit to its plasticity index

d. liquid limit minus the natural water content to the plasticity index.

21. Total pressure on the vertical face of a retaining wall of height h acts parallel to free surface and from the base at a distance of

a. h/4

b. h/3

c. h/2

d. 2h/3

22. The maximum shear stress (qmax) in a rectangular beam is

a. 1.50 times the average

b. 1.75 times the average

c. 2.0 times the average

d. 2.5 times the average.

23. A pre-stressed concrete member

a. is made of concrete

b. is made of reinforced concrete

c. is stressed after casting

d. possesses internal stresses.

24. An R.C.C. roof slab is designed as a two way slab if

a. it supports live loads in both directions

b. the ratio of spans in two directions is less than 2

c. the slab is continuous over two supports

d. the slab is discontinuous at edges.

25. The minimum thickness of walls built in cement mortar (1 : 6) for a single storey building, is

a. 15 cm

b. 20 cm

c. 25 cm

d. 30 cm.

26. The maximum permissible differential settlement, in case of foundations in clayey soil, is usually limited to

a. 20 mm

b. 30 mm

c. 40 mm

d. 50 mm.

27. For preparing ordinary concrete, the quantity of water used, is

a. 10% by weight of aggregates plus 10% of weight of cement

b. 5% by weight of aggregates plus 30% of weight of cement

c. 30% by weight of aggregates plus 10% of weight of cement

d. none of these.

28. Concrete is unsuitable for compaction by a vibrator if it is

a. earth moist

b. semi-plastic

c. plastic

d. none of these.

29. The bed slope in slow sand filters, is generally kept

a. 1 in 50

b. 1 in 75

c. 1 in 100

d. 1 : 200

30. The permissible amount of nitrites present in potable water, is

a. 15 ppm

b. 20 ppm

c. 45 ppm

d. Nil.

31. The sewer which transports the sewage to the point of treatment, is called

a. out-fall sewer

b. branch sewer

c. lateral

d. main sewer.

32. 3.0 ml of raw sewage is diluted to 300 ml. The D.O. concentration of the diluted sample at the beginning of the test was 8 mg/l. After 5 day-incubation at 20°C, the DO. concentration was 5 mg/l. The BOD of raw sewerage is

a. 100 mg/l

b. 200 mg/l

c. 300 mg/l

d. 400 mg/l.

33. The discharge over a trapezoidal crest of a Sarda fall with free over fall conditions, is

a. 0.45 2g . LH3/2 (H/B)1/3

b. 0.45 2g . LH2/3 (H/B)2/3

c. 0.45 2g . LH3/2 (H/B)1/6

d. 0.45 2g . LH1/6 (H/B)3/2

34. Pick up the correct statement from the following:

a. Gravity water is harmful to crops

b. Hygroscopic water remains attached to soil molecules by chemical bond

c. Capillary moisture held in the soil pores against gravity by surface tension, is utilised by plants

d. All the above.

35. The optimum depth of kor watering for a rice crop, is

a. 19.0 cm

b. 17.5 cm

c. 13.5 cm

d. 12.0 cm

36. In a barrage, the crest level is kept

a. low with large gates

b. high with large gates

c. high with no gates

d. low with no gates.

37. For a poorly graded sub-grade soil, thickness of sub-base, is

a. 15 cm

b. 20 cm

c. 25 cm

d. 30 cm

38. One-way streets are generally provided in crowded cities as, these

a. reduce delays to vehicles

b. permit higher speed

c. reduce the number of accidents

d. all the above.

39. Minimum permissible speed on high speed roads, is decided on the basis of

a. 15 percentile cumulative frequency

b. 20 percentile cumulative frequency

c. 30 percentile cumulative frequency

d. 40 percentile cumulative frequency.

40. Selection of the routes, of highways depends upon

a. avoidance of cutting hard rocks

b. minimum number of bridges

c. minimum possible length of alignment

d. all the above.

41. The expected out turn of cement concrete 1 : 2 : 4 per mason per day is

a. 1.5 m3

b. 2.5 m3

c. 3.5 m3

d. 5.0 m3

42. Pick up the correct statement regarding the centre line method of estimating a building

a. Product of the centre line of the walls and area of cross-section of any item, gives total quantity of the item

b. The centre line is worked out separately for different sections of walls of a building

c. The centre line length is reduced by half the layer of main wall joining the partition wall

d. All the above.

43. Pick up the correct statement from the following:

a. Additional supervising staff engaged at work site, is called work charged establishment

b. Detailed specifications specify qualities, quantities and the proportions of materials to be used for a particular item

c. The cost per unit at which the article can be procured, from the open market at a given time, is called 'market rate'

d. All the above.

44. The 'centre line method' is specially adopted for estimating

a. Hexagonal buildings

b. Octagonal buildings

c. Other geometrical shaped buildings

d. All the above.

45. The difference between the time avail-to do a job and the time required to do the job, is known as

a. event

b. float

c. duration

d. constraint.

46. For the supply of materials for concrete, form work reinforcing and placing of concrete, removal of form work and curing of concrete, number of bar(s) required on bar chart, is

a. 1

b. 2

c. 3

d. 4

47. Completion of an activity on CPM network diagram, is generally known

a. Event

b. Node

c. Connector

d. All the above.

48. A construction schedule is prepared after collecting

a. output of labour

b. output of machinery

c. quantity of various items

d. all the above.

49. An aircraft is flying in an atmosphere of 30°C with a speed of 1260 km ph. Its speed is known as

a. subsonic

b. sonic

c. super-sonic

d. Mach.

50. Pick up the correct statement from the following:

a. The distance between the points of intersection of the extreme tangents to the transition curve is kept greater than 7500 m x sum of grade changes at the point of intersection

b. The rate of change of grade is limited to 0.3% per 30 m length of the curve

c. According to I.C.A.O. the maximum longitudinal gradient along a runway is limited to 1.5%

d. All the above.