***Sub Engineer***

1. Tacheometric formula for horizontal distances using horizontal sights can also suitable be employed for inclined sights through θ by multiplying

a. the constants by cos2 θ

b. the constants by cos θ

c. the constants by sin θ

d. the multiplying constant by cos2 θ and additive constant by cos θ.

2. The most reliable method of plotting a theodolite traverse, is

a. by consecutive co-ordinates of each station

b. by independent co-ordinates of each station

c. by plotting included angles and scaling off each traverse leg

d. by the tangent method of plotting.

3. Pick up the correct statement from the following :

a. mistakes arise from inattention, inexperience or carelessness

b. systematic errors persist and have regular effects in the survey performances

c. accidental errors occur inspite of every precaution is taken

d. all the above.

4. Pick up the correct statement from the following :

a. the eyepiece plays no part in defining the line of sight

b. the diaphragm plays no part in defining the line of sight

c. the optical centre of the objective plays no part in defining the line of sight

d. none of these.

5. The minimum compressive strength of 1st class bricks should be

a. 90 kg/cm2

b. 100 kg/cm2

c. 120 kg/cm2

d. 130 kg/cm2.

6. The percentage of water for normal consistency, is

a. 10% to 25%

b. 15% to 25%

c. 20% to 30%

d. 25% to 35%.

7. Stones used for rubble masonry must be

a. soft

b. hard

c. light

d. heavy.

8. Softer variety of steel may be obtained by

a. Cementation process

b. crucible process

c. Bessemer process

d. Open hearth process.

9. In plastic analysis, the shape factor for a circular section, is

a. 1.5

b. 1.6

c. 1.7

d. 1.75

10. Stress may be expressed in Newtons

a. per millimetre square (N/mm2)

b. per centimetre square (N/cm2)

c. per metre square (N/m2)

d. All of the above.

11. The point of contraflexture occurs in

a. continuous beams only

b. over hanging beams only

c. all types of beams

d. both (a) and (b).

12. The deflection of any rectangular beam simply supported, is

a. inversely proportional to its width

b. inversely proportional to the cube of its depth

c. directly proportional to the cube of its length

d. none of these.

13. The dimensions MLT-2 refers to

a. specific weight

b. force

c. discharge

d. none of these.

14. The ratio of the percentagge error in the discharge and percentage error in the measurement of head over a triangular notch, is

a. 2/3

b. 3/2

c. 2/5

d. 5/2

15. Unit of kinematic viscosity is

a. m2/sec

b. Newton sec/m2

c. Newton sec/m3

d. Kg sec/m2.

16. Poise is the unit of

a. viscosity

b. velocity gradient

c. mass density

d. kinematic viscous.

17. If there is no impervious boundary at the bottom of a hydraulic structure, stream lines tend to follow :

a. a straight line

b. a parabola

c. a semi-ellipse

d. a semi-circle.

18. The fundamental equation of specific gravity (G), dry density (γd), unit weight of water (γω) and void ratio (e), is

a. e =

b. G =

c. γd =

d. γω =

19. Pick up the correct statement from the following:

a. Silty clayloam contains highest percentage of silt

b. Stiff boulder clay offers maximum shear strength

c. Soft chalk carries least safe load

d. All the above.

20. Pick up the correct statement from the following:

a. Illite bond is stronger than montmorillonite bond

b. Illites do not swell when wet

c. Illites are composed of two silica tetrahedralsheets with a central octahedral sheet

d. All the above.

21. In the zone of R.C.C. beam where shear stress is less than 5 kg/cm2, nominal reinforcement is provided at a pitch of

a. one-half lever arm of the section

b. one-third lever arm of the section

c. lever arm of the section

d. one and half lever arm of the section.

22. Top bars are extended to the projecting parts of the combined footing of two columns L distance apart for a distance of

a. 0.1 L from the outer edge of column

b. 0.1 L from the centre edge of column

c. half the distance of projection

d. one-fourth the distance of projection.

23. If Ac, Asc and A are areas of concrete, longitudinal steel and section of a R.C.C. column and m and σc are the modular ratio and maximum stress in the configuration of concrete, the strength of column is

a. σcAc + m σcAsc

b. σc(A - Asc) + m σcAsc

c. σc[A + (m - 1)ASC]

d. all the above.

24. The live load to be considered for an inaccessible roof, is

a. Nil

b. 75 kg/m2

c. 150 kg/cm2

d. 200 kg/m2

25. The additional piles which are driven to increase the capacity of supporting loads on vertical piles, are known

a. raking piles

b. eccentric piles

c. sinking piles

d. none of these.

26. The vertical side member of a shutter frame, is known

a. style

b. reveal

c. mullion

d. post.

27. Pick up the incorrect statement from the following:

a. A rich mix of concrete possesses higher strength than that a lean mix of desired workability with excessive equantity of water

b. The strength of concrete decreases as the water cement ratio increases

c. If the water cement ratio is less than 0.45, the concrete is not workable and causes honey-combed structure

d. Good compaction by mechanical vibrtions, increases the strength of concrete

28. 1.S. Sieve Nos. 10 mm and 4.75 mm are geneally used for grading of

a. fine aggregates

b. neither (a) nor (b)

c. both (a) and (b)

d. none of these.

29. Yield of a drainage basin is :

a. run off of the area expressed as instantaneous rate

b. average run off over a short period

c. total volume of water flowing annually

d. all the above.

30. Distribution mains of any water supply, is normally designed for its average daily requirement

a. 100%

b. 150%

c. 200%

d. 225%.

31. A well oxidized sewage contains nitrogen mainly as

a. nitrates

b. nitrites

c. free ammonia

d. none of these.

32. Flocculated particles do not change their

a. size

b. shape

c. weight

d. none of these.

33. A hydraulic jump is generally formed when a stream moving with

a. a hyper-critical velocity meets a stream moving with a critical velocity

b. a hyper-critical velocity meets a stream moving with a hyper-critical velocity

c. A hyper-critical velocity meets a stream moving with a sub-critical velocity

d. a sub-critical velocity meets a stream moving with a hyper-critical velocity.

34. If average particle size of the silt in millimetres is m, the Lacey's silt factor f is proportional to

a. m3

b. m1/2

c. m1/3

d. m2/3

35. Effective precipitation for a crop may be defined as

a. total precipitation minus the loss due to evaporation

b. total precipitation minus the loss due to infiltration

c. total precipitation during the crop period

d. available water stored in soil within root zone of the crop.

36. A river training work is generally required when the river is

a. meandering

b. aggrading

c. degrading

d. all the above.

37. The wall constructed for the stability of an excavated portion of a road on the hill side, is known as

a. retaining wall

b. breast wall

c. parapet wall

d. all the above.

38. If the cross slope of a country is 25% to 60%, the terrain is classified as

a. plain

b. rolling

c. steep

d. mountainous.

39. If the designed speed on a circular curve of radius 1400 m is 80 km/hour, no superelevation is provided, if the camber, is

a. 4%

b. 3%

c. 2%

d. 1.7%

40. Roadway width for a National highways and State highways (two-lanes) is

a. 12 m

b. 9 m

c. 9.5 m

d. 15 m

41. Carpet area does not include the area of

a. verandah, corridor and passage

b. bath room and lavatory

c. kitchen and pantry

d. All the above.

42. Pick up the item of work not included in the plinth area estimate

a. Room area

b. Verandah area

c. W.C. area

d. Courtyard area.

43. The rate of payment is made for 100 cu m (per % cu m) in case of

a. Rock cutting

b. Excavation in trenches for foundation

c. Earth work in filling the plinth

d. all the above.

44. Pick up the excavation where measurements are made in square metres for payment.

a. surface dressing up to 15 cm depths

b. Surface excavation up to 30 cm depths

c. Both (b) and (c)

d. Both (a) and (b)

45. The first stage of a construction, is

a. survey of the site

b. initiation of proposal

c. preparation of tender

d. allotment of funds.

46. The salient feature of functional organisation is

a. separation of planning and design part

b. each individual maintains functional efficiency

c. work is properly planned and distributed

d. all the above.

47. Which one of the following represents an activity

a. curing of concrete

b. setting of question paper

c. preparation of breakfast

d. all the above.

48. Construction team means

a. an architect

b. an owner

c. a contractor

d. all the above.

49. Effective length of a runway is the distance between

a. ends of the runway

b. point of intersection of the obstruction clearance line and the extended plane of the runway surface, and the other end of the runway

c. point of intersection of the glide path and the extended plane of the runway surface and the other end of the runway

d. ends of the clear way on either side.

50. Pick up the correct statement from the following:

a. The wind data of an air port is depicted in the form of a chart known as wind rose

b. The landing and take off of the air craft is made against the wind direction

c. The product of wind loading and power loading is called air craft rating

d. All the above.