***Sub Engineer***

1. The reduced level of a floor is 99.995 m, the staff reading on the floor is 1.505 m. If the inverted staff reading against the roof is 1.795 m, the floor level below the slab, is

a. 3.290 m

b. 3.300 m

c. 3.275 m

d. 2.790 m

2. Diaphragm of a surveying telescope is held inside

a. objective

b. telescope tube at its mid point

c. telescope at the end nearer the eye-piece

d. telescope at its end nearer the objective.

3. True meridian of different places

a. converge from the south pole to the north pole

b. converge from the north pole to the south pole

c. converge from the equator to the poles

d. run parallel to each other.

4. Surveys which are carried out to provide a national grid of control for preparation of accurate maps of large areas, are known

a. plane surveys

b. geodetic surveys

c. geographical surveys

d. topographical surveys.

5. The melting point of silica is :

a. 1570°C

b. 1630°C

c. 1730°C

d. 1850°C.

6. Pick up the correct statement regarding low heat cement from the following:

a. Its initial setting time is about one hour

b. Its final setting time is about 10 hours

c. Its mainly used for mass concrete work

d. All the above.

7. The plastics prepared from Vinyl resin are

a. non-toxic

b. transparent

c. colourless

d. all of these.

8. Ground glass

a. is made by grinding its one side

b. is made by melting powdered glass paints surface

c. is used for getting light without transparency

d. all the above.

9. Pick up the correct statement from the following:

a. Hoop strain of the walls of a cylinder due to liquid is

b. Longitudinal strain in the walls of a cylinder due to liquid is

c. Volumetric change in the cylinderdue to liquid is

d. All the above.

10. For determining the force in the member AB of the truss shown in the given figure by method of sections, the section is made to pass through AB, AD and ED and the moments are taken about

a. joint C

b. joint B

c. joint D

d. joint A

11. For a beam having fixed ends, the unknown element of the reactions, is

a. horizontal components at either end

b. vertical components at either end

c. horizontal component at one end and vertical component at the other

d. horizontal and vertical components at both the ends.

12. A simply supported beam (l + 2a) with equal overhangs (a) carries a uniformly distributed load over the whole length, the B.M. changes sign if

a. l > 2a

b. l < 2a

c. l = 2a

d. l = 4a

13. In two dimensional flow the components of velocity are given by u = ax; v = by. The stream lines will be

a. circular

b. parabolic

c. hyperbolic

d. elliptical.

14. Pick up the correct statement from the following :

a. For maximum velocity of flow, the depth of water in the circular channel must be 0.81 times the diameter of the channel

b. For maximum velocity, the hydraulic mean depth must be 0.3 times the diameter of circular channel

c. For maximum discharge the depth of flow must be 0.95 times the diameter of circular channel

d. All the above.

15. If v1 and v2 are the velocities of flow before and after sudden enlargement in a pipe, the head loss given by Carnot and Borda equation, is

a. (v22 - v12)/2g

b. (v1 - v2)2/2g

c. (v2 - v1)2/2g

d. (v1 + v2)2/2g.

16. If the Mach number for a fluid flow is less than 1, the flow is

a. sonic

b. supersonic

c. sub-sonic

d. none of these.

17. A fundamental equation of void ratio(e), specific gravity (G), water content (ω) and degree of saturation (Sr) is

a. e =

b. ω =

c. G =

d. Sr =

18. The compressibility of clays, is caused due to:

a. expulsion of double layer water from in between the grains

b. sliping of particles to new positions of greater density

c. bending of particles as elastic sheets

d. all the above.

19. Pick up the correct statement from the following:

a. In hydrometer method, weight Wd per ml of suspension is found directly

b. In pipette analysis, weight Wd per ml of suspension is found indirectly

c. In pipette analysis, weight Wd per ml of suspension is found directly

d. None of these.

20. The ratio of the weight of water to the weight of solids in a given mass of soil, is known

a. porosity

b. specific gravity

c. void ratio

d. water content.

21. A flat slab is supported

a. on columns

b. on beams and columns

c. on columns monolithicaily built with slab

d. all the above

22. Design of a two way slab simply supported on edges and having no provision to prevent the corners from lifting, is made by

a. Marcus formula

b. Rankine Grashoff formula

c. Grashoff formula

d. Rankine-Marcus formula.

23. The reinforced concrete beam which has width 25 cm, lever arm 40 cm, shear force 6t/cm2, safe shear stress 5 kg/cm2 and B.M. 24 mt,

a. is safe in shear

b. is unsafe in shear

c. is over safe in shear

d. needs redesigning.

24. The thickness of base slab of a retaining wall generally provided, is

a. one-third of the width of the stem at the bottom

b. one fourth of the width of the steam at the bottom

c. width of the stem at the bottom

d. twice the width of the steam at the bottom.

25. The type of bond in which every course contains both headers and stretchers, is called

a. English bond

b. Flemish bond

c. Russian band

d. Mixed bond.

26. The process of working a flat for the finishing coat, is known

a. dubbing out

b. floating

c. knetting

d. blistering.

27. Water cement ratio is

a. weight of water to that of cement

b. weight of concrete to that of water

c. volume of concrete to that of water

d. both (a) and (b) of the above.

28. For the construction of cement con-certe dams, the maximum permissible size of the aggregates, is

a. 40 mm

b. 50 mm

c. 60 mm

d. 70 mm

29. The chlorine supply cylinders are generally kept at 38°C to 40°C to prevent

a. conversion into crystals

b. it from burning

c. it from explosion

d. none of these.

30. Raw water treated with only chlorine, is known as

a. plain chlorination

b. pre-chlorination

c. first-chlorination

d. de-chlorination

31. The sewer which resists sulphide corrosion, is

a. Cast iron sewer

b. R.C.C. sewer

c. Lead sewer

d. None of these.

32. The disintegrating pump in which solid matter is broken up before passing out, is

a. centrifugal pump

b. reciprocating pump

c. pneumatic ejector

d. none of these.

33. Retrogression of the bed level of a river downstream a weir, occurs due to

a. heavy impact of water

b. increase of the bed level

c. less percentage of silt

d. soft soil strata.

34. According to Khosla, the exist gradient of surface flow

a. depends upon the b/d ratio

b. is independent of the b/d ratio

c. is independent of the depths of d/s cut off walls

d. none of these.

35. The most suitable section of a lined canal, is

a. trapezoidal section with rounded corners for large canals

b. rectangular section with rounded corners for large canals

c. both (a) and (b)

d. both (a) and (c)

36. The length and width of a meander and also the width of the river, vary roughly as

a. square root of the discharge

b. discharge

c. square of the discharge

d. cube of the discharge.

37. Width of vehicles affects the width of

a. lanes

b. shoulders

c. parking spaces

d. all the above.

38. Increase in traffic volume, due to increase in transport vehicles, is known as

a. normal traffic growth

b. generated traffic growth

c. current traffic

d. none of these.

39. Along high ways confirmatory route markers are generally fixed

a. before the crossing on the left side

b. after the crossing on the left side

c. before the crossing on the right side

d. after the crossing on the right side.

40. First operation during the detailed survey of a hill road, is

a. adjustment of alignment along with curves

b. derivation of longitudinal and cross-sections

c. fixation of Bench Marks

d. staking the ground.

41. Size, capacity and materials need be specified for

a. Stop-cocks

b. Ferrules

c. Ball valves

d. All the above.

42. The area of a sloping surface of a protective embankment of mean height d, side slopes S : 1 and length L is

a. d x d x s

b. d2 x (ds)2

c. L.D 1 + s2

d. 2 LD 1 + s2

43. The ground surface slopes 1 in 50 along a proposed railway embankment 150 m in length. The height of the embankment at zero chainage is 0.5 m, the width is 11 m and side slopes 2:1. If the falling gradient of the embankment is 1 in 150, the quantity of the earthwork calculated by prismoidal formula, is

a. 3250 m3

b. 3225 m3

c. 3275 m3

d. 3300 m3

44. For 12 mm thick cement plastering 1 : 6 on 100 sq.m new brick work, the quantity of cement required, is

a. 0.200 m3

b. 0.247 m3

c. 0.274 m3

d. 0.295 m3

45. A CPM family includes

a. CPP (Critical Path Plotted)

b. MCE (Minimum Cost Expenditure)

c. CPS (Critical Path Scheduling)

d. All the above.

46. Pick up the correct statement from the following:

a. Latest occurrence time is denoted by TL

b. Contractual obligation time is denoted by Ts

c. Latest occurrence time is taken as contractual obligation time

d. All the above.

47. If D is the duration, ES and EF are the earliest start and finish, LS and LF are latest start and latest finish time, then the following relation holds good

a. LS = LF - D

b. LF = LS + D

c. D = EF - ES

d. all the above.

48. Completion of an activity on CPM network diagram, is generally known

a. Event

b. Node

c. Connector

d. All the above.

49. In instrument landing system

a. L.O.M. and L.M.M. are installed on one side and Localizer antenna on the other side

b. L.O.M. and Localizer are installed on one side and L.M.M. on the other side

c. Localizer and L.M.M. are installed on one side and L.O.M. on the other side

d. None of these.

50. Pick up the correct statement from the following:

a. The basic length of a runway is increased at a rate of 7% per 300 m of elevation of M.S.L.

b. The standard temperature at the site is obtained by reducing the standard sea level temperature of 15°C at the rate of 6.5°C per 1000 m rise in elevation

c. The aerodrome reference temperature is the monthly mean of the mean daily temperature for the hottest month of the year

d. All the above.