***Sub Engineer***

1. Pick up the correct statement from the following :

a. the handles are on swivel joints to prevent twisting of the chain

b. the fifth tag from either end of the chain is numbered 5

c. the length of the Gunter's chain is 66 ft.

d. all the above.

2. Under ordinary conditions, the precision of a theodolite traverse is affected by

a. systematic angular errors

b. accidental linear errors

c. systematic linear errors

d. accidental angular errors.

3. Pick up the correct statement from the following :

a. mistakes arise from inattention, inexperience or carelessness

b. systematic errors persist and have regular effects in the survey performances

c. accidental errors occur inspite of every precaution is taken

d. all the above.

4. Profile levelling is usually done for determining

a. contours of an area

b. capacity of a reservoir

c. elevations along a straight line

d. boundaries of property

5. Pick up the correct statement from the following:

a. The commercial plywoods are available upto 150 cm wide and upto 300 cm long

b. The plywoods possess uniform tensile strength in all directions

c. The plywoods are light in weight

d. All the above.

6. Chlorite, a green colour mineral is mainly derived from the decomposition of

a. augite

b. biotite

c. horn blende

d. All of these.

7. Invar contains

a. 12% of nickel

b. 24% of nickel

c. 30% to nickel

d. 36% of nickel.

8. Bulking of sand is caused due to

a. surface moisture

b. air voids

c. viscosity

d. clay contents

9. A composite beam is composed of two equal strips one of brass and other of steel. If the temperature is raised

a. brass experiences compressive force

b. composite beam gets subjected to a couple

c. composite beam bends

d. All the above.

10. The assumption in the theory of bending of beams, is :

a. material is isotropic

b. Young's modulus is same in tension as well as in compression

c. each layer is independent to expand or to contract

d. all the above.

11. The ratio of the flexural strengths of two square beams one placed with its two sides horizontal and the other placed with one diagonal vertical, diagonal, is

a. 2

b. 2

c. 5

d. 7

12. Beams of uniform strength are preferred to those of uniform section because these are economical for

a. large spans

b. heavy weights

c. light weights

d. short spans.

13. For a most economical rectangular channel, the hydraulic mean depth, is equal to

a. the depth of flow

b. half the depth of flow

c. one-third depth of flow

d. None of these.

14. In C.G.S. system the unit of visocity is

a. joule

b. poise

c. Newton

d. none of these.

15. To measure very low pressure, we use

a. barometers

b. piezometers

c. manometers

d. differential manometers.

16. Pick up the correct statement regarding Borda's mouth piece running full from the following :

a. Theoretical velocity at the outlet is 2gH

b. Coefficient of velocity is 1/2

c. Coefficient of contraction is 1

d. All the above.

17. The compressibility of clays, is caused due to:

a. expulsion of double layer water from in between the grains

b. sliping of particles to new positions of greater density

c. bending of particles as elastic sheets

d. all the above.

18. A partially saturated sample of soil has a unit weight of 2.0 g/cm3 and specific gravity of soil particles is 2.6. If the moisture content in the soil is 20%, the degree of saturation is

a. 20%

b. 77%

c. 92%

d. none of these.

19. The triaxial apparatus is usually used for

a. unconsolidated-undrained test

b. consolidated-undrained test

c. drained test

d. all the above tests.

20. If a soil undergoes a change in shape and volume by application of external loads over it, but recovers its shape and volume immediately after removal of the load, the property of the soil is said to be

a. Resilience of soils

b. Elasticity of soils

c. Compressibility of soils

d. None of these.

21. If the size of a column is reduced above the floor, the main bars of the columns, are

a. continued up

b. bent inward at the floor level

c. stopped just below the floor level and separate lap bars provided

d. all the above.

22. Dimensions of a beam need be changed if the shear stress is more than

a. 10 kg/cm2

b. 15 kg/cm2

c. 20 kg/cm2

d. 25 kg/cm2

23. For initial estimate for a beam design, the width is assumed

a. 1/20th of span

b. 1/25th of span

c. 1/30th of span

d. 1/40th of span.

24. The radius of a bar bend to form a hook, should not be less than

a. twice the diameter

b. thrice the diameter

c. four times the diameter

d. five times the diameter

25. The vertical member running through middle of a shutter frame, is

a. style

b. reveal

c. mullion

d. post.

26. The form work from the slabs excluding props, can be removed only after

a. 1 day

b. 4 days

c. 7 days

d. 14 days

27. Under normal conditions using an ordinary cement, the period of removal of the form work, is :

a. 14 days for bottom slabs of spans 4.6 m and more

b. 21 days for bottom beams over 6 m spans

c. 2 days for vertical sides of columns

d. all the above.

28. Too wet concrete may cause

a. excessive laitance

b. segregation

c. lower density

d. all the above.

29. For controlling algae, the most commonly used chemical, is

a. copper sulphate

b. alum

c. lime

d. bleaching powder.

30. 135 litres of water per person per day, is provided in

a. nurses homes

b. hostels

c. residential schools

d. all the above.

31. The most widely used pump for lifting sewage is

a. centrifugal pump

b. reciprocating pump

c. pneumatic ejector

d. air pressure pump.

32. The suitable cross-section of sewers to carry combined flow, is

a. circular

b. egg shaped

c. rectangular

d. horse shoe shaped.

33. For a unique design of a channel by Kennedy's theory

a. its breadth must only be known

b. its depth must only be known

c. its breadth and depth ratio must only be known

d. all the above.

34. A river is said to be of

a. aggrading type if it builds up its bed to a certain slope

b. degrading type if it cuts its bed to a certain slope

c. meandering type if it flows in sinuous curve

d. all the above.

35. Pick up the correct statement from the following

a. Approach of the water line in a flumed channel section should not be steeper than

b. Departure of the water line from a flumed channel section should not be steeper than 30°

c. Approach of the water line should not be steeper than 30° and departure line not steeper than  in a flumed channel section

d. Approach and departure of the water line in a flumed channel section, should not be steeper than

36. A current meter measures the velocity of flow, if it is held

a. on the surface of water

b. at the bottom of channel

c. at any point within the cross-section

d. none of these.

37. The normal road land width for a major district road in open area, is

a. 45 m

b. 25 m

c. 15 m

d. 12 m

38. If the radius of curvature of a hill road is 50 m, the percentage grade compensation should be

a. 70/R

b. 75/R

c. 80/R

d. 100/R

39. Length of vehicles does not affect

a. minimum radius of turning

b. passing sight distance

c. width of shoulders

d. none of these.

40. If cross slope of a country is upto 10% the terain is classified as

a. plain

b. rolling

c. mountainous

d. steep.

41. The weight of an item is measured correct to nearest

a. 0.50 kg

b. 0.75 kg

c. 1.00 kg

d. 5 kg

42. A portion of an embankment having a uniform up-gradient 1 in 500 is circular with radius 1000 m of the centre line. It subtends 180° at the centre. If the height of the bank is 1 m at the lower end, and side slopes 2:1, the earth work involved.

a. 26, 000 m3

b. 26, 500 m3

c. 27, 000 m3

d. 27, 500 m3

43. The assumption on which the trapezoidal formula for volumes is based, is

a. The end sections are parallel planes

b. The mid-area of a pyramid is half the average area of the ends

c. The volume of the prismoidal is over-es-timted and hence a prismoidal correction is applied

d. All the above.

44. Pick up the correct statement from the following:

a. The actual expenditure involved to complete a work including incidental, establishment and travelling charges, is called actual cost

b. The formal acceptance by the administrative department for incurring an expenditure on the work, is called administrative approval

c. The order of a competent authority sanctioning a properly detailed estimate of the cost of a work of construction or repair is called technical sanction

d. All the above.

45. A construction schedule is prepared after collecting

a. output of labour

b. output of machinery

c. quantity of various items

d. all the above.

46. Which one of the following represents an event ?

a. concrete cured

b. fixing of door

c. plastering of walls

d. selecting sites

47. Pick up the correct statement from the following:

a. If the float is positive and the activity is delayed by a period equal to its total float, the completion of project is not delayed

b. If the float of an activity is negative, delay in its performance is bound to delay the completion of project

c. If the float of an activity is zero, the activity is critical and any delay in its performance will delay the whole project

d. All the above.

48. A Milestone chart

a. shows the interdependencies of various jobs

b. depicts the delay of jobs, if any

c. points outgoing ahead of schedule of jobs, if any

d. none of these.

49. If lift off distance of an craft is 2000 m, the clear way at the end of the runway should not be less than

a. 145 m

b. 152.5 m

c. 162.5 m

d. 172.5 m

50. Pick up the incorrect statement from the following:

a. L.M.M. is installed at 1.0 km from the threshold

b. Localizer antenna is installed at 300 m from the other end of the runway

c. Glide slope antenna is installed at the centre of the runway about 150 m on one side

d. None of these.