***Sub Engineer***

1. The operation of resection involves the following stepsrough orientation of the plane tablethe three lines form a triangle of errordrawing lines back through the three control pointsselect a point in the triangle of error such that each ray is equally rotated either clockwise or anti clockwisethe points obtained by three rays is the correct location. The correct sequence is

a. 1, 3, 2, 4, 5

b. 1, 2, 3, 4, 5

c. 1, 4, 3, 2, 5

d. 1, 4, 2, 3, 5

2. The magnetic bearing of a line is 32° and the magnetic declination is 10° 15' W. The true bearing is

a. 21° 45'

b. 42° 15'

c. 42° 15' W

d. 21° 45' W

3. While measuring with a metallic tape of 30 m length pull should be applied

a. 1 kg

b. 2 kg

c. 3 kg

d. 4 kg

4. A uniform slope was measured by the method of stepping. If the difference in level between two points is 1.8 m and the slope distance between them is 15 m, the error is approximately equal to

a. cumulative, + 0.11 m

b. compensating, ± 0.11 m

c. cumulative, - 0.11 m

d. none of these

5. When a brick is immersed in water for 24 hours and then dried, if

a. no grey or white deposits appear on the surface, the brick is free from soluble salts

b. 10 per cent surface is covered with grey or white deposits, the brick has slight efflorescence

c. 50 per cent surface is covered with grey or white deposits, the brick has serious efflorescence

d. All the above.

6. Wrought iron is used for

a. structural works in beams

b. small sized water pipes

c. columns and struts

d. none to these.

7. The compound of Portland cement which reacts immediately with water and also sets first is

a. Tri-calcium silicate

b. Di-calcium silicate

c. Tri-calcium aluminate

d. Tetra calcium alumino ferrite.

8. The cracks caused by shrinkage of the exterior surface of the wood exposed to atmosphere, are called:

a. radial shakes

b. heart shakes

c. wind cracks

d. twisted fibres.

9. The strain energy stored in a spring when subjected to greatest load without being permanently distorted, is called

a. stiffness

b. proof resilience

c. proof stress

d. proof load.

10. The general expression for the B.M. of a beam of length l is the beam carries

a. a uniformly distributed load w/unit length

b. a load varying linearly from zero at one end to w at the other end

c. an isolated load at mid span

d. none of these.

11. The bending moment at E for the structure shown in below figure, is

a. zero

b. 10 Tm

c. 20 Tm

d. 40 Tm.

12. For a beam, if fundamental equations of statics are not sufficient to determine all the reactive forces at the supports, the structure is said to be

a. determinate

b. statically determinate

c. statically indeterminate

d. none of these.

13. The total pressure force on a plane area is equal to the area multiplied by the intensity of pressure at its centroid, if

a. area is horizontal

b. area is vertical

c. area is inclined

d. all the above.

14. When the whole fluid mass rotates either due to fluid pressure or gravity or rotation previously imparted, the motion is known as

a. free vortex

b. forced vortex

c. non-potential vortex

d. rotational vortex.

15. If the pressure at the inlet of a pipe is 90 kg/cm2 and pressure drop over the pipe line is 10 kg/cm2, the efficiency of transmission, is

a. 66.6%

b. 77.7%

c. 55.5%

d. 88.8%.

16. An orifice is called a large orifice if water head, is

a. twice the diameter of the orifice

b. thrice the diameter of the orifice

c. four times the diameter of the orifice

d. five times the diameter of the orifice.

17. The effective size of particles of soil is denoted by

a. D10

b. D20

c. D30

d. D60

18. The inventor of the term soil mechanics, was

a. Kray

b. Dr. Karl Terzaghi

c. Leygue

d. Fellenius.

19. You are given a sample of soil containing coarse grains to determine its water content, you will use

a. pycnometer

b. oven-drying method

c. calcium carbide method

d. alcohol method.

20. Pile foundations are generally preferred to for

a. bridge foundations

b. sky scrapper buildings

c. residential buildings

d. runways.

21. In a cantilever retaining wall without a heel slab

a. thickness of the stem is kept same throughout

b. base slab is made 10 cm thicker than the stem

c. width of the base slab is kept 0.7 time the total height of the wall

d. all the above.

22. The maximum shear stress (qmax) in a rectangular beam is

a. 1.50 times the average

b. 1.75 times the average

c. 2.0 times the average

d. 2.5 times the average.

23. In a combined footing for two columns carrying unequal loads, the maximum hogging bending moment occurs at

a. more loaded column

b. a point equidistant from either column

c. a point of the maximum shear force

d. a point of zero shear force.

24. To ensure uniform pressure distribution, the thickness of the foundation, is

a. kept uniform throughout

b. increased gradually towards the edge

c. decreased gradually towards the edge

d. kept zero at the edge.

25. The additional piles which are driven to increase the capacity of supporting loads on vertical piles, are known

a. raking piles

b. eccentric piles

c. sinking piles

d. none of these.

26. Suitable spacing of timber piles, is

a. 60 cm

b. 70 cm

c. 80 cm

d. 90 cm.

27. The preliminary test is repeated if the difference of compressive strength of three test specimens, exceeds

a. 8 kg/cm2

b. 10 kg/cm2

c. 12 kg/cm2

d. 15 kg/cm2

28. The percentage of the aggregate of F.M. 2.6 to be combined with coarse aggregate of F.M. 6.8 for obtaining the aggregates of F.M. 5.4, is

a. 30%

b. 40%

c. 50%

d. 60%.

29. The best quality of filter material is obtained from quartzite if it does not loose weight when placed in hydro-chloric acid for 24 hours, more than

a. 5%

b. 8%

c. 10%

d. 12%

30. The equation  in which Q is yearly run off in cm, P is yearly total rainfall in cm, H is difference of R.L.s. of lowest and highest points and A is area of catchment in square metres, is known as

a. English formula

b. Khosla's formula

c. Justin's formula

d. Vermule's formula.

31. Nitrogen cycle of sewage, is

a. Liberation of ammonia-formation of nitrites-formation of nitrates-liberation of nitrogen

b. Liberation of nitrogen-liberation of ammonia-formation of nitrites- formation of nitrates

c. Liberation of nitrogen-formation of nitrates-formation of nitrites-liberation of ammonia

d. Formation of nitrates-formation of nitrites-liberation of nitrates-liberation of nitrates

32. The self-cleansing velocity of water flowing through pipe lines, is

a. 2 metres/sec

b. 1 metre/sec

c. 0.5 metre/sec

d. 0.25 metre/sec.

33. If D is the depth of water upstream of the throat above its sill, B is the width of the throat, to achieve critical flow in an open venturi flume, the theoretical maximum flow Q, is

a. Q = 1.71 BD

b. Q = 1.71 BD3/2

c. Q = 1/71 BD2/3

d. none of these.

34. For the conditions enumerated to provide a crossing at C1 You will probably provide

a. an aqueduct

b. a super-passage

c. a syphon aqueduct

d. none of these.

35. When a canal and a drainage approach each other at the same level, the structure so provided, is

a. an aqueduct

b. a syphon

c. a level crossing

d. inlet and outlet.

36. The consumptive use of water for a crop

a. is measured as the volume of water per unit area

b. is measured as depth of water on irrigated area

c. may be supplied partly by precipitation and partly by irrigation

d. all the above.

37. For a vehicle moving with a speed of 80 km per hour, the brake reaction time, in ordinary cases, is

a. 1.5 sec

b. 2.0 sec

c. 2.5 sec

d. 3.0 sec

38. Retaining walls are generally constructed with dry rubble stones with 60 cm top width and

a. 1 : 2 front batter

b. 1 : 3 front batter

c. 1 : 4 front batter

d. 1 : 5 front batter

39. If the rate of change of the super-elevation along a curved portion of a 7 metre wide road is 1 in 150 and the maximum super-elevation allowed is 1 in 15, the maximum length of the transition curve to be provided at either end, is

a. 70 m

b. 75 m

c. 80 m

d. 90 m

40. The top height of a route marker above crown level is

a. 1.50 m

b. 1.75 m

c. 2.00 m

d. 2.25 m

41. The diameter of a domestic sewer pipe laid at gradient 1 in 100 is recommended

a. 100 mm

b. 150 mm

c. 200 mm

d. 175 mm

42. The item of the brick structure measured in sq.m, is

a. Reinforced brick work

b. Broken glass coping

c. Brick edging

d. Brick work in arches.

43. Pick up the correct statement from the following:

a. Cement is paid per 50 kg bag

b. Lime is paid per quintal

c. Brick aggregates is paid per cum

d. All the above.

44. In case of laying gullies, siphons, intercepting traps, the cost includes

a. Setting and laying

b. Bed concreting

c. Connection to drains

d. All of these.

45. Construction team means

a. an architect

b. an owner

c. a contractor

d. all the above.

46. If TL is the latest allowable event occurrence time, total activity slack(s), is equal to

a. LST-EST

b. LFT-EFT

c. TL-EFT

d. all the above.

47. Modular co-ordination of construction means proper

a. planning

b. designing

c. execution

d. all the above.

48. Pick up the incorrect statement from the following:

a. The difference between the maximum time available for the job and actual time it consumes, is called total float

b. The difference between the latest start time and earliest start time of an activity, is called total float

c. The difference between the earliest finish time of an activity and the earliest start time of its successor activity, is called free float of the activity

d. None of these.

49. An aircraft is flying in an atmosphere of 30°C with a speed of 1260 km ph. Its speed is known as

a. subsonic

b. sonic

c. super-sonic

d. Mach.

50. Two single runways may be arranged so as to have

a. L-shape

b. T-shape

c. X-shape

d. All the above.