***Sub Engineer***

1. Centering error of a theodolite produces an error

a. in all angles equally

b. which does not vary with the direction or pointing

c. which varies with the direction of pointing and inversely with the length of sight

d. none of these.

2. Measuring with a 30 m chain, 0.01 m too short, introduces

a. positive compensating error

b. negative compensating error

c. positive cumulative error

d. negative cumulative error.

3. Reduced bearing of a line is an angle between

a. north line and given line measured clockwise

b. north line and given line measured anticlockwise

c. east or west and the given line

d. given line and the part of the meridian whether N end or S end, lying adjacent to it.

4. Ranging in chain survey means

a. establishing an intermediate point on the line

b. determining the distance between end points

c. determining the offset distance

d. none of these

5. Pick up the correct statement from the following:

a. The polymer consists of thousands of monomers joined together

b. The polymer molecule is called macro-molecule

c. A polymetric material consists of a large number of long-chain molecules

d. All the above.

6. Clay and silt content in a good brick earth must be at least

a. 50%

b. 40%

c. 30%

d. 25%

7. Quartzite is a

a. metamorphic rock

b. argillaceous rock

c. calcareous rock

d. silicious rock.

8. When a brick is immersed in water for 24 hours and then dried, if

a. no grey or white deposits appear on the surface, the brick is free from soluble salts

b. 10 per cent surface is covered with grey or white deposits, the brick has slight efflorescence

c. 50 per cent surface is covered with grey or white deposits, the brick has serious efflorescence

d. All the above.

9. Slenderness ratio of a long column, is

a. area of cross-section divided by radius of gyration

b. area of cross-section divided by least radius of gyration

c. radius of gyration divided by area of cross-section

d. length of column divided by least radius of gyration.

10. For calculating the permissible stress  is the emprical formula, known as

a. Straight line formula

b. Parabolic formula

c. Perry's formula

d. Rankine's formula.

11. The type of butt joints in common use, is :

a. single inverted V-butt joint

b. double V-butt joint

c. double U-butt joint

d. single V-butt joint.

12. The maximum bending moment due to a moving load on a simply supported beam, occurs

a. at the supports

b. under the load

c. anywhere on the beam

d. none of these.

13. If Cv, Cc, Cd and Cr are the hydraulic coefficients of an orifice, then

a. Cd = Cc . Cv

b. Cr = 1 + Cv2/Cd

c. Cv = Cc + Cd

d. Cc = Cv/Cd

14. Chezy's formula is used to determine

a. head loss due to friction in pipe

b. velocity of flow in pipe

c. velocity of flow in open channels

d. none of these.

15. Molecules of fluids get attracted due to

a. surface tension

b. adhesion

c. cohesion

d. none of these.

16. The total pressure force on a plane area is equal to the area multiplied by the intensity of pressure at its centroid, if

a. area is horizontal

b. area is vertical

c. area is inclined

d. all the above.

17. In non-cohesive soil in passive state of plastic equilibrium

a. major principal stress is horizontal

b. minor principal stress is vertical

c. major principal stress is vertical

d. minor and major principal stresses are equally inclined to the horizontal.

18. For determining the moisture content of a soil sample, the following data is available Weight of container = 260 g, Weight of soil sample and = 320 g container, Weight of soil sample (dried) and = 310 g container. The moisture content of the soil sample, is

a. 15%

b. 18%

c. 20%

d. 25%

19. Pick up the correct statement from the following:

a. Isotropic consolidation of clay can be obtained in the triaxial apparatus under equal all-round pressure.

b. If the present effective stress is the maximum to which the clay has ever been subjected, it is called normally consolidated clay

c. If the present effective stress in the past was more than present effective stress, it is called over-consolidated clay

d. All the above.

20. If drainage is permitted throughout the test, during the application of both normal, and shear stresses so that full consolidation occurs and no excess pore pressure is set up at any stage of the test, is known as

a. quick test

b. drained test

c. consolidated undrained test

d. none of these.

21. Lapped splices in tensile reinforcement are generally not used for bars of size larger than

a. 24 mm diameter

b. 30 mm diameter

c. 36 mm diameter

d. 32 mm diameter

22. In a prestressed beam carrying an external load W with a bent tendon is having angle of inclination θ and prestressed load P. The net downward load at the centre is

a. W - P cos θ

b. W - P sin θ

c.  W - 2P sin θ

d. W + 2P sin θ

23. The length of lap in tension reinforcement should not be less than the bar diameter x (actual tension / four times the permissible average bond stress) if it is more than

a. 18 bar diameters

b. 24 bar diameters

c. 30 bar diameters

d. 36 bar diameters

24. An R.C.C. beam not provided with shear reinforcement may develop cracks in its bottom inclined roughly to the horizontal at

a. 35°

b. 45°

c. 55°

d. 60°

25. The projections of head or sill of a door or window frame, are

a. transoms

b. horns

c. stops

d. chocks.

26. Dado is usually provided in

a. bath rooms

b. living rooms

c. verandah

d. roofs.

27. If the various concrete ingredients i.e. cement, sand and aggregates are in the ratio of 1:3:6, the grade of concrete, is

a. M 100

b. M 150

c. M 200

d. M 250

28. On a grading curve, the gap grading is represented by

a. a horizontal line

b. a vertical line

c. N.W. inclined line

d. N.E. inclined line

29. Pick up the correct statement from the following :

a. The amount of rainfall in one year, is known as annual rainfall

b. The rain cycle period in India is taken as 35 years

c. The ratio of the actual rainfall in the year to the normal rainfall at a place, is called index of the wetness

d. All the above.

30. The best quality of filter material is obtained from quartzite if it does not loose weight when placed in hydro-chloric acid for 24 hours, more than

a. 5%

b. 8%

c. 10%

d. 12%

31. R.M.O. expenses include

a. running expenses

b. maintenance expenses

c. operation expenses

d. all the above.

32. The sewer pipe which carries sewage from a building for immediate disposal is

a. house sewer

b. lateral sewer

c. intercepting sewer

d. main sewer.

33. A hydraulic jump is generally formed when a stream moving with

a. a hyper-critical velocity meets a stream moving with a critical velocity

b. a hyper-critical velocity meets a stream moving with a hyper-critical velocity

c. A hyper-critical velocity meets a stream moving with a sub-critical velocity

d. a sub-critical velocity meets a stream moving with a hyper-critical velocity.

34. The ratio of the rate of change of discharge of an outlet and parent channel, is known as

a. efficiency

b. sensitivity

c. flexibility

d. modular limit.

35. Process of meandering is due to

a. sediment load of streams

b. discharge and hydraulic properties of streams

c. relative erodibility of the bed and banks

d. all the above.

36. A water-logged land is found suitable for cultivation due to

a. ease of tillage for preparation of the field for optimum condition of germination

b. absence of aeration of soil from root zone of the plant

c. regular supply of water to plants from the water table by capillary action

d. none of these.

37. Shoulders for high traffic volume roads, should

a. be stable throughout the year to be used by vehicles in the case of emergency

b. support and protect the ends of carriage ways

c. not allow entrance of water to sub-grade

d. all the above.

38. Thickness of broken centre line markings for a four lane road, is generally kept

a. 12 cm

b. 15 cm

c. 18 cm

d. 20 cm

39. When an up gradient of a highway meets a downgrade, the vertical curve provided, is known as

a. valley curve

b. sag curve

c. summit curve

d. all the above.

40. If a Lemniscate curve having an angle of deflection Δ, is transitional throughout, the maximum polar angle is

a. Δ/3

b. Δ/4

c. Δ/5

d. Δ/6

41. Pick up the correct statement in case of water supply.

a. Cutting through walls and floors are included with the item

b. Pipes are classified according to their sizes and quality

c. In laying pipes, the method of jointing and fixing is specifically specified

d. All the above.

42. Pick up the correct statement from the following:

a. Plastering is measured in sq.m

b. Glazing is measured in sq.m

c. Striking is measured in sq.m

d. All the above.

43. The main factor to be considered while preparing a detailed estimate, is

a. Availability of materials

b. Transportation of materials

c. Location of site and local labour charges

d. All the above.

44. The damp proof course (D.P.C.) is measured in

a. Cub.m

b. Sq m

c. Metres

d. None of these

45. The salient feature of functional organisation is

a. separation of planning and design part

b. each individual maintains functional efficiency

c. work is properly planned and distributed

d. all the above.

46. Pick up the correct statement from the following:

a. The difference of latest occurence time and earliest expected time, is called slack

b. The activities connecting the events having zero slack, lie on the critical path

c. The critical path consumes the maximum time

d. All the above.

47. Completion of an activity on CPM network diagram, is generally known

a. Event

b. Node

c. Connector

d. All the above.

48. For the supply of materials for concrete, form work reinforcing and placing of concrete, removal of form work and curing of concrete, number of bar(s) required on bar chart, is

a. 1

b. 2

c. 3

d. 4

49. Pick up the incorrect statement from the following:

a. In single engine aeroplanes, the engine is provided in the nose of the aircraft

b. In double engine aeroplanes, one engine on either wing is placed symmetrically

c. In three engine aeroplanes, two engines are placed on both wings and one engine is placed in the tail

d. None of these.

50. The length of runway is increased per 300 m rise above M.S.L.

a. 4%

b. 5%

c. 6%

d. 7%