***Sub Engineer***

1. If vertical angles of inclined sights do not exceed 10° and non-verticality of the staff remains within 1°, stadia system of tacheometric observations are made on

a. staff normal

b. staff vertical

c. staff normal as well as vertical

d. none of these.

2. Staff readings on pegs x and y from X station are 1.755 m and 2.850 m, and from station Y on staff head at Y and X are 0.655 m and 1.560 m. If reduced level of X is 105.5 m, the reduced level of Y is

a. 104.0 m

b. 104.5 m

c. 105.0 m

d. 105.5 m

3. The correction to be applied to each 30 metre chain length along θ° slope, is

a. 30 (sec  θ - 1) m

b. 30 (sin  θ - 1) m

c. 30 (cos  θ - 1) m

d. 30 (tan  θ - 1) m

4. The curve composed of two arcs of different radii having their centres on the opposite side of the curve, is known

a. a simple curve

b. a compound curve

c. a reverse curve

d. a vertical curve.

5. Pick up the polymineralic rock from the following:

a. Pure gypsum

b. Magnesite

c. Granite

d. None of these.

6. Pick up the correct statement from the following:

a. Quick lime is obtained by burning pure lime stone:

b. Hydraulic lime is obtained by burning lime stone containing clay 5% to 30%

c. Poor lime is obtained by burning lime stone containing impurities more than 5%

d. All the above.

7. Stones used for ornamental work must be

a. soft

b. hard

c. light

d. heavy.

8. Percentage content of silica in window glass, is

a. 40 to 45

b. 50 to 55

c. 60 to 65

d. 70 to 75.

9. In a shaft, the shear stress is not directly proportional to

a. radius of the shaft

b. angle of twist

c. length of the shaft

d. modulus of rigidity.

10. In the truss shown in given figure, the force in member BD is

a. 100 t compressive

b. 100 t tensile

c. zero

d. indeterminate

11. The B.M. diagram of the beam shown in below figure, is

a. a rectangle

b. a triangle

c. a trapezium

d. a parabola

12. At either end of a plane frame, maximum number of possible bending moments, are

a. two

b. three

c. four

d. zero.

13. A rise or fall of liquid in a glass tube of a very small diameter when dipped is

a. directly proportional to the force per unit length of periphery

b. direcly proportional to the sine of the angle of contact

c. directly proportional to the specific weight of liquid

d. inversely proportional to the diameter of the glass tube.

14. A floating body attains stable equilibrium if its metacentre is

a. at the centroid

b. above the centroid

c. below the centroid

d. anywhere.

15. In order to avoid capillary correction, the minimum diameter of a manometer used for measuring pressure, should be

a. 4 mm

b. 6 mm

c. 8 mm

d. 10 mm.

16. If H is the difference of liquid levels on two sides of an orifice (width b, depth d), the discharge through the orifice will be

a. Cdbd22gH

b. Cdb2d2gH

c. Cdbd2gh

d. Cdbd.2g H1/2.

17. In a fow net

a. flow lines and equipotential lines cross each other at right angles

b. fields are rectangles whose length is twice the breadth

c. smaller the dimensions of the field, smaller will be the hydraulic gradient and velocity of flow through it

d. for homogeneous soil, the curves are smooth and circular.

18. Geologic cycle for the formation of soil, is

a. Weathering → upheaval → transportation → deposition

b. Transportation → upheaval → weathering → deposition

c. Weathering → transportation → deposition → upheaval

d. None of these.

19. The ratio of the volume of water present in a given soil mass to the total volume of its voids, is known

a. porosity

b. void ratio

c. percentage voids

d. degree of saturation.

20. The density of soil can be increased

a. by reducing the space occupied by air

b. by elastic compression of soil grains

c. by expelling water from pores

d. All the above.

21. In a simply supported slab, alternate bars are curtailed at

a. 1/5th of the span

b. 1/6th of the span

c. 1/7th of the span

d. none of these.

22. An intermediate T-beam reinforced with two layers of tensile steel with clear cover 13 cm encasted with the floor of a hall 12 metres by 7 metres, is spaced at 3 metres from adjoining beams and if the width of the beam is 20 cm, the breadth of the flange is

a. 233 cm

b. 176 cm

c. 236 cm

d. 255 cm

23. A T-beam behaves as a rectangular beam of a width equal to its flange if its neutral axis

a. remains within the flange

b. remains below the slab

c. coincides the geometrical centre of the beam

d. none of these.

24. The horizontal portion of a step in a stairs case, is known as

a. rise

b. flight

c. winder

d. tread.

25. The stone masonry of finely dressed stones laid in cement or lime, is

a. random rubble masonry

b. coursed rubble masonry

c. dry rubble masonry

d. ashlar masonry.

26. Pick up the correct statement from the following :

a. Double-flemish bond is suitable for brick masonry to give uniform face appearance

b. The double-flemish bond is used for the construction of single brick residential building

c. The stretcher bond is used for the construction of half brick masonry brick

d. all the above.

27. Curing a concrete for long period ensures better

a. strength

b. water resistance

c. water tightness and durability

d. all the above.

28. A concrete having a slump of 6.5 cm, is said to be

a. earth moist

b. semi-plastic

c. plastic

d. none of these.

29. The factor affecting per capita demand, is

a. climatic conditions

b. pressure in water mains

c. cost of water

d. all the above.

30. The best quality of filter material is obtained from quartzite if it does not loose weight when placed in hydro-chloric acid for 24 hours, more than

a. 5%

b. 8%

c. 10%

d. 12%

31. The ratio of the diameter of a circular section and the side of a square section hydraulically equivalent, is

a. 1.095

b. 1.085

c. 1.075

d. 1.065

32. Pathogens (or pathogentic bacterias) in water may cause

a. cholera

b. dysentery

c. infectious hepatitis

d. all the above.

33. Lacey's equation for calculating flood discharge in rivers, is

a. V = 10.8 R1/2 S1/2

b. V = 10.8 R2/3 S1/2

c. V = 10.8 R2/3 S1/3

d. V = 10.8 R1/3 S2/3

34. The water face of the guide banks, is protected by

a. one men stone pitching

b. two man stone pitching

c. three man stone pitching

d. four man stone pitching

35. Groynes are generally built

a. perpendicular to the bank

b. inclined up stream up to 30°

c. inclined down stream upto 30°

d. all the above.

36. In a concrete canal the approximate permissible velocity of water should not exceed

a. 1 m/sec

b. 1.5 m/sec

c. 2 m/sec

d. 2.5 m/sec

37. Any gradient on a road is said to be an exceptional gradient, if it is

a. more than ruling gradient

b. less than average gradient

c. more than floating gradient

d. less than minimum gradient or more than maximum gradient.

38. Design of highways is based on

a. axle spacings

b. wheel bases

c. over all length

d. all the above.

39. For a poorly graded sub-grade soil, thickness of sub-base, is

a. 15 cm

b. 20 cm

c. 25 cm

d. 30 cm

40. Road width 8.8 m of two lane National highways or State highways in mountainous terrain

a. excludes the width of parapet (0.6 m)

b. excludes the width of side drain (0.6 m)

c. excludes the width of parapet and side drain

d. includes the width of parapet and side drain

41. The value of 'A' of Indian type W.C. shown in the given figure is :

a. 25 cm

b. 30 cm

c. 40 cm

d. 45 cm

42. The expected out turn for earth work in excavation in ordinary soil per mazdoor per day is

a. 1.00 cubic meter

b. 2.00 cubic meter

c. 3.00 cubic meter

d. 4.00 cubic meter

43. The expected out turn of cement concrete 1 : 2 : 4 per mason per day is

a. 1.5 m3

b. 2.5 m3

c. 3.5 m3

d. 5.0 m3

44. Pick up the correct statement from the following:

a. In order to check up the average depth of excavation, 'Dead mans' are left at the mid-widths of borrow pits

b. The earthwork calculation in excavation is made from the difference in levels obtained with a level

c. The earth work in excavation to form the road embankment includes the formation of correct profile and depositing the soil in layers

d. All the above.

45. PERT is

a. limited of event oriented diagrams

b. used for research and development projects

c. based on three time estimates for activities linking up two events

d. all the above.

46. Pick up the correct statement from the following:

a. The duration of time by which an activity can be delayed without affecting the succeeding activity, is called free float

b. The float which affects neither the processor nor the successor activities, is called independent float

c. The difference between total float and free float, is called interfering float

d. All the above.

47. The Overall in-charge of an organisation at the site responsible for the execution of the works, is

a. Engineer

b. Junior Engineer

c. Sub overseer

d. Assistant Engineer.

48. The salient feature of functional organisation is

a. separation of planning and design part

b. each individual maintains functional efficiency

c. work is properly planned and distributed

d. all the above.

49. Conical surface of the approach area rises outwards

a. 1 in 10

b. 1 in 15

c. 1 in 20

d. 1 in 25

50. Pick up the correct statement from the following:

a. The basic length of a runway is increased at a rate of 7% per 300 m of elevation of M.S.L.

b. The standard temperature at the site is obtained by reducing the standard sea level temperature of 15°C at the rate of 6.5°C per 1000 m rise in elevation

c. The aerodrome reference temperature is the monthly mean of the mean daily temperature for the hottest month of the year

d. All the above.