***Sub Engineer***

1. Pick up the method of surveying in which field observations and plotting proceed simultaneously from the following

a. chain surveying

b. compass surveying

c. plan table surveying

d. tacheometric surveying.

2. The limiting length of an offset does not depend upon

a. accuracy of the work

b. method of setting out perpendiculars

c. scale of plotting

d. indefinite features to be surveyed.

3. The bearing of line AB is 152° 30' and angle ABC measured clockwise is 124° 28'. The bearing of BC is

a. 27° 52'

b. 96° 58'

c. 148° 08'

d. 186° 58'

4. The tangent to the liquid surface in a level tube, is parallel to the axis of the level tube at

a. every point of the bubble

b. either end of the bubble

c. the mid-point of the bubble

d. no where.

5. A rock contains only one mineral. It is called

a. homogeneous

b. non-homogeneous

c. monomineralic

d. polymineralic.

6. A badly mixed cement concrete results in

a. segregation

b. bleeding

c. honey combing

d. none to these.

7. For slaking of 10 kg of CaO, the theoretical amount of water is

a. 2.2 kg

b. 1.5 kg

c. 3.2 kg

d. None of these.

8. Pick up the correct statement from the following:

a. Catalysts are added to assist and accelerate the hardening of resin,

b. The fillers are inert materials and they impart strength and hardness

c. Fibrous fillers increase thermal resistance

d. All the above.

9. The ratio of tangential and normal components of a stress on an inclined plane through θ° to the direction of the force, is :

a. cos θ

b. tan θ

c. cos θ

d. sec θ

10. A composite beam is composed of two equal strips one of brass and other of steel. If the temperature is raised

a. brass experiences compressive force

b. composite beam gets subjected to a couple

c. composite beam bends

d. All the above.

11. The ratio of the flexural strengths of two square beams one placed with its two sides horizontal and the other placed with one diagonal vertical, diagonal, is

a. 2

b. 2

c. 5

d. 7

12. The direction of the reaction at support B of a truss shown in below figure will be

a. East of North

b. West of North

c. East of South

d. West of South

13. If velocities of fluid particles vary from point to point in magnitude and direction, as well as from instant to instant, the flow is said to be

a. laminar

b. turbulent flow

c. uniform flow

d. non-uniform flow.

14. On an inclined plane, centre of pressure is located

a. at the centroid

b. above the centroid

c. below the centroid

d. anywhere.

15. For a most economical rectangular channel, the hydraulic mean depth, is equal to

a. the depth of flow

b. half the depth of flow

c. one-third depth of flow

d. None of these.

16. Hydraulic coefficient of an orifice means the coefficient of

a. velocity

b. contraction

c. resistance

d. all the above.

17. A compacted soil sample using 10% moisture content has a weight of 200 g and mass unit weight of 2.0 g/cm3. If the specific gravity of soil particles and water are 2.7 and 1.0, the degree of saturation of the soil is

a. 11.1%

b. 55.6%

c. 69.6%

d. none of these.

18. The reduction in volume of soil due to squeezing out of water from the voids, is termed

a. primary consolidation

b. primary compression

c. primary time effect

d. all the above.

19. If water content of a soil is 40%, G is 2.70 and void ratio is 1.35, the degree of saturation is

a. 75%

b. 80%

c. 85%

d. 90%

20. Pick up the correct statement from the following :

a. To an agriculturist, soil is the substance existing on the earth's surface, which grows and develops plants

b. To a geologist, soil is the material in a relatively thin surface zone within which roots occur, and rest of the crust is termed as rock irrespective of hardness

c. To an engineer, soil is the unaggregated and uncemented deposits of minerals and organic particles covering the earth's crust

d. All the above.

21. In a combined footing if shear stress does not exceed 5 kg/cm2, the nominal stirrups provided are

a. 8 legged

b. 10 legged

c. 12 legged

d. none of these.

22. The length of lap in tension reinforcement should not be less than the bar diameter x (actual tension / four times the permissible average bond stress) if it is more than

a. 18 bar diameters

b. 24 bar diameters

c. 30 bar diameters

d. 36 bar diameters

23. The anchorage value of a hook is assumed sixteen times the diameter of the bar if the angle of the bend, is

a. 40°

b. 45°

c. 60°

d. all the above.

24. The effective span of a simply supported slab, is

a. distance between the centres of the bearings

b. clear distance between the inner faces of the walls plus twice the thickness of the wall

c. clear span plus effective depth of the slab

d. none of these.

25. While designing a stair, the product of rise and going is approximately kept equal to

a. 420

b. 450

c. 500

d. 600.

26. Weep holes are provided in retaining and breast walls

a. to drain off the water from the filling

b. to ventilate the stone masonry

c. to add architectural beauty

d. to reduce the weight of the earth retained

27. Segregation is responsible for

a. porous layers in concrete

b. surface scaling in concrete

c. sand streaks in concrete

d. all the above.

28. Pick up the correct statement from the following:

a. Cement should be mixed for at least one minute

b. 10% of water is placed in the rotating drum before adding dry material

c. 10% of water is added after placing the other ingredients in the drum

d. All the above.

29. 135 litres of water per person per day, is provided in

a. nurses homes

b. hostels

c. residential schools

d. all the above.

30. Chlorination of water does not remove

a. ammonia content

b. B.O.D.

c. organic matter content

d. dissolved oxygen.

31. Pick up the correct statement from the following :

a. Rational method of estimating peak run off, may be used precisely for areas less than 50 hectares

b. The period after which the entire area starts contributing to the run off, is called the time of concentration

c. The time required by water to reach the outlet of a catchment from the most remote point, is called time of concentration

d. All the above.

32. Dry water flow in a combined sewer, is

a. industrial sewage

b. domestic sewage

c. storm water

d. inclusive of domestic and industrial sewage but excludes storm water.

33. The field capacity of a soil depends upon

a. capillary tension in soil

b. porosity of soil

c. both (a) and (b)

d. neither (a) nor (b).

34. Pick up the correct statement from the following:

a. Gravity water is harmful to crops

b. Hygroscopic water remains attached to soil molecules by chemical bond

c. Capillary moisture held in the soil pores against gravity by surface tension, is utilised by plants

d. All the above.

35. The measure to remove water logging of land, is

a. to reduce percolation from canals and water courses

b. to increase outflow from the ground water reservoir

c. both (a) and (b)

d. neither (a) nor (b)

36. While deciding the alignment of a proposed canal taking off from a river at B, three alignments of approximately equal lengths are available. These cross a drainages at C1, C2 and C3 where drainage bed levels C1 > C2 > C3, you will provide at site C3

a. an aqueduct

b. a syphon aqueduct

c. a super passage

d. a syphon.

37. Full amount of extra width of a pavement on the curve, is provided at

a. beginning of the transition curve

b. centre of the transition curve

c. beginning of the circular curve

d. centre of the circular curve.

38. Design of horizontal curves on highways, is based on

a. permissible friction on the road surface

b. permissible centrifugal ratio

c. permissible maximum super-elevation

d. all the above.

39. Traffic engineering only includes

a. design and application of control devices

b. analysis of traffic characteristics

c. traffic opeation

d. all the above.

40. On the recommendations of Nagpur Conference, the minimum width of a village road may be

a. 2.45 m

b. 2.75 m

c. 3.66 m

d. 4.90 m

41. A portion of an embankment having a uniform up-gradient 1 in 500 is circular with radius 1000 m of the centre line. It subtends 180° at the centre. If the height of the bank is 1 m at the lower end, and side slopes 2:1, the earth work involved.

a. 26, 000 m3

b. 26, 500 m3

c. 27, 000 m3

d. 27, 500 m3

42. Pick up the correct statement from the following:

a. The bent up bars at a support resist the negative bending moment

b. The bent up bars at a support resist the sharing force

c. The bending of bars near supports is generally at 45°

d. All the above.

43. Pick up the correct statement from the following:

a. The earth work of cutting in trenches or borrow pits in fairly uniform ground is measured with the help of average depths of the dead men

b. The earth work in trenches or borrow pits in irregular ground is measured by taking the difference in levels before and after completion of work

c. The earth work in trenches or borrow pits, where neither a nor b is feasible, are measured from the fillings after deduction of voids

d. All the above.

44. While estimating the qualities for the construction of a building, the correct metric unit is

a. Cubic metre for area

b. Square metres for volume

c. Kilogram for weight

d. Litre for capacity

45. While filling the tender for any work, the contractor considers

a. availability of construction materials

b. availability of labour

c. study of specifications

d. all the above.

46. Pick up the correct statement from the following:

a. Programme Evaluation and Review Technique, is event oriented

b. Programme Evaluation and Review Technique is not event oriented

c. Critical Path Method is event oriented

d. Critical Path method is event oriented.

47. Which one of the following represents an event ?

a. concrete cured

b. fixing of door

c. plastering of walls

d. selecting sites

48. The estimated time required to perform an activity, is known as

a. event

b. dummy

c. duration

d. float.

49. The bearing of the longest line of a wind rose is S 45° E, the bearing of the runway will be numbered

a. 315°

b. 13

c. 31

d. both (c) and (d)

50. In instrument landing system

a. L.O.M. and L.M.M. are installed on one side and Localizer antenna on the other side

b. L.O.M. and Localizer are installed on one side and L.M.M. on the other side

c. Localizer and L.M.M. are installed on one side and L.O.M. on the other side

d. None of these.