***Sub Engineer***

1. In a precision traverse, included angles are measured by setting the vernier

a. to read zero exactly on back station

b. to read 5° exactly on back station

c. some where near zero and reading both verniers on back station

d. all the above.

2. The correction to be applied to each 30 metre chain length along θ° slope, is

a. 30 (sec  θ - 1) m

b. 30 (sin  θ - 1) m

c. 30 (cos  θ - 1) m

d. 30 (tan  θ - 1) m

3. The distance between steps for measuring down hill to obtain better accuracy

a. decreases with decrease of slope

b. increases with increase of slope

c. decreases with increase of slope

d. decreases with decrease of weight of the chain.

4. The area of a plane triangle ABC, having its base AC and perpendicular height h, is

a.  ba sin C

b.  bc sin A

c. S(S - a)(S - b)(S - c) where S is

d. all the above

5. Quick setting cement is produced by adding

a. more amount of gypsum in very fine powdered form

b. aluminium sulphate in very fine powdered form

c. pozzolana in very fine powdered form

d. none of these.

6. The softest rock is

a. marble

b. diamond

c. talc

d. quartz.

7. Seasoning of timber is done

a. to make it water proof

b. to paint its surface

c. to increase its temperature

d. to remove water.

8. Pick up the correct statement from the following:

a. Impurities float on the molten iron as slag

b. The slag contains lime about 45%

c. The molten iron is made to run, in a long channel formed in sand called 'sow'

d. All the above.

9. In plastic analysis, the shape factor for rectangular section, is

a. 1.4

b. 1.5

c. 1.6

d. 1.7

10. For calculating the allowable stress of long columns  is the empirical formula, known as

a. Straight line formula

b. Parabolic formula

c. Perry's formula

d. Rankine's formula.

11. A member which is subjected to reversible tensile or compressive stress may fail at a stress lower than the ultimate stress of the material. This property of metal, is called

a. plasticity of the metal

b. elasticity of the metal

c. fatigue of the metal

d. workability of the metal.

12. Maximum deflection of a

a. simply supported beam carrying a concentrated load W at mid-span is

b. cantilever beam, carrying a uniformly distributed load over span is

c. simply supported beam carrying a uniformly distributed load over the span is

d. All the above.

13. For critical depth of flow of water in open channels, fc the specific energy must be :

a. minimum

b. maximum

c. average of maximum and minimum

d. None of these.

14. The imaginary line drawn such that the tangents at its all points indicate the direction of the velocity of the fluid particles at each point, is called

a. path line

b. stream line

c. potential line

d. streak line.

15. A water tank partially filled with water is being carried on a truck moving with a constant horizontal acceleration. The level of the water

a. falls on the back side of the tank

b. remains the same at both sides of the tank

c. rises on the back side and falls on the front side

d. none of these.

16. Chezy's formula is used to determine

a. head loss due to friction in pipe

b. velocity of flow in pipe

c. velocity of flow in open channels

d. none of these.

17. The critical exist gradient of seepage water in soils, increases with

a. a decrease in specific gravity

b. a decrease in void ratio

c. both (a) and (c)

d. none of these.

18. Sedimentation analysis is based on the assumption:

a. soil particles are spherical

b. particles settle independent of other particles

c. walls of the jar do not affect the settlement .

d. all the above.

19. Pick up the correct statement from the following:

a. Isotropic consolidation of clay can be obtained in the triaxial apparatus under equal all-round pressure.

b. If the present effective stress is the maximum to which the clay has ever been subjected, it is called normally consolidated clay

c. If the present effective stress in the past was more than present effective stress, it is called over-consolidated clay

d. All the above.

20. When the seepage pressure becomes equal to the pressure due to submerged weight of a soil, the effective pressure is reduced to zero and the soil particles have a tendency to move up in the direction of flow. This phenomenon is generally known

a. quick condition

b. boiling condition

c. quick sand

d. all the above.

21. Dimensions of a beam need be changed if the shear stress is more than

a. 10 kg/cm2

b. 15 kg/cm2

c. 20 kg/cm2

d. 25 kg/cm2

22. A column is regarded as long column if the ratio of its effective length and lateral dimension, exceeds

a. 15

b. 20

c. 25

d. 30

23. The maximum ratio of span to depth of a slab simply supported and spanning in two directions, is

a. 30

b. 35

c. 40

d. 15

24. Pick up the correct statement from the following:

a. Lateral reinforcement prevents the shearing of concrete on diagonal plane

b. Lateral reinforcement stops breaking away of concrete cover, due to buckling

c. Lateral reinforcement in R.C.C. columns, is kept not less than 5 mm diameter

d. All the above.

25. Pick up the correct statement from the following:

a. Slump test is performed to check concrete strength

b. Curing of concrete is done for proper compaction of cement

c. Fineness modulus is the index number expressing the relative sizes of both coarse and fine aggregates

d. Concrete is a mixture of binding material, coarse aggregate and water.

26. The form work from the slabs excluding props, can be removed only after

a. 1 day

b. 4 days

c. 7 days

d. 14 days

27. To ensure constant moisture content in aggregates

a. height of each aggregate pile should not exceed 1.50 m

b. aggregate pile should be left for 24 hours before aggregates are used

c. conical heaps of aggregates should be avoided to prevent moisture variation

d. all the above.

28. Curing a concrete for long period ensures better

a. strength

b. water resistance

c. water tightness and durability

d. all the above.

29. The maximum hourly consumption, is generally taken as

a. 120%

b. 130%

c. 140%

d. 150%.

30. The coefficient of permeability of soils, is generally expressed in

a. cm/sec

b. cm/minute

c. cm2/sec

d. cm2/minute.

31. Dry weather flow is :

a. average monthly rate of flow

b. average annual rate of flow

c. water supply allowance per capita

d. none of these.

32. Pick up the incorrect size of stone ware sewers

a. 67.5 cm

b. 82.5 cm

c. 90 cm

d. none of these.

33. Pick up the correct statement from the following

a. The capacity of escapes should not be less than the capacity of the canal at its location

b. The escapes must lead the surplus water to natural drainages

c. The escapes are aligned to take advantage of contours of lower values

d. All the above.

34. If average particle size of the silt in millimetres is m, the Lacey's silt factor f is proportional to

a. m3

b. m1/2

c. m1/3

d. m2/3

35. A counter berm is

a. a horizontal benching provided on the inside slope

b. a horizontal benching provided on the outside slope

c. a vertical benching provided on the outer edge of the bank

d. a vertical benching provided on the inner edge of the bank.

36. The measure to remove water logging of land, is

a. to reduce percolation from canals and water courses

b. to increase outflow from the ground water reservoir

c. both (a) and (b)

d. neither (a) nor (b)

37. For maximum strength and durability minimum percentage of cement, by weight is

a. 20%

b. 25%

c. 30%

d. 33%

38. Over taking time required for a vehicle with design speed 50 km ph and overtaking acceleration 1.25 m/sec2 to overtake a vehicle moving at a speed 30 km ph, is

a. 5.0 secs

b. 6.12 secs

c. 225.48 secs

d. 30 secs

39. In a right angle bend of a road provided with a transition throughout, the maximum polar angle will be

a. 15°

b. 20°

c. 30°

d. 45°

40. Set-back distance is the distance between

a. road land boundary and building line

b. road land boundary and control line

c. building line and control line

d. road land boundary and control line.

41. The ground surface slopes 1 in 50 along a proposed railway embankment 150 m in length. The height of the embankment at zero chainage is 0.5 m, the width is 11 m and side slopes 2:1. If the falling gradient of the embankment is 1 in 150, the quantity of the earthwork calculated by prismoidal formula, is

a. 3250 m3

b. 3225 m3

c. 3275 m3

d. 3300 m3

42. For 100 sq. m cement concrete (1 : 2: 4) 4 cm thick floor, the quantity of cement required, is

a. 0.90 m3

b. 0.94 m3

c. 0.98 m3

d. 1.00 m3

43. Pick up the item of work not included in the plinth area estimate

a. Room area

b. Verandah area

c. W.C. area

d. Courtyard area.

44. In the mid-section formula

a. The area of mid-sections is calculated by using mean depth

b. The volume of the earth work is calculated by multiplying the mid-section area by the distance between the two sections

c. The volume of the earth work is calculated by multiplying the mid-section area by the distance between the two original sections

d. (a), (b) and (c) of the above.

45. Henry Gantt developed Bar charts for planning and scheduling of projects in

a. 1900

b. 1920

c. 1940

d. 1950

46. Construction team means

a. an architect

b. an owner

c. a contractor

d. all the above.

47. A goiden rule for the procurement of construction stones, suggests

a. 67% at the site and 33% under procurement

b. 50% at the site and 50% under procurement

c. 33% at the site and 67% under procurement

d. none of these.

48. Pick up the incorrect statement from the following:

a. The difference between the maximum time available for the job and actual time it consumes, is called total float

b. The difference between the latest start time and earliest start time of an activity, is called total float

c. The difference between the earliest finish time of an activity and the earliest start time of its successor activity, is called free float of the activity

d. None of these.

49. The thickness design of the pavement, is decided on the load carried by

a. main gears

b. nose wheel

c. tail wheel

d. all the above.

50. Pick up the correct statement from the following:

a. Rubberised tar concrete hot blast as well as spillage

b. Epoxy asphalt concrete resists the jet fuels and provides more elastic pavement

c. Epoxy asphalt concrete sets in very small time

d. All the above.