***Sub Engineer***

1. Metric chains are generally available in

a. 10 m and 20 m length

b. 15 m and 20 m length

c. 20 m and 30 m length

d. 25 m and 100 m length

2. On a diagonal scale, it is possible to read up to

a. one dimension

b. two dimensions

c. three dimensions

d. four dimensions.

3. Pick up the correct statement from the following :

a. spherical aberration may be minimised by replacing the single lens by a combination of the lenses.

b. in telescope objectives, a combination of convex lens and concave lens is used.

c. in eyepieces, two plano-convex lenses placed at a certain distance apart are used

d. all the above.

4. Chain surveying is well adopted for

a. small areas in open ground

b. small areas with crowded details

c. large areas with simple details

d. large areas with difficult details.

5. Stones used for ornamental work must be

a. soft

b. hard

c. light

d. heavy.

6. Fibre boards can be

a. distempered

b. painted

c. painted and distempered

d. used for furniture.

7. The minimum compressive strength of Ilnd class bricks should be

a. 75 kg/cm2

b. 90 kg/cm2

c. 100 kg/cm2

d. 120 kg/cm2

8. Pick up the correct statement from the following:

a. The articles prepared from clay which is burnt at low temperature and cooled down slowly, are called earthen-ware

b. The articles prepared from refractory clays which as mixed with stone and crushed pottery, are called stone ware

c. The articles prepared from fine earthen ware which is white, thin and semi-transparent, are called procelain

d. All the above.

9. In the truss shown in given figure, the force in member BD is

a. 100 t compressive

b. 100 t tensile

c. zero

d. indeterminate

10. A spring of mean radius 40 mm contains 8 action coils of steel (N = 80000 N/mm2), 4 mm in diameter. The clearance between the coils being 1 mm when unloaded, the minimum compressive load to remove the clearance, is

a. 25 N

b. 30 N

c. 35 N

d. 40 N

11. The stress at which extension of a material takes place more quickly as compared to the increase in load, is called

a. elastic point

b. plastic point

c. breaking point

d. yielding point.

12. The ratio of strengths of solid to hollow shafts, both having outside diameter D and hollow having inside diameter D/2, in torsion, is

a. 1/2

b. 1/16

c. 15/16

d. 3/8

13. Pick up the correct statement from the following :

a. For tranquil flow h > 2g or v <

b. For critical flow

c. For torrential flow h >  or v < gh

d. None of these.

14. The hydrostatic force acts through

a. centre of pressure

b. centre of top edge

c. centre of bottom edge

d. metacentre.

15. Mach number is the ratio of inertia force to

a. viscosity

b. surface tension

c. gravitational force

d. elasticity.

16. An open container filled with water is moved vertically downward with a uniform linear acceleration. The pressure at its bottom will be

a. greater than static pressure

b. equal to static pressure

c. lesser than static pressure

d. none of these.

17. A sample of saturated soil has 30% water content and the specific gravity of soil grains is 2.6. The dry density of the soil mass in g/cm3, is

a. 1.47

b. 1.82

c. 1.91

d. none of these.

18. Pick up the correct statement from the following:

a. The ratio of the liquid limit minus the natural water content to the plasticity index of soils, is called consistency index

b. The ratio of natural water content minus its plastic limit to its plasticity index is called liquidity index

c. The ratio between plasticity index and flow index (i.e. slope of flow curve in case of liquid limit), is called toughness index

d. All the above.

19. The length/diameter ratio of cylindrical specimens used in triaxial test, is generally

a. 1.5

b. 2

c. 2.5

d. 3

20. Negative skin friction on piles

a. is caused due to relative settlement of the soil

b. is caused in soft clays

c. decreases the pile capacity

d. all of the above.

21. In testing a pile by load test, pile platform is loaded with one and half times the design load and a maximum settlement is noted. The load is gradually removed and the consequent rebound is measured. For a safe pile, the net settlement (i.e. total settlement minus rebound) per tonne of test load should not exceed

a. 15 mm

b. 20 mm

c. 25 mm

d. 30 mm

22. If the sides of a slab simply supported on edges and spanning in two directions are equal, the maximum bending moment is multiplied by

a. 0.3

b. 0.4

c. 0.5

d. 0.7

23. If the tendon is placed at an eccentricity e below the centroidal axis of the lon-gitudial axis of a rectangular beam (sectional modulus Z and stressed load P in tendon) the stress at the extreme top edge

a. is increased by

b. is increased by

c. is decreased by

d. remains unchanged.

24. Steel bars are generally connected together to get greater length than the standard length by providing

a. strainght bar splice

b. hooked splice

c. dowel splice

d. all the above

25. Pick up the correct statement from the following :

a. A combined footing may be either rectangular or trapezoidal in shape

b. Rectangular footings are provided if two column loads are equal or interior column carries relatively greater load

c. Trapezoidal shaped footings may be provided under any loading

d. All the above.

26. The entrained concrete is used in lining walls and roofs for making

a. heat insulated

b. sound insulated

c. neither (a) nor (b)

d. both (a) and (b).

27. In the method of voids for determination of the quantity of cement paste, it is assumed that

a. Voids in fine aggregates are filled by the cement paste

b. Volume of fine aggregates is equal to total voids in coarse aggregates plus 10% extra

c. Volume of cement paste required is equal to total volume of voids in fine aggregates plus 15% extra

d. All the above.

28. An ordinary Portland cement when tested for its fineness, should not leave any residue on I.S. seive No. 9, more than

a. 10%

b. 15%

c. 20%

d. 25%

29. Steel pipe are

a. connected by riveted or welded joints

b. generally laid under ground and no expansion joint is required

c. likely to last 100 years under ordinary conditions

d. None of these.

30. The four major water supply distribution systems, are

a. dead end, tree, grid iron and reticulation

b. dead end, tree, grid iron and circular

c. tree, grid iron, ring and radial

d. tree, reticulation, circular and ring.

31. Primary treatment of sewage consists of removal of

a. large suspended organic solids

b. oil and grease

c. sand and girt

d. floating materials

32. If D is the diameter of upper circular portion, the overall depth of New Egg shaped sewer section, is

a. 1.250 D

b. 1.350 D

c. 1.425 D

d. 1.625 D.

33. Lacey's equation for calculating flood discharge in rivers, is

a. V = 10.8 R1/2 S1/2

b. V = 10.8 R2/3 S1/2

c. V = 10.8 R2/3 S1/3

d. V = 10.8 R1/3 S2/3

34. If straight sides of a triangular section of a lined canal with circular bottom of radius R, make an angle θ with horizontal, the area of its cross-section, is

a. R(θ + cot θ)

b. R2(θ + tan θ)

c. R2(θ + cot θ)

d. R(θ + tan θ)

35. Solution of Laplacian equation in three dimensions  of water in a syphon, is done by

a. analytical method

b. Khosla's method

c. method of relaxation

d. Unwin's method.

36. The velocity of drainage water in the barrels of a syphon-aqueduct, is normally limited to

a. 1 to 2 m per second

b. 2 to 3 m per second

c. 3 to 4 m per second

d. 4 to 5 m per second.

37. For clear distinct vision, images of obstructions should fall on the retina with a cone of

a. 3°

b. 4°

c. 5°

d. 6°

38. In water bound macadam roads, binding material, is

a. sand

b. stone dust

c. cement

d. brick dust.

39. Intermediate catch water drains are provided only, if

a. catchment area of the watershed above road is large

b. intensity of rainfall is heavy

c. single catch water drain is inadequate

d. all the above.

40. Volume of traffic which would immediately use a new road or an improved one when opened to traffic, is known

a. development traffic

b. current traffic

c. general traffic

d. normal traffic growth.

41. While preparing a detailed estimate

a. Dimension should be measured correct to 0.01 m

b. Area should be measured correct to 0.01 sqm

c. Volume should be measured correct to O.Olcum

d. All the above.

42. The unit of measurement is per quintal for the following:

a. Collapsible gates with rails

b. Rolling shutters

c. Expanded metal wire netting

d. M.S. reinforcement of R.C.C. works.

43. The expected out turn for earth work in excavation in ordinary soil per mazdoor per day is

a. 1.00 cubic meter

b. 2.00 cubic meter

c. 3.00 cubic meter

d. 4.00 cubic meter

44. Cost of fittings and their fixing is specified for the following sanitary fittings

a. Flushing pipes

b. Lavatary basins

c. Sinks

d. All the above.

45. Power stations are generally treated as

a. light construction

b. heavy construction

c. industrial construction

d. electrical construction.

46. Modular co-ordination of construction means proper

a. planning

b. designing

c. execution

d. all the above.

47. The estimated time required to perform an activity, is known as

a. event

b. dummy

c. duration

d. float.

48. Time and progress chart of a construction, is also known as

a. Gantt chart

b. Modified Mile stone chart

c. Critical path method chart

d. All the above.

49. Pick up the correct statement from the following:

a. The basic length of a runway is increased at a rate of 7% per 300 m of elevation of M.S.L.

b. The standard temperature at the site is obtained by reducing the standard sea level temperature of 15°C at the rate of 6.5°C per 1000 m rise in elevation

c. The aerodrome reference temperature is the monthly mean of the mean daily temperature for the hottest month of the year

d. All the above.

50. Effective length of a runway is the distance between

a. ends of the runway

b. point of intersection of the obstruction clearance line and the extended plane of the runway surface, and the other end of the runway

c. point of intersection of the glide path and the extended plane of the runway surface and the other end of the runway

d. ends of the clear way on either side.