***Sub Engineer***

1. During levelling if back sight is more than foresight

a. The forward staff is at lower point

b. The back staff is at lower point

c. The difference in level, cannot be ascertained.

d. none of these.

2. The horizontal angle between true meridian and magnetic meridian, is known

a. bearing

b. magnetic declination

c. dip

d. convergence.

3. The direction of steepest slope on a contour, is

a. along the contour

b. at an angle of 45° to the contour

c. at right angles to the contour

d. none of these.

4. A traverse deflection angle is

a. less than 90°

b. more than 90° but less than 180°

c. the difference between the included angle and 180°

d. the difference between 360° and the included angle.

5. Quick lime

a. reacts with carbon dioxide

b. may be used for white-washing

c. when mixed with water forms slaked lime

d. all the above.

6. Pick up the metal refractory from the following:

a. Molyblendum

b. Tungsten

c. Zicronium

d. All of these.

7. Wrought iron is used for

a. structural works in beams

b. small sized water pipes

c. columns and struts

d. none to these.

8. Pegmatite is a/an

a. intrusive igneous rock

b. extrusive igneous rock

c. sedimentary rock

d. metamorphic rock.

9. The ratio of lateral strain to axial strain of a homogeneous material, is known

a. Yield ratio

b. Hooke's ratio

c. Poisson's ratio

d. Plastic ratio.

10. The forces acting normally on the cross section of a bar shown in the given figure introduce

a. compressive stress

b. tensile stress

c. shear stress

d. none of these.

11. A beam of length L is pinned at both ends and is subjected to a concentrated bending couple of moment M at its centre. The maximum bending moment in the beam is

a. M

b. M/2

c. M/3

d. ML/2

12. Struts are load carrying members of a frame structure which are subjected to

a. axial tension loads

b. axial compressive loads

c. torsional loads

d. transverse loads.

13. The hydrostatic force acts through

a. centre of pressure

b. centre of top edge

c. centre of bottom edge

d. metacentre.

14. A piezometer opening in pipes measures

a. velocity head

b. static pressure

c. total pressure

d. negative static pressure.

15. Pick up the correct statement from the following :

a. Total energy gradient is the graphical representation of the total head at any section of a pipe line

b. Vertical distance between the total energy line and hydraulic grade line is equal to the velocity head

c. Vertical distance between the total energy line and total energy gradient represents loss of head

d. all the above.

16. The velocity distribution of viscous fluid through a circular/pipe is :

a. hyperbolic

b. circular

c. parabolic

d. elliptical.

17. The ratio of the weight of given volume of soil solids to the weight of an equal volume of distilled water at the given temperature, is known

a. porosity

b. specific gravity

c. void ratio

d. water content.

18. The minimum water content at which the soil retains its liquid state and also possesses a small shearing strength against flowing, is known

a. liquid limit

b. plastic limit

c. shrinkage limit

d. permeability limit.

19. Fundamental relationship between dry density (γd), bulk density (γ) and water content (ω), is :

a. γ =

b. γd =

c. ω =

d. ω =

20. When the seepage pressure becomes equal to the pressure due to submerged weight of a soil, the effective pressure is reduced to zero and the soil particles have a tendency to move up in the direction of flow. This phenomenon is generally known

a. quick condition

b. boiling condition

c. quick sand

d. all the above.

21. The angle of internal friction of soil mass is the angle whose

a. tangent is equal to the rate of the maximum resistance to sliding on any internal inclined plane to the normal pressure acting on the plane

b. sine is equal to the ratio of the maximum resistance to sliding on any internal inclined plane to the normal pressure acting on the plane

c. cosine is equal to the ratio of the maximum resistance sliding on any internal inclined plane to the normal pressure acting on the plane

d. none of these.

22. The stem of a cantilever retaining wall which retains earth level with top is 6 m. If the angle of repose and weight of the soil per cubic metre are 30° and 2000 kg respectively, the effective width of the stem at the bottom, is

a. 52.5

b. 53.5

c. 54.5

d. 55.5

23. The spacing of transverse reinforcement of column is decided by the following consideration.

a. The least lateral dimension of the column

b. Sixteen times the diameter of the smallest longitudinal reinforcing rods in the column

c. Forty-eight times the diameter of transverse reinforcement

d. All the above.

24. Post tensioning system

a. was widely used in earlier days

b. is not economical and hence not generally used

c. is economical for large spans and is adopted now a days

d. none of these.

25. The line of intersection of two surfaces of a sloping roof forming an internal angle less than 180°, is known as

a. ridge

b. hip

c. valley

d. none of these.

26. The line of intersection of the surfaces of a sloping roof forming an external angle exceeding 180°, is

a. ridge

b. hip

c. valley

d. none of these.

27. The minimum percentage of chemical ingredient of cement is that of

a. magnesium oxide

b. iron oxide

c. alumina

d. lime

28. For compacting plain concrete road surface of thickness less than 20 cm, we use

a. internal vibrator

b. screed vibrator

c. form vibrator

d. none of these.

29. For the prediction of future population of a city, the factor to be considered, is

a. births

b. deaths

c. migrants

d. all the above.

30. Maximum threshold number permitted for indicating the odour of public water supplies, is

a. 2

b. 3

c. 4

d. 5

31. To prevent settling down of sewage both at the bottom and on the sides of a large sewer, self-cleaning velocity recommended for Indian conditions, is

a. 0.60 m/sec

b. 0.70 m/sec

c. 0.75 m/sec

d. 1.0 m/sec.

32. The effluent of a septic tank is

a. fit for discharge into any open drain

b. foul and contains dissolved and suspended solids

c. as good as that from a complete treatment

d. none of these.

33. If the straight sides of a triangular section of a lined canal with circular bottom of radius D, make an angle θ with horizontal, the hydraulic mean depth is

a. D/2

b. D/3

c. D/4

d. D/5

34. A fall which maintains its depth, is

a. a trapezoidal notch fall

b. a low weir fall

c. a rectangular notch fall

d. all the above.

35. Pick up the incorrect statement from the following

a. A bank in the cross-section of a canal is provided on the right

b. A bank in the cross-section of a canal is provided on the left

c. A 'dowla' in the cross-section of a canal is provided on the left

d. None of these.

36. The width of a meander belt is the transverse distance between

a. apex point of one curve and apex point of the reserve curve

b. apex point and the crossing

c. two banks of meandering river

d. none of these.

37. If d is the economic designed depth of a slab, the thickness of the cement concrete pavement slab at interior, is kept

a. 1.125 d

b. 0.85 d

c. 0.75 d

d. 0.60 d

38. Bottom-most component of a flexible pavement, is

a. subgrade

b. sub-base

c. base

d. base course.

39. Normal formation width of a hill road for one-way traffic, is

a. 4.8 m

b. 6.6 m

c. 7.2 m

d. 8 m

40. The length of the side of warning sign boards of roads is

a. 30 cm

b. 40 cm

c. 45 cm

d. 50 cm

41. Cost of fittings and their fixing is specified for the following sanitary fittings

a. Flushing pipes

b. Lavatary basins

c. Sinks

d. All the above.

42. The expected out turn of cement concrete 1 : 2 : 4 per mason per day is

a. 1.5 m3

b. 2.5 m3

c. 3.5 m3

d. 5.0 m3

43. The measurement is made in square metre in case of

a. R.C.C. structure

b. Hollow concrete block wall

c. Concrete fencing posts

d. None of these.

44. The inspection pit or chamber is a manhole provided in a base drainage system

a. at every change of gradient

b. at every 30 m intervals

c. at the point where vertical soil pipe joins the house drain

d. All the above.

45. PERT analysis is based on

a. optimistic time

b. pessimistic time

c. most likely time

d. all the above.

46. The Overall in-charge of an organisation at the site responsible for the execution of the works, is

a. Engineer

b. Junior Engineer

c. Sub overseer

d. Assistant Engineer.

47. If the total float and duration of an activity are 5 and 10 days respectively, the particular activity can be

a. started 5 days later

b. completed 5 days later

c. performed at slower rate in 15 days

d. all the above.

48. Pick up the correct statement from the following:

a. The difference of latest occurence time and earliest expected time, is called slack

b. The activities connecting the events having zero slack, lie on the critical path

c. The critical path consumes the maximum time

d. All the above.

49. The thickness design of the pavement, is decided on the load carried by

a. main gears

b. nose wheel

c. tail wheel

d. all the above.

50. The reduced levels of the ends A and B of a runway are 3025 m and 3035 m and that of its mid-point is 3015 m. The reduced level of the horizontal surface, is

a. 3060 m

b. 3075 m

c. 3015 m

d. none of these.