***Sub Engineer***

1. Pick up the correct statement from the following :

a. the apparent error on reversal is twice the actual error

b. the correction may be made equal to half the observed discrepancy.

c. the good results may be obtained from a defective instrument by reversing and taking the mean of two erroneous results

d. all the above.

2. Diurnal variation of magnetic declination is

a. greater at equator than nearer the poles

b. less at equator than nearer the poles

c. less in summer than in winter

d. same at all latitudes and during different months.

3. For a curve of radius 100 m and normal chord 10 m, the Rankine's deflection angle, is

a. 0°35'.95

b. 1°25'.53

c. 1°35'.95

d. 2°51'.53.

4. Resolving power of a telescope depends on

a. the diameter of the aperture

b. the pupil aperture of the eye

c. the diameter of the object glass

d. all the above.

5. Knots in timber are

a. defects caused by crushing fibres

b. splits radiating from the centre

c. speckled strains

d. signs of branches cut off.

6. In   the   method   of   condensation polymerization,

a. low-molecular substances are removed from the high molecular substance

b. the reaction proceeds with an evolution of ammonia

c. the reaction proceeds with an evolution of hydrogen chloride

d. all the above.

7. The method of addition polymerization is used for obtaining:

a. polypropylene

b. polyvinylchloride

c. polystyrene

d. All of these.

8. The wedging is adopted for quarrying costly stratified rock such as

a. marble

b. limestone

c. sandstone

d. All the above.

9. The force in AD of the truss shown in given figure, is

a. 3.0t compression

b. 0.5t compression

c. 0.5t tension

d. zero.

10. Pick up the correct statement from the following:

a. Hoop strain of the walls of a cylinder due to liquid is

b. Longitudinal strain in the walls of a cylinder due to liquid is

c. Volumetric change in the cylinderdue to liquid is

d. All the above.

11. A long vertical member, subjected to an axial compressive load, is called

a. a column

b. a strut

c. a tie

d. a stanchion

12. The under mentioned type is simple strain

a. compressive strain

b. shear strain

c. volumetric strain

d. all the above.

13. In two dimensional flow the components of velocity are given by u = ax; v = by. The stream lines will be

a. circular

b. parabolic

c. hyperbolic

d. elliptical.

14. When a liquid rotates at constant angular velocity about a vertical axis of a rigid body, the pressure

a. varies inversely as the altitude along any vertical line

b. varies as the square of the radial distance

c. decreases as the square of the radial distance

d. none of these.

15. The flow in open channel is said to be subcritical if the Froude number is

a. less than 1.0

b. equal to 1.0

c. greater than 1.0

d. none.

16. The unit of the viscosity is

a. kg sec/m2

b. Newton sec per m2

c. Newton-sec2/m3

d. m2 per sec.

17. The property of a soil which allows it to be deformed rapidly without rupture, elastic rebound and also a volume change, is known

a. porosity

b. plasticity

c. permeability

d. ductility.

18. The fundamental equation of air content (ac), degree of saturation (Sr) and void ratio (e), is

a. ac =

b. ac =

c. ac =

d. ac = 1 + e/Sr

19. The neutral stress in a soil mass is

a. force per neutral area

b. force per effective area

c. stess taken up by the pore water

d. stress taken up by solid particles.

20. The ratio of settlement at any time 't' to the final settlement, is known as

a. co-efficient of consolidation

b. degree of consolidation

c. consolidation index

d. consolidation of undisturbed soil.

21. A foundation is called shallow if its depth, is

a. half of its width

b. three-fourth of its width

c. equal to its width

d. all the above.

22. Columns may be made of plain concrete if their unsupported lengths do not exceed their least lateral dimension

a. three times

b. four times

c. five times

d. six times.

23. On an absolutely rigid foundation base, the pressure will

a. be more at the edges of the foundation

b. be uniform

c. not be uniform

d. be zero at the centre of the foundation.

24. The minimum thickness of the cover at the end of a reinforcing bar should not be less than twice the diameter of the bar subject to a minimum of

a. 15 mm

b. 20 mm

c. 25 mm

d. 30 mm

25. The additional piles which are driven to increase the capacity of supporting loads on vertical piles, are known

a. raking piles

b. eccentric piles

c. sinking piles

d. none of these.

26. Pick up the incorrect statement from the following :

a. In Flemish bond every header in each course lies centrally over every stretcher of the underlying course

b. In English bond, stretchers are laid in every course

c. In English bond, headers and stretchers are laid in alternate courses

d. None of these.

27. The cement becomes useless if its absorbed moisture content exceeds

a. 2%

b. 3%

c. 4%

d. 5%

28. The risk of segregation is more for

a. wetter mix

b. larger proportion of maximum size aggregate

c. coarser grading

d. all the above.

29. The strainer type tube well, is unsuitable for

a. coarse gravels

b. fine sandy strata

c. clean gravels

d. none of these.

30. Disinfection of drinking water, is done to remove

a. odour

b. bacterias

c. turbidity

d. colour.

31. You are asked to design sewer pipes of diameters 0.4 m to 0.9 m at maximum flow, you will assume the sewer flow running at

a. full depth

b. half full

c. two third full

d. three fourth full.

32. A well oxidized sewage contains nitrogen mainly as

a. nitrates

b. nitrites

c. free ammonia

d. none of these.

33. A deficit of sediments in flowing water may cause a river

a. meandering type

b. aggrading type

c. degrading type

d. sub-critical type.

34. Pick up the correct sequence of the part of a canal system from the following

a. Head work-distributary-branch canal-minor

b. Head works-main canal-branch canal-dis-tributary-minor

c. Head works-main canal-branch canal-minor-distributary

d. Heads works-branch canal-main canal distributary, minor.

35. The consumptive use of water for a crop

a. is measured as the volume of water per unit area

b. is measured as depth of water on irrigated area

c. may be supplied partly by precipitation and partly by irrigation

d. all the above.

36. When a canal flowing under pressure is carried below a natural drainage such that its F.S.L. does not touch the underside of the supporting structure, the structure so provided, is called

a. syphon

b. aqueduct

c. super passage

d. syphon-aqueduct.

37. The first stage of deciding the alignment of a hill road, is

a. reconnaissance

b. detailed survey

c. trace-out

d. preliminary survey.

38. According to Indian Road Congress, the width of carriageway, is

a. 7.0 m for two lanes without raised kerbs

b. 7.5 m for two lanes with raised kerbs

c. 10.5 m for three lanes pavements

d. All the above

39. Pick up the incorrect statement from the following. On highways circular curves may be

a. compound curves

b. reverse curves

c. vertical curves

d. all the above.

40. If x% is the gradient of an alignment and y% is the gradient after proper superelevation along a curved portion of a highway, the differential grade along the curve, is

a. (x - y)%

b. (y - x)%

c. (x x y)%

d. (y + x)%

41. The slope of the oultet of 'P trap' below the horizontal is kept

a. 8°

b. 10°

c. 12°

d. 14°

42. The trap which is provided to disconnect the house drain from the street sewer is called

a. Intercepting trap

b. Interception manhole

c. Intercepterchamber

d. All the above.

43. Cost of fittings and their fixing is specified for the following sanitary fittings

a. Flushing pipes

b. Lavatary basins

c. Sinks

d. All the above.

44. Brick walls are measured in sq. m if the thickness of the wall is

a. 10 cm

b. 15 cm

c. 20 cm

d. None of these.

45. Mile Stone charts were invented in the year of

a. 1920

b. 1930

c. 1940

d. 1950

46. In CPM analysis,

a. uncertainties are not allowed

b. activities are represented by arrows

c. beginning and end of an activity, are denoted by nodes

d. all the above.

47. PERT analysis is based on

a. optimistic time

b. pessimistic time

c. most likely time

d. all the above.

48. The first method invented for planning projects, was

a. Bar chart method

b. Milestone chart

c. Critical path method (CPM)

d. Programme Evaluation and Review Technique (PERT)

49. Pick up the component not applicable to aeroplanes

a. engines

b. air screw

c. fuse lage

d. none of these.

50. Pick up the correct statement from the following:

a. Rubberised tar concrete hot blast as well as spillage

b. Epoxy asphalt concrete resists the jet fuels and provides more elastic pavement

c. Epoxy asphalt concrete sets in very small time

d. All the above.