***Sub Engineer***

1. Number of subdivisions per metre length of a levelling staff is

a. 100

b. 200

c. 500

d. 1000

2. Mistakes which may produce a very serious effect upon the final results arise due to

a. in attention

b. in experience

c. carelessness

d. all of these.

3. In tacheometrical observations, vertical staff holding is generally preferred to normal staffing, due to

a. ease of reduction of observations

b. facility of holding

c. minimum effect of careless holding on the result

d. none of these.

4. If S is the length of a subchord and R is the radius of simple curve, the angle of deflection between its tangent and sub-chord, in minutes, is equal to

a. 573 R/S

b. 171.9 S/R

c. 1718.9 R/S

d. 1718.9 S/R.

5. Pick up the correct statement from the following:

a. The plastic sheets are made by the calendering process in which the plastic material is allowed to pass between cylindrical rollers

b. The application of thermo-setting resins on sheets of paper, is called laminating process

c. The plastic articles made by placing raw material in the desired moulds, is known as moulding process

d. All the above.

6. French polish is

a. oil paint

b. distemper

c. spirit varnish

d. none to these.

7. The base material for distemper, is

a. chalk

b. lime

c. lime putty

d. cement wash.

8. Pick up the correct statement from the following:

a. The heating of a material to redness in contact with air, is known as calcination

b. The property of lime by which it sets or hardens in damp places having no free circulation of air is called setting

c. The product that remains after calcination of limestone, is called lime

d. All the above.

9. The ratio of crippling loads of a column having both the ends fixed to the column having both the ends hinged, is

a. 1

b. 2

c. 3

d. 4

10. The load on a spring per unit deflection, is called

a. stiffness

b. proof resilience

c. proof stress

d. proof load.

11. The areas of cross-section of a square beam and a circular beam subjected to equal bending moments, are same.

a. square beam is more economical

b. both the beams are equally strong

c. both the beams are equally economical

d. none of these.

12. The maximum twisting moment a shaft can resist, is the product of the permissible shear stress and

a. moment of inertia

b. polar moment of inertia

c. polar modulus

d. modulus of rigidly.

13. A cylindrical vessel 40 cm high is revolved about its vertical axis so that the water touches the bottom when it just spills out. If the radius of the cylinder is 5 cm, the angular velocity of rotation, is

a. 2 rad/sec

b. 3 rad/sec

c. 4 rad/sec

d. 5 rad/sec

14. An ideal fluid

a. is frictionless and incompressible

b. obeys Newton's law of velocity

c. is similar to gas

d. is very viscous.

15. Frictional loss of head includes the loss of energy due to

a. viscosity

b. turbulence

c. both (a) and (b)

d. none of these.

16. If total head available at the inlet of pipe and f1 is the loss of head due to friction in the pipe, the maximum efficiency of transmission of power (ηmax) is

a. 1/2

b. 2/3

c. 3/4

d. 4/5

17. For slopes of limited extent the surface of slippage, is usually along

a. a parabolic arc

b. an elliptical arc

c. a straight line

d. a circular arc.

18. The capillary rise of water

a. depends upon the force responsible

b. increases as the size of the soil particles increases

c. decreases as the size of the soil particles decreases

d. is less in wet soil than in dry soil.

19. When a cohesionless soil attains quick condition, it looses

a. shear strength

b. bearing capacity

c. both (a) and (b)

d. neither (a) nor (6).

20. The fundamental equation of specific gravity (G), dry density (γd), unit weight of water (γω) and void ratio (e), is

a. e =

b. G =

c. γd =

d. γω =

21. For a ribbed slab

a. clear spacing between ribs shall not be greater than 4.5 cm

b. width of the rib shall not be less than 7.5 cm

c. overall depth of the slab shall not exceed four times the breadth of the rib

d. all the above.

22. Steel beam theory is used for

a. design of simple steel beams

b. steel beams encased in concrete

c. doubly reinforced beams ignoring compressive stress in concrete

d. beams if shear exceeds 4 times allowable shear stress.

23. The self-weight of the footing, is

a. not considered for calculating the upward pressure on footing

b. also considered for calculating the upward pressure on footihg

c. not considered for calculating the area of the footing

d. both (b) and (c)

24. In a prestressed beam carrying an external load W with a bent tendon is having angle of inclination θ and prestressed load P. The net downward load at the centre is

a. W - P cos θ

b. W - P sin θ

c.  W - 2P sin θ

d. W + 2P sin θ

25. The steel pile which is generally sunk in soft clay or loose sand of low bearing capacity, is

a. pipe pile

b. screw pile

c. disc pile

d. none of these.

26. The black cotton soil

a. swells excessively when wet

b. shrinks excessively when dry

c. has a tendency of swelling and shrinking due to clay particles

d. all the above.

27. An aggregate is said to be flaky if its least dimension is less than

a. 2/5th of mean dimension

b. 3/5th of mean dimension

c. 4/5th of mean dimension

d. none of these.

28. According to the recommendations of IS : 456-1978, the expansion joints

a. are provided to ensure minimum resistance

b. are supported on separate columns

c. do not carry reinforcement across them

d. all the above.

29. Surface water is obtained from

a. well

b. springs

c. artesian well

d. rain.

30. The intake opening is generally covered by a screen to prevent entry of debris etc. and its level is kept

a. at the level of water of the source

b. at the bottom of water of the source

c. at about 2.5 m above the bottom

d. none of these.

31. The water carried sewerage system removes

a. domestic sewage

b. industrial sewage

c. storm sewage

d. all the above.

32. R.M.O. expenses include

a. running expenses

b. maintenance expenses

c. operation expenses

d. all the above.

33. If average particle size of the silt in millimetres is m, the Lacey's silt factor f is proportional to

a. m3

b. m1/2

c. m1/3

d. m2/3

34. A deficit of sediments in flowing water may cause a river

a. meandering type

b. aggrading type

c. degrading type

d. sub-critical type.

35. When a canal flowing under pressure is carried below a natural drainage such that its F.S.L. does not touch the underside of the supporting structure, the structure so provided, is called

a. syphon

b. aqueduct

c. super passage

d. syphon-aqueduct.

36. For the stability of a structure against seepage pressure according to Khosla's creep theory, the critical gradient is

a. 0.25

b. 0.50

c. 0.75

d. 1.00

37. Depth of reinforcement below the surface of a concrete pavement, is generally kept

a. 5 cm

b. 6 cm

c. 7 cm

d. 8 cm

38. If the ruling gradient on any highway is 3%, the gradient provided on the curve of 300 metre radius, is

a. 2.25%

b. 2.50%

c. 2.75%

d. 3.00%

39. If the difference in elevation of an edge of the pavement 9 m wide and its crown is 15 cm, the camber of the pavement, is

a. 1 in 45

b. 1 in 30

c. 1 in 15

d. 1 in 7.5

40. Set-back distance is the distance between

a. road land boundary and building line

b. road land boundary and control line

c. building line and control line

d. road land boundary and control line.

41. The value of 'B' of Indian type W.C. shown in the given figure is :

a. 45 cm

b. 50 cm

c. 30 cm

d. 25 cm

42. Pick up the item whose weight is added to the weight of respective item, is

a. Brackets

b. Bolts

c. Distance separators

d. All the above.

43. The floor area includes the area of the balcony up to

a. 100%

b. 75%

c. 50%

d. 25%

44. The assumption on which the trapezoidal formula for volumes is based, is

a. The end sections are parallel planes

b. The mid-area of a pyramid is half the average area of the ends

c. The volume of the prismoidal is over-es-timted and hence a prismoidal correction is applied

d. All the above.

45. Pick up the incorrect statement from the following:

a. The beginning and end of a job, are called events

b. The activity which consumes maximum time, is called a node

c. Logically and sequentially connected activities and events form a network

d. None of these.

46. Critical Path Net Work helps an engineer

a. to concentrate his attention on critical activities

b. to divert the resources from non-critical advanced activities to critical activities

c. to be cautious for avoiding any delay in the critical activities to avoid delay of the whole project

d. all the above.

47. The main principle of an organisation, is

a. cohegency

b. effective control at all levels

c. delegation of authority

d. all the above.

48. Frederick W. Taylor introduced a system of working known as

a. line organisation

b. line and staff organisation

c. functional organisation

d. effective organisation.

49. Conical surface of the approach area rises outwards

a. 1 in 10

b. 1 in 15

c. 1 in 20

d. 1 in 25

50. Pick up the component not applicable to aeroplanes

a. engines

b. air screw

c. fuse lage

d. none of these.