***Sub Engineer***

1. For a closed traverse the omitted measurements may be calculated

a. bearing of one side only

b. both length and bearing of one side

c. length or bearing of adjacent side

d. all the above.

2. A back sight

a. is always taken on a point of known elevation or can be computed

b. is added to the known level to obtain the instrument height

c. taken on an inverted staff is treated as negative

d. all the above.

3. An imaginary line lying throughout on the surface of the earth and preserving a constant inclination to the horizontal, is called

a. contour line

b. contour gradient

c. level line

d. line of gentle scope.

4. In chain surveying tie lines are primarily provided

a. to check the accuracy of the survey

b. to take offsets for detail survey

c. to avoid long offsets from chain lines

d. to increase the number of chain lines.

5. Stones used for the construction of retaining walls must be

a. soft

b. hard

c. light

d. heavy.

6. Pick up the plutonic rock from the following:

a. Granite

b. Dolerite

c. Basalt

d. All the above.

7. Acrylic sheets

a. are generally unaffected by most household detergents

b. possess the light transmission rate of 93%

c. are available in various shapes

d. all the above.

8. For high grade instruments the steel preferred to, is

a. cast steel

b. bessemer steel

c. mild steel

d. whitworth compressed steel.

9. Pick up the correct statement from the following:

a. Hoop strain of the walls of a cylinder due to liquid is

b. Longitudinal strain in the walls of a cylinder due to liquid is

c. Volumetric change in the cylinderdue to liquid is

d. All the above.

10. For beams breadth is constant,

a. depth d ∝ M

b. depth d ∝ M

c. depth d ∝ 3 M

d. depth d ∝

11. The ratio of the flexural strengths of two square beams one placed with its two sides horizontal and the other placed with one diagonal vertical, diagonal, is

a. 2

b. 2

c. 5

d. 7

12. At either end of a plane frame, maximum number of possible bending moments, are

a. two

b. three

c. four

d. zero.

13. Pick up the correct statement regarding Borda's mouth piece running full from the following :

a. Theoretical velocity at the outlet is 2gH

b. Coefficient of velocity is 1/2

c. Coefficient of contraction is 1

d. All the above.

14. For an irrotational flow, the equation  is given by

a. Cauchy-Riemann

b. Reynold

c. Laplaces

d. Bernoulli.

15. A nozzle is fitted at the end of a pipe whose length is 320 m and diameter is 10 cm. If the value of f = 0.01, the diameter of the nozzle for the maximum transmission of power through the nozzle is

a. 2.4 cm

b. 2.5 cm

c. 2.6 cm

d. 2.7 cm.

16. The continuity equation

a. expresses the relationship between work and energy

b. relates the momentum per unit volume between two points on a stream line

c. relates mass rate of flow along a stream line

d. requires that Newton's second law of motion be satisfied at every point in fluid.

17. Darcy's law is applicable to seepage if a soil is

a. homogeneous

b. isotropic

c. incompressible

d. all the above.

18. The water content in a soil sample when it continues to loose weight without loosing the volume, is called

a. Shrinkage limit

b. Plastic limit

c. liquid limit

d. semi-solid limit.

19. The ratio of the weight of given volume of soil solids to the weight of an equal volume of distilled water at the given temperature, is known

a. porosity

b. specific gravity

c. void ratio

d. water content.

20. Minimum size of the particles of silt soil, is

a. 0.002 mm

b. 0.04 mm

c. 0.06 mm.

d. 0.08 mm

21. The effective span of a simply supported slab, is

a. distance between the centres of the bearings

b. clear distance between the inner faces of the walls plus twice the thickness of the wall

c. clear span plus effective depth of the slab

d. none of these.

22. The steel generally used in R.C.C. work, is

a. stainless

b. mildsteel

c. high carbon steel

d. high tension steel.

23. For M 150 mix concrete, according to I.S. specifications, local bond stress, is

a. 10 kg/cm2

b. 15 kg/cm2

c. 20 kg/cm2

d. 25 kg/cm2

24. In a combined footing if shear stress does not exceed 5 kg/cm2, the nominal stirrups provided are

a. 8 legged

b. 10 legged

c. 12 legged

d. none of these.

25. A covering of concrete placed on the exposed top of an external wall, is known as

a. cornice

b. coping

c. frieze

d. lintal.

26. Pick up the correct statement from the following :

a. inclined borings are made for taking samples under existing structures

b. inclined borings are occasionally used instead of vertical holes.

c. the spacing of inclined borings is kept such that one bore hole is vertically above the bottom of an adjacent bore hole.

d. all the above.

27. Pick up the correct statement from the following:

a. The quantity of water required for concreting, depends upon the grading of aggregate amd method of compaction

b. 10% excess of water reduces the strength of concrete by 15%

c. 30% excess of water reduces the strength of concrete by 50%

d. All the above.

28. Pick up the incorrect statement from the following. While performing preliminary test on concrete

a. cement should be mixed by hand in order to maintain uniformity

b. concrete mix should be stored in air-tight containers

c. concrete ingredients should be kept at a temperature of 37° ± 2°C

d. none of these.

29. Slow sand filter is used if maximum turbidity of raw water is less than

a. 20 gm/litre

b. 30 gm/litre

c. 40 gm/litre

d. 50 gm/litre.

30. The bed slope in slow sand filters, is generally kept

a. 1 in 50

b. 1 in 75

c. 1 in 100

d. 1 : 200

31. Pick up the incorrect statement from the following for allowing workers to enter sewers

a. Proper tests for the presence of poisonous gases must be carried out

b. The men entering the manhole should be advised to smoke in the sewer

c. Warning signals should be erected

d. None of these.

32. Which one of the following statements regarding septic tanks is wrong :

a. a gap of 7.5 cm between the bottom of the covering slab and the top level of scum is provided

b. the outlet invert level is kept 5 to 7.5 cm below the inlet invert level

c. the minimum width of septic level is 90 cm

d. the depth of tank is kept equal to its width.

33. The level of the fioor of a syphon aqueduct can be obtained

a. by subtracting the depth of the culvert from the canal bed level

b. by subtracting the thickness of culvert plus the depth of the culvert from the canal bed level

c. both (a) and (b)

d. none of these.

34. Pick up the correct statement from the following

a. The capacity of escapes should not be less than the capacity of the canal at its location

b. The escapes must lead the surplus water to natural drainages

c. The escapes are aligned to take advantage of contours of lower values

d. All the above.

35. The water face of the guide banks, is protected by

a. one men stone pitching

b. two man stone pitching

c. three man stone pitching

d. four man stone pitching

36. Process of meandering is due to

a. sediment load of streams

b. discharge and hydraulic properties of streams

c. relative erodibility of the bed and banks

d. all the above.

37. During last phase of the reconnaissance, details of the grade line is recorded on 2 metre poles to indicate

a. direction of the proposed alignment

b. distance between the previous and forward pegs

c. relative elevations of pegs

d. all the above.

38. Pick up the incorrect statement from the following:

a. Tresguet did not provide the top camber for the drainage of surface water

b. Tresguet provided the top camber for the drainage of surface water

c. Telford provided two layers of stones in the central 5.4 m width and one layer was provided on the sides

d. Macadam provided a camber to the formation at the dug-up state, to drain percolated water.

39. In India the modes of transportation, in the order of their importance, are

a. shipping, roads, railways, air transport

b. roads, railways, air transport, shipping

c. railways, roads, shipping, air transport

d. shipping, railways, roads, air transport.

40. Border Roads Organisation for hilly regions, was formed in

a. 1954

b. 1958

c. 1960

d. 1962

41. The main factor to be considered while preparing a detailed estimate, is

a. Availability of materials

b. Transportation of materials

c. Location of site and local labour charges

d. All the above.

42. The item of steel work which is measured in sq.m, is

a. Rolling shutters

b. Steel doors

c. Ventilators and glazing.

d. All the above.

43. The area is measured correct to the nearest

a. 0.01 sqm

b. 0.02 sqm

c. 0.03 sqm

d. 0.04 sqm

44. The minimum width of a septic tank is taken

a. 70 cm

b. 75 cm

c. 80 cm

d. 90 cm

45. The performance of a specific task in CPM, is known

a. Dummy

b. Event

c. Activity

d. Contract.

46. Pick up the incorrect statement from the following:

a. The beginning and end of a job, are called events

b. The activity which consumes maximum time, is called a node

c. Logically and sequentially connected activities and events form a network

d. None of these.

47. The first method invented for planning projects, was

a. Bar chart method

b. Milestone chart

c. Critical path method (CPM)

d. Programme Evaluation and Review Technique (PERT)

48. The first stage of a construction, is

a. survey of the site

b. initiation of proposal

c. preparation of tender

d. allotment of funds.

49. The bearing of the longest line of a wind rose is S 45° E, the bearing of the runway will be numbered

a. 315°

b. 13

c. 31

d. both (c) and (d)

50. The depressions and undulations in the pavement, are caused due to

a. impact of heavy wheel loads

b. punching effect

c. flexible gears

d. all the above.