***Sub Engineer***

1. The area of a plane triangle ABC, having its base AC and perpendicular height h, is

a.  ba sin C

b.  bc sin A

c. S(S - a)(S - b)(S - c) where S is

d. all the above

2. The probable error of the adjusted bearing at the middle is

a.  rn

b.  rn

c.  rn

d.  rn.

3. Straight, parallel and widely spaced contours represent

a. a steep surface

b. a flat surface

c. an inclined plane surface

d. curved surface.

4. The sag of 50 m tape weighing 4 kg under 5 kg tension is roughly

a. 0.053 m

b. 0.063 m

c. 0.073 m

d. 0.083 m

5. Smith's test of stones is performed to find out

a. the presence of soluble matter of stone

b. the compressive strength of the stone

c. the hardness of the stone

d. the toughness of the stone.

6. Quartzite is a

a. metamorphic rock

b. argillaceous rock

c. calcareous rock

d. silicious rock.

7. Pick up the non-inflammable plastic from the following:

a. Cellulose acetate plastics

b. Polyvinyl chloride plastics

c. Phenol formaldehyde plastic

d. Urea formaldehyde plastic.

8. Pick up the constituent of good brick earth whose excess causes the raw bricks shrink and warp during drying and burning, from the following:

a. Alumina

b. Lime

c. Ironoxide

d. Magnesia.

9. Keeping the depth d constant, the width of a cantilever of length l of uniform strength loaded with a uniformly distributed load w varies from zero at the free end and

a.  at the fixed end

b.  at the fixed end

c.  at the fixed end

d.  at the fixed end

10. Pick up the incorrect statement from the following : The torsional resistance of a shaft is directly proportional to

a. modulus of rigidity

b. angle of twist

c. reciprocal of the length of the shaft

d. moment of inertia of the shaft section.

11. Maximum deflection of a

a. simply supported beam carrying a concentrated load W at mid-span is

b. cantilever beam, carrying a uniformly distributed load over span is

c. simply supported beam carrying a uniformly distributed load over the span is

d. All the above.

12. A cast iron T section beam is subjected to pure bending. For maximum compressive stress to be three times the maximum tensile stress, centre of gravity of the section from flange side is

a. h/4

b. h/3

c. h/2

d. 2/3 h.

13. The flow in open channel is said to be subcritical if the Froude number is

a. less than 1.0

b. equal to 1.0

c. greater than 1.0

d. none.

14. Power transmitted through a pipe is maximum when friction head loss, is

a. one-half of the total head supplied

b. one-third of the total head supplied

c. one-fouth of the total head supplied

d. equal to the total head supplied.

15. The value of kinetic energy correction factor (a) for a laminar flow through a circular pipe, is

a. 1.0

b. l.5

c. 2.0

d. 2.5

16. A non-uniform steady flow is through

a. a long tube at a decreasing rate

b. an expanding tube at constant rate

c. an expanding tube at increasing rate

d. a long pipe at increasing rate.

17. The weight of a pycnometer containing 400 g sand and water full to the top is 2150 g. The weight of pycnometer full of clean water is 1950 g. If specific gravity of the soil is 2.5, the water content is

a. 5%

b. 10%

c. 15%

d. 20%

18. A grillage foundation

a. is provided for heavily loaded isolated columns

b. is treated as spread foundation

c. consists of two sets of perpendicularly placed steel beams

d. all the above.

19. The degree of saturation of the soil sample stated in Q. No, 216, is

a. 60%

b. 62%

c. 64%

d. 66%

20. The liquid limit and plastic limit exist in

a. sandy soils

b. silty soils

c. gravel soils

d. clay soils.

21. The weight of a foundation is assumed as

a. 5% of wall weight

b. 7% of wall weight

c. 10% of wall weight

d. 12% of wall weight

22. The ratio of the breadth to effective depth of a beam is kept

a. 0.50

b. 0.70

c. 0.75

d. none of these.

23. According to the steel beam theory of doubly reinforced beams

a. compression is resisted by compression steel

b. stress in tension steel equals the stress in compression steel

c. no stress is developed in compression concrete as well as in tension concrete

d. all the above.

24. The effective width of a column strip of a flat slab, is

a. half the width of the panel

b. radius of the column

c. diameter of the column

d. none of these.

25. In flat roof of reinforced cement concrete, the recommended angle of slope, is

a. zero

b. a few degrees

c. 10°

d. 200°.

26. The arrangement made to support an unsafe structure temporarily, is known as

a. shoring

b. scaffolding

c. underpinning

d. jacking

27. The bulk density of aggregates, is generally expressed as

a. kg/cubic metre

b. kg/litre

c. g/cm3

d. none of these.

28. Concrete mainly consists of

a. aggregates

b. admixture

c. water

d. all the above.

29. Manholes are less common in

a. cast iron pipes

b. steel pipes

c. hume steel pipes

d. R.C.C. pipes.

30. An aquiclude is

a. a non artesian aquifer

b. an artesian acquifer

c. a confined bed of impervious material between acquifers

d. a large water body under ground.

31. In trickling filter, B.O.D. is reduced to

a. 40 to 60%

b. 60 to 80%

c. 80 to 90%

d. 95%.

32. No treatment of the sewage is given if dilution factor is

a. between 150 to 200

b. between 200 to 300

c. between 400 to 500

d. more than 500

33. For standing crops in undulating sandy fields, the best method of irrigation, is

a. sprinkler irrigation

b. free flooding

c. check method

d. furrow method

34. In a concrete canal the approximate permissible velocity of water should not exceed

a. 1 m/sec

b. 1.5 m/sec

c. 2 m/sec

d. 2.5 m/sec

35. For the conditions enumerated to provide a crossing at C1 You will probably provide

a. an aqueduct

b. a super-passage

c. a syphon aqueduct

d. none of these.

36. The velocity of drainage water in the barrels of a syphon-aqueduct, is normally limited to

a. 1 to 2 m per second

b. 2 to 3 m per second

c. 3 to 4 m per second

d. 4 to 5 m per second.

37. Maximum number of vehicles that can pass a given point on a lane during one hour without creating unreasonable delay, is known as

a. traffic density of lane

b. basic capacity of lane

c. probable capacity of lane

d. practical capacity of lane.

38. For a poorly graded sub-grade soil, thickness of sub-base, is

a. 15 cm

b. 20 cm

c. 25 cm

d. 30 cm

39. The pavement width of a road depends upon

a. terrain

b. type of traffic

c. number of lanes

d. all the above.

40. If the radii of a compound curve and a reverse curve are respectively the same, the length of common tangent

a. of compound curve will be more

b. of reverse curve will be more

c. of both curves will be equal

d. none of these.

41. Pick up the correct statement in case of water supply.

a. Cutting through walls and floors are included with the item

b. Pipes are classified according to their sizes and quality

c. In laying pipes, the method of jointing and fixing is specifically specified

d. All the above.

42. The weight of an item is measured correct to nearest

a. 0.50 kg

b. 0.75 kg

c. 1.00 kg

d. 5 kg

43. The area is measured correct to the nearest

a. 0.01 sqm

b. 0.02 sqm

c. 0.03 sqm

d. 0.04 sqm

44. The rate of payment is made for 100 cu m (per % cu m) in case of

a. Rock cutting

b. Excavation in trenches for foundation

c. Earth work in filling the plinth

d. all the above.

45. A Milestone chart

a. shows the interdependencies of various jobs

b. depicts the delay of jobs, if any

c. points outgoing ahead of schedule of jobs, if any

d. none of these.

46. The object of technical planning, is

a. preparation of estimates

b. initiating the procurement action of resources

c. taking remedial action for likely bottleneck in the execution

d. all the above.

47. If t is the duration of an activity, t1 is the latest finish possible moment of its preceding activity and t2 is the earliest start possible moment, the independent float of the activity is

a. (t1 - t2) - t

b. t - (t1 - t2)

c. (t1 + t2) - t

d. t + (t1 - t2)

48. For the supply of materials for concrete, form work reinforcing and placing of concrete, removal of form work and curing of concrete, number of bar(s) required on bar chart, is

a. 1

b. 2

c. 3

d. 4

49. The landing and take off of the air craft is made against the direction of wind. In no case the centre line of the runway should make an angle with the wind direction exceeding

a. 10°

b. 20°

c. 30°

d. 40°

50. The bearing of the runway at threshold is 290°, the runway number is

a. N 70° W

b. 290°

c. 29°

d. W 20° N