***Sub Engineer***

1. The apparent error on reversal is

a. equal to the actual error

b. twice the actual error

c. thrice the actual error

d. none of these.

2. If the length of a transition curve to be introduced between a straight and a circular curve of radius 500 m is 90 m, the maximum deflection angle to locate its junction point, is

a. 1°43' 08"

b. 1°43' 18"

c. 1°43' 28"

d. 1°43' 38"

3. The slope correction for a length of 30 m along a gradient of 1 in 20, is

a. 3.75 cm

b. 0.375 cm

c. 37.5 cm

d. 2.75 cm.

4. The horizontal angle between true meridian and magnetic meridian, is known

a. bearing

b. magnetic declination

c. dip

d. convergence.

5. Pick up the correct statement from the following:

a. Blistering may be cured by applying water paint finished with oil paint dried with a little copal varnish

b. Cracked paints may be cured by removing paint and giving a fresh coat of paint

c. Crawling paints may be cured by sand preparing the surface and giving a fresh coat with plenty of turps

d. All the above.

6. The commonly used thinner in oil paints, is

a. turpentine

b. both (a) and (b)

c. neither (a) nor (b)

d. none the these.

7. The size of modular bricks, is

a. 10 x 10 x 9 cm

b. 19 x 9 x 9 cm

c. 22.5 x 10 x 8.5 cm

d. 22.5 x 8.0 x 9 cm.

8. In stone masonry, if stones are so placed that their layers are parallel to the direction of load, they

a. split easily

b. are affected by moisture

c. both (a) and (b)

d. none of these.

9. In a shaft, the shear stress is not directly proportional to

a. radius of the shaft

b. angle of twist

c. length of the shaft

d. modulus of rigidity.

10. In case of principal axes of a section

a. sum of moment of inertia is zero

b. difference of moment inertia is zero

c. product of moment of inertia is zero

d. none of these.

11. The reaction at support A of the beam shown in below figure, is

a. zero

b. 5 T

c. 10 T

d. 1 T

12. Maximum deflection of a

a. simply supported beam carrying a concentrated load W at mid-span is

b. cantilever beam, carrying a uniformly distributed load over span is

c. simply supported beam carrying a uniformly distributed load over the span is

d. All the above.

13. An orifice is called a large orifice if water head, is

a. twice the diameter of the orifice

b. thrice the diameter of the orifice

c. four times the diameter of the orifice

d. five times the diameter of the orifice.

14. On an inclined plane, centre of pressure is located

a. at the centroid

b. above the centroid

c. below the centroid

d. anywhere.

15. Uniform flow is said to occur when

a. size and shape of the cross-section in a particular length remain constant

b. size and shape of the cross-section change along a length

c. frictional loss in the particular length of the channel will the more than the drop in its elevation

d. frictional loss in the particular length of the channel, will be less than the drop in elevation.

16. A steady uniform flow is through

a. a long pipe at constant rate

b. an expanding tube at constant rate

c. an expanding tube at increasing rate

d. a long pipe at increasing rate.

17. If voids ratio is 0.67, water content is 0.188 and specific gravity is 2.68, the degree of saturation of the soil, is

a. 25%

b. 40%

c. 60%

d. 75%

18. The consistency index of a soil is defined as the ratio of

a. liquid limit plus the natural water content to the plasticity index of the soil

b. liquid limit minus the natural water content to the plasticity index of the soil

c. natural water content of a soil minus plastic limit to the plasticity index of the soil

d. natural water content of a soil plus its plastic limit to the plasticity index of the soil.

19. Minimum size of the particles of silt soil, is

a. 0.002 mm

b. 0.04 mm

c. 0.06 mm.

d. 0.08 mm

20. Degree of saturation of a natural soil deposit having water content 15%, specific gravity 2.50 and void ratio 0.5, is

a. 50%

b. 60%

c. 75%

d. 80%

21. The stresses developed in concrete and steel in reinforced concrete beam 25 cm width and 70 cm effective depth, are 62.5 kg/cm2 and 250 kg/cm2 respectively. If m = 15, the depth of its neutral axis is

a. 25 cm

b. 30 cm

c. 35 cm

d. 40 cm

22. The diameter of the column head support a flat slab, is generally kept

a. 0.25 times the span length

b. 0.25 times the diameter of the column

c. 4.0 cm larger than the diameter of the column

d. 5.0 cm larger than the diameter of the column

23. In a prestressed beam carrying an external load W with a bent tendon is having angle of inclination θ and prestressed load P. The net downward load at the centre is

a. W - P cos θ

b. W - P sin θ

c.  W - 2P sin θ

d. W + 2P sin θ

24. If the diameter of the main reinforcement in a slab is 16 mm, the concrete cover to main bars is

a. 13 mm

b. 14 mm

c. 15 mm

d. 16 mm

25. The process of filling hollow spaces of walls before plastering, is known

a. dubbing out

b. blishering

c. peeling

d. all the above.

26. The compaction of concrete in the drilled pile hole is done by compressed air in the case of

a. simplex pile

b. Franki pile

c. pressure pile

d. vibro pile.

27. The minimum percentage of chemical ingredient of cement is that of

a. magnesium oxide

b. iron oxide

c. alumina

d. lime

28. A construction joint is provided where

a. bending moment is small

b. shear force is small

c. the member is supported by other member

d. all the above.

29. Pick up the correct statement from the following :

a. Average consumption of commercial use of water is 25% of total consumption

b. Consumption of water on public use is 10% of total consumption

c. Waste water and leakage is 15% of total consumption

d. all the above.

30. The specific retention is least in case of

a. Clay

b. Sand

c. Silt

d. Coarse gravel.

31. If D is the diameter of upper circular portion, the overall depth of a standard egg shaped section, is

a. 1.25 D

b. 1.5 D

c. 1.75 D

d. 2 D.

32.  rd or  th extra space is left in sewer pipes at maximum discharge for

a. low estimates of the average and maximum flows

b. large scale infiltration of storm water

c. unexpected increase in population

d. all the above.

33. Pick up the correct statement from the following:

a. The full supply level of a canal should be above ground level

b. According to Lacey, regime conditions require a particular slope for a given discharge and silt factor

c. In case the ground slope is less than the required bed slope, the silt factor must be reduced by permitting the entry of coarse silt

d. All the above.

34. The ratio of the head recovered to the head put in, is known as

a. efficiency

b. sensitivity

c. flexibility

d. modular limit.

35. If D is the depth of water upstream of the throat above its sill, B is the width of the throat, to achieve critical flow in an open venturi flume, the theoretical maximum flow Q, is

a. Q = 1.71 BD

b. Q = 1.71 BD3/2

c. Q = 1/71 BD2/3

d. none of these.

36. The discharge over a trapezoidal crest of a Sarda fall with free over fall conditions, is

a. 0.45 2g . LH3/2 (H/B)1/3

b. 0.45 2g . LH2/3 (H/B)2/3

c. 0.45 2g . LH3/2 (H/B)1/6

d. 0.45 2g . LH1/6 (H/B)3/2

37. If L is the length of the transition curves provided on either side of a circular curve of radius R, the maximum angle of deflection with tangent for the junctions of the transition curve and circular curve, is

a. L/2R

b. L/3R

c. L/4R

d. none of these.

38. The width of the right of way in urban area, is kept between 24 m to 60 m for

a. National Highways

b. State Highways

c. Both (a) and (b)

d. None of these.

39. If the width of carriage way is 12.5 metres, outer edge 50 cm higher than the inner edge, the required super elevation is

a. 1 in 25

b. 1 in 400

c. 1 in 40

d. 1 in 24

40. For the administration of road transport, a Motor Vehicle Act was enacted in

a. 1934

b. 1939

c. 1947

d. 1950

41. The item of the brick structure measured in sq.m, is

a. Reinforced brick work

b. Broken glass coping

c. Brick edging

d. Brick work in arches.

42. The weight of an item is measured correct to nearest

a. 0.50 kg

b. 0.75 kg

c. 1.00 kg

d. 5 kg

43. The total length of a cranked bar through a distance (d) at 45° in case of a beam of effective length L, is

a. L + 0.42 d

b. L + 2 x 0.42 d

c. L - 0.42 d

d. L - 2 x 0.4 d

44. The order of booking dimensions is

a. Length, breadth, height

b. Breadth, length, height

c. Height, breadth, length

d. None of these.

45. A goiden rule for the procurement of construction stones, suggests

a. 67% at the site and 33% under procurement

b. 50% at the site and 50% under procurement

c. 33% at the site and 67% under procurement

d. none of these.

46. The salient feature of functional organisation is

a. separation of planning and design part

b. each individual maintains functional efficiency

c. work is properly planned and distributed

d. all the above.

47. An event is indicated on the network by a number enclosed in

a. a square

b. a triangle

c. an ellipse

d. all the above.

48. While scheduling a project by C.P.M.

a. required time for each activity is established

b. sequence of various activities is made according to their importance

c. net work is drawn by connecting the activities and the events

d. All the above.

49. To cope up high temperature of 196°C, the taxi ways and aprons are constructed with

a. rubberised tar concrete

b. plain concrete

c. epoxy asphalt concrete

d. all the above.

50. The runway orientation is made so that landing and take off are

a. against the wind direction

b. along the wind direction

c. perpendicular to wind direction

d. none of these.