***Sub Engineer***

1. The power of a lens

a. is reciprocal of its focal length

b. is positive if it is a convex lens

c. is negative if it is a concave lens

d. is measured in diopter.

2. Deviation of the actual road gradient from the proposed contou'r gradient up hill side, involves

a. embankment on the centre line

b. excavation on the centre line

c. earth work on the centre line

d. none of these.

3. In a constant level tube, size of the bubble remains constant because upper wall is

a. of relatively larger radius

b. of relatively smaller radius

c. flat

d. convex downwards.

4. To avoid large centering error with very short legs, observations are generally made

a. to chain pins

b. by using optical system for centering the theodolite

c. to a target fixed on theodolite tripod on which theodolite may be fitted easily

d. all the above.

5. Ultimate strength to cement is provided by

a. Tricalcium silicate

b. Di-calcium silicate

c. Tri-calcium aluminate

d. Tetra calcium alumino ferrite.

6. Lime stones are generally known as

a. aqueous rocks

b. sedimentary rocks

c. stratified rocks

d. all the above.

7. Pick up the correct statement from the following:

a. During rusting, first ferrous bicarbonates are formed

b. On further oxidation ferrous bicarbonates get converted to ferric bicarbonates

c. Ultimately hydrated ferric oxide is formed during rusting and carbon dioxide gets liberated

d. All the above.

8. For obtaining vinyl chloride acetate, the method used, is

a. addition polymerization

b. condensation polymerization

c. co-polymerization

d. none of these.

9. The ratio of crippling loads of a column having both the ends fixed to the column having both the ends hinged, is

a. 1

b. 2

c. 3

d. 4

10. Maximum strain theory for the failure of a material at the elastic limit, is known as

a. St. Venant's theory

b. Rankine's theory

c. Haig's theory

d. Von Mises's theory.

11. The stress in the wall of a cylinder in a direction normal to its longitudinal axis, due to a force acting along the circumference, is known as

a. longitudinal stress

b. hoop stress

c. circumferential stress

d. ultimate stress.

12. In a three hinged arch, the bending moment will be zero

a. at right hinge only

b. at left hinge only

c. at both right and left hinges

d. at all the three hinges.

13. The discharge through an internal mouth piece is more if its length is

a. <  diameter

b. < diameter

c. ≥ diameter

d. none of these.

14. For solving network problems of pipes, necessary condition is

a. continuity equation

b. energy equation

c. Darcy-Weisbach equation

d. all the above.

15. An orifice is taken as large if

a. (H2 - H1) > H

b. (H2 - H1) > H/4

c. (H2 - H1) > H/2

d. (H2 - H1) > H/3

16. With a clinging nappe of a weir, the excess discharge, is

a. 8% to 10%

b. 18% to 20%

c. 25% to 30%

d. 30% to 40%.

17. If there is no impervious boundary at the bottom of a hydraulic structure, stream lines tend to follow :

a. a straight line

b. a parabola

c. a semi-ellipse

d. a semi-circle.

18. A partially saturated sample of soil has a unit weight of 2.0 g/cm3 and specific gravity of soil particles is 2.6. If the moisture content in the soil is 20%, the degree of saturation is

a. 20%

b. 77%

c. 92%

d. none of these.

19. The fundamental equation of air content (ac), degree of saturation (Sr) and void ratio (e), is

a. ac =

b. ac =

c. ac =

d. ac = 1 + e/Sr

20. If W1, W2, W3 and W4 are the sequential weights obtained during observations in pycnometer method for determining water content, the formula to be used, is

a. W =

b. W =

c. W =

d. W =

21. If the loading on a prestressed rectangular beam, is uniformly distributed, the tendon to be provided should be .

a. parabolic with convexity downward

b. parabolic with convexity upward

c. straight above centroidal axis

d. none of these.

22. By over-reinforcing a beam, the moment of resistance can be increased not more than

a. 10%

b. 15%

c. 20%

d. 25%

23. After prestressing process is completed, a loss of stress is due to

a. elastic shortening of concrete

b. creep of concrete

c. creep of steel

d. all the above.

24. Pick up the incorrect statement from the following. The intensity of horizontal shear stress at the elemental part of a beam section, is directly proportional to

a. area of the section

b. distance of the C.G. of the area from its neutral axis

c. moment of the beam section about its neutral axis

d. width of the beam.

25. Stability of an existing structure may be disturbed by

a. vibrations caused by traffic movements

b. mining in the neighbourhood

c. excavation in the neighbourhood

d. all the above.

26. The range of spread from the wall base to outer edge of a brick work foundation does not exceed

a. 1/2 horizontal to 1 vertical

b. 2/3 horizontal to 1 vertical

c. 1 horizontal to 1 vertical

d. 2 horizontals to 1 vertical.

27. The bulk density of aggregates does not depend upon :

a. specific gravity of aggregates

b. grading of aggregates

c. size and shape of the container

d. none of these.

28. If the average compressive strength is 4000 kg/cm2 and standard deviation is 500, the co-efficient of variation is

a. 12.5%

b. 15%

c. 18.5 %

d. 20%

29. The yield of a rapid gravity filter as compared to that of slow sand filter, is

a. 15 times

b. 20 times

c. 30 times

d. 35 times.

30. The valves provided at low points of pipes to drain off water quickly under gravity, are called

a. blow off valves

b. drain valves

c. sewer valves

d. all the above.

31. The sewage treatment units in which anaerobic decomposition of organic matter is used, are called

a. imhoff tanks

b. trickling filters

c. sludge sedimentation tanks

d. none of these.

32. Sewer ventilating columns are generally placed at

a. distances 150 m to 300 m

b. upper ends of branch sewers

c. every change in the size of sewers

d. all the above.

33. For the stability of a structure against seepage pressure according to Khosla's creep theory, the critical gradient is

a. 0.25

b. 0.50

c. 0.75

d. 1.00

34. Cross regulators in main canals are provided

a. to regulate water supply in the distributaries

b. to increase water head upstream when a main canal is running with low supplies

c. to overflow excessive flow water

d. none of these.

35. According to Lacey, in regime conditions

a. silt is kept in suspension by vertical components of eddies

b. entire cross-section of the channel is generated at all points by the forces normal to the wetted perimeter

c. both (a) and (b)

d. neither (a) nor (b).

36. If A is the area of the surface, x is the depth of its C.G. from the surface of the water and ω is the density of water, then

a. total pressure on the surface is equal to ω x

b. depth of the point at which total pressure acts is equal to its moment of inertia divided by A x

c. depth of the centre of pressure is 2/3H vertically below the surface

d. all the above.

37. According to the recommendations of Nagpur Conference, the width formation of an ideal National Highway in hard rock cutting, is

a. 7.9 m

b. 6.9 m

c. 6.5 m

d. 7.5 m

38. According to Indian Road Congress, the width of carriageway, is

a. 7.0 m for two lanes without raised kerbs

b. 7.5 m for two lanes with raised kerbs

c. 10.5 m for three lanes pavements

d. All the above

39. The usual width of side drains along Highways in hilly region, is

a. 60 cm

b. 70 cm

c. 80 cm

d. 100 cm

40. As per recommendations of I.R.C., traffic volume study is carried out for rural roads for 7 days continuously during

a. harvesing

b. lean season

c. harvesting and lean season

d. none of these.

41. The cost of the earthwork in excavation for the surface drain of cross-section shown in the given figure for a total length of 5 metres @ Rs. 450% cum, is

a. Rs. 400

b. Rs. 425

c. Rs. 450

d. Rs. 500

42. A cement concrete road is 1000 m long, 8 m wide and 15 cm thick over the sub-base of 10 cm thick gravel. The box cutting in road crust is

a. 500 m3

b. 1000 m3

c. 1500 m3

d. 2000 m3

43. Berms are provided in canals if these are

a. fully in excavation

b. partly in excavtion and partly in embankment

c. fully in embankment

d. All the above.

44. Pick up the correct statement from the following:

a. In a gully trap, a water seal of 6 to 7.5 cm is provided

b. The gully trap collects waste water from the kitchen, sink, wash basins, etc.

c. The gully trap disconnects the sullage drain from the main drainage system

d. The grating provided over gully traps is 23 cm square.

45. A critical ratio scheduling

a. determines the status of each activity

b. adjusts automatically changes in activity progress

c. is a dynamic system

d. none of these.

46. Which one of the following represents an event ?

a. concrete cured

b. fixing of door

c. plastering of walls

d. selecting sites

47. Which one of the following represents an activity

a. curing of concrete

b. setting of question paper

c. preparation of breakfast

d. all the above.

48. Pick up the correct network for the activities of pouring concrete, erection of form work, removal of form work and curing of concrete from the following:

a. 1. Pouring of Concrete  2. Erection of form work 3. Curing of concrete 4. Removal of form work

b. 1. Erection of form work 2. Pouring of concrete 3. Curing of concrete 4. Removal of form work

c. 1. Removal of form work  2. Erection of form work 3. Pouring of concrete 4. Curing of concrete

d. 1. Pouring of concrete 2. Curing of concrete 3. Erection of form work 4. Removal of form work

49. The coordinates of the ends of a runway are (5000, 5000) and (8000, 7000). The co-ordinates of an other runway are (4600, 5100) and (7000, 5300). The co-ordinates of the A.R.P. are

a. (6500, 6000)

b. (5800, 5200)

c. (61500, 5600)

d. (8000, 7000)

50. The length of runway is increased per 300 m rise above M.S.L.

a. 4%

b. 5%

c. 6%

d. 7%