***Sub Engineer***

1. Diaphragm of a surveying telescope is held inside

a. objective

b. telescope tube at its mid point

c. telescope at the end nearer the eye-piece

d. telescope at its end nearer the objective.

2. For indirect ranging, number of ranging rods required, is

a. 2

b. 3

c. 4

d. 5

3. The method of finding out the difference in elevation between two points for eliminating the effect of curvature and refraction, is

a. reciprocal levelling

b. precise levelling

c. differential levelling

d. flying levelling

4. The bearings of the lines AB and BC are 146° 30' and 68° 30'. The included angle ABC is

a. 102°

b. 78°

c. 45°

d. none of these.

5. Ground glass

a. is made by grinding its one side

b. is made by melting powdered glass paints surface

c. is used for getting light without transparency

d. all the above.

6. Turpentine oil is used in paints as

a. thinner

b. vehicle

c. base

d. drier.

7. For high grade instruments the steel preferred to, is

a. cast steel

b. bessemer steel

c. mild steel

d. whitworth compressed steel.

8. Most commonly used solvent in oil paints, is

a. petroleum

b. spirit

c. coaltar

d. turpentine.

9. The horizontal thrust on the ends of a two hinged semicircular arch of radius R carrying

a. a uniformly distributed load ω per unit run over its right half span, is

b. a uniformly distributed load ω per unit run over its entire span is

c. a distributed load varying from zero at the left end to ω per unit horizontal run at the right end, is

d. all the above.

10. In plastic analysis, the shape factor for a circular section, is

a. 1.5

b. 1.6

c. 1.7

d. 1.75

11. The length of a column, having a uniform circular cross-section of 7.5 cm diameter and whose ends are hinged, is 5 m. If the value of E for the material is 2100 tonnes/cm2, the permissible maximum crippling load will be

a. 12.88

b. 128.8 tonnes

c. 288.0

d. none of these.

12. A three-hinged arch is said to be :

a. statically determinate structure

b. statically indeterminate structure

c. a bent beam

d. none of these.

13. In steady flow, which one of the following changes with time

a. velocity

b. pressure

c. density

d. none of these.

14. Uniform flow is said to occur when

a. size and shape of the cross-section in a particular length remain constant

b. size and shape of the cross-section change along a length

c. frictional loss in the particular length of the channel will the more than the drop in its elevation

d. frictional loss in the particular length of the channel, will be less than the drop in elevation.

15. To avoid the force of surface tension in an inclined manometer, the minimum angle of inclination is

a. 2°

b. 3°

c. 4°

d. 5°

16. A fluid particle may possess the displacement of

a. translation

b. rotation

c. distortion

d. all the above.

17. During seepage through a soil, direction of seepage is always

a. parallel to equipotential lines

b. perpendicular to stream lines

c. perpendicular to equipotential lines

d. none of these.

18. The maximum shear stress occurs on the filament which makes an angle with the horizontal plane equal to

a. 30°

b. 45°

c. 60°

d. 90°

19. The vane shear test is used for the in-situ determination of the undrained strength of the intact fully saturated

a. sands

b. clays

c. gravels

d. highly organic soils.

20. A partially saturated sample of soil has a unit weight of 2.0 g/cm3 and specific gravity of soil particles is 2.6. If the moisture content in the soil is 20%, the degree of saturation is

a. 20%

b. 77%

c. 92%

d. none of these.

21. If the size of a column is reduced above the floor, the main bars of the columns, are

a. continued up

b. bent inward at the floor level

c. stopped just below the floor level and separate lap bars provided

d. all the above.

22. The zone in which transverse bending is likely to occur may be obtained by drawing a line from the faces of the column making an angle θ° with horizontal where θ° is

a. 30°

b. 45°

c. 60°

d. none of these.

23. The width of the flange of a T-beam, which may be considered to act effectively with the rib depends upon

a. overall thickness of the rib

b. centre to centre distance between T-beams

c. span of the T-beam

d. all the above.

24. For M 150 grade concrete (1:2:4) the moment of resistance factor is

a. 8.50

b. 7.50

c. 5.80

d. none of these.

25. Pick up the correct statement from the following :

a. the roof slabs of multi-storeyed buildings are constructed monolithically to carry the various floor loads

b. the beams of multi-storeyed buildings rest on girders and are the main load transferring members to the columns

c. the slab is spanned across the secondary beams provided between the main beams

d. All of these.

26. The type of bond in which every course contains both headers and stretchers, is called

a. English bond

b. Flemish bond

c. Russian band

d. Mixed bond.

27. Pozzolana cement is used with confidence for construction of

a. massive foundations

b. abutments

c. R.C.C. structures

d. all the above.

28. According to the recommendations of IS : 456-1978, the expansion joints

a. are provided to ensure minimum resistance

b. are supported on separate columns

c. do not carry reinforcement across them

d. all the above.

29. Pick up the correct statement from the following :

a. Rain water which percolates through the ground, is free from suspended materials

b. Under ground water may dissolve minerals and salts present in the earth's layers

c. Small quantities of iron, calcium and magnesium dissolved in water, may be useful for human health

d. All the above.

30. A centrifugal pump is required to be primed before starting if it is located

a. at higher level than water level of reservoir

b. at lower level than water level of reservoir

c. both (a) and (b)

d. neither (a) nor (b).

31. In areas where rainy season is limited to a few months, the type of sewerage system recommended is

a. combined system

b. partially separate system

c. separate system

d. none of these.

32. For design purposes, the normal rate of infiltration of ground water into the sewer, is

a. 1000 litres/km/cm

b. 1500 litres/km/cm

c. 2000 litres/km/cm

d. 2500 litres/km/cm.

33. In north Indian Plains, optimum depth of kor watering for wheat, is

a. 19.0 cm

b. 17.5 cm

c. 13.5 cm

d. 12.0 cm

34. The velocity of drainage water in the barrels of a syphon-aqueduct, is normally limited to

a. 1 to 2 m per second

b. 2 to 3 m per second

c. 3 to 4 m per second

d. 4 to 5 m per second.

35. The measure to remove water logging of land, is

a. to reduce percolation from canals and water courses

b. to increase outflow from the ground water reservoir

c. both (a) and (b)

d. neither (a) nor (b)

36. A current meter measures the velocity of flow, if it is held

a. on the surface of water

b. at the bottom of channel

c. at any point within the cross-section

d. none of these.

37. Curves in the same direction separated by short tangents, are called

a. compound curves

b. transition curves

c. broken-back curves.

d. None of these.

38. Which one of the following transportation systems is not used for the conveyance of passengers in India?

a. roads

b. shipping

c. air lines

d. pipe lines.

39. Maximum number of vehicles that can pass a given point on a lane during one hour without creating unreasonable delay, is known as

a. traffic density of lane

b. basic capacity of lane

c. probable capacity of lane

d. practical capacity of lane.

40. Non-passing sight distance along a road is the longest distance at which the driver of a moving vehicle, may see an obstacle on the pavement

a. 10 cm high

b. 25 cm high

c. 50 cm high

d. 100 cm high

41. Pick up the correct statement from the following:

a. Plastering is measured in sq.m

b. Glazing is measured in sq.m

c. Striking is measured in sq.m

d. All the above.

42. The brick work is not measured in cu m in case of

a. One or more than one brick wall

b. Brick work in arches

c. Reinforced brick work

d. Half brick wall.

43. Due to change in price level, a revised estimate is prepared if the sanctioned estimate exceeds

a. 2.0%

b. 2.5%

c. 4.0%

d. 5.0%

44. Size, capacity and materials need be specified for

a. Stop-cocks

b. Ferrules

c. Ball valves

d. All the above.

45. While scheduling a project by C.P.M.

a. required time for each activity is established

b. sequence of various activities is made according to their importance

c. net work is drawn by connecting the activities and the events

d. All the above.

46. A CPM family includes

a. CPP (Critical Path Plotted)

b. MCE (Minimum Cost Expenditure)

c. CPS (Critical Path Scheduling)

d. All the above.

47. The main disadvantage of line organisation, is

a. rigid structure

b. extraordinary delay in communications

c. top level executions over work

d. all the above.

48. A critical ratio scheduling

a. determines the status of each activity

b. adjusts automatically changes in activity progress

c. is a dynamic system

d. none of these.

49. Pick up the correct statement from the following:

a. Minimum turning radius of aircrafts decides the size of the apron and the radius of the curves at taxi-ends

b. Take off and landing distances for an aircraft, determine the minimum runway length

c. The length of the normal haul of the air craft decides the frequency of operation

d. All the above.

50. According to the International Civil Aviation Organisation (I.C.A.O.) the strength of runway pavements, have been coded by

a. Last Seven English alphabets

b. First Seven English alphabets

c. First seven numbers

d. None of these.