***Sub Engineer***

1. Which one of the following procedures for getting accurate orientation is the most distinctive feature of the art of plane tabling

a. radiation

b. intersection

c. traversing

d. resection.

2. The distance between terminal points computed from a subsidiary traverse run between them, is generally known, as

a. traverse leg

b. a base

c. traverse base

d. all the above.

3. The bubble tube is nearly filled with

a. alcohol or chloroform

b. a liquid which is very mobile

c. a liquid having low freezing point

d. all the above.

4. The distance between steps for measuring down hill to obtain better accuracy

a. decreases with decrease of slope

b. increases with increase of slope

c. decreases with increase of slope

d. decreases with decrease of weight of the chain.

5. The slag which floats on the surface of the molten iron generally contains

a. Silica (SiO2) 35%

b. Alumina (Al2O3) 12%

c. MgO, CaSO4, KMnO2 and FeO 8%

d. all the above.

6. The variety of pig iron used for the manufacture of steel by Bessemer process, is

a. Bessemer pig

b. Grey pig

c. White forge pig

d. Mottled pig.

7. Pick up the correct statement from the following:

a. In basic Bessemer process, the steel heats the converter

b. In open-hearth process, the furnace heats the steel

c. In Siemens process, the impurities of pig iron are oxidised by the oxygen of the ore

d. all the above.

8. The presence of original rounded surface on the manufactured piece of timber, is called

a. Wane

b. Torn grain

c. Diagonal grain

d. Chipmark.

9. Pick up the correct statement from the following:

a. For I sections, the shear centre coincides with the centroid of the cross section of the beam

b. For channels, the shear centre does not concide its centroid

c. Bending loads should pass through the shear centre to avoid twisting

d. All the above.

10. A body is said to be in equilibrium if

a. it moves horizontally

b. it moves vertically

c. it rotates about its C.G.

d. none of these.

11. If the beam is supported so that there are only three unknown reactive elements at the supports. These can be determined by using the following fundamental equation of statics

a. ∑V = 0

b. ∑H = 0; ∑H = 0

c. ∑H = 0; ∑V = 0; ∑M = 0

d. ∑M = 0; ∑H = 0

12. A short masonry pillar is 60 cm x 60 cm in cross-section, the core of the pillar is a square whose side is

a. 14.14 cm

b. 20.00 cm

c. 22.36 cm

d. 25.22 cm.

13. Molecules of fluids get attracted due to

a. surface tension

b. adhesion

c. cohesion

d. none of these.

14. When two layers of a fluid separated by dy move over the other with a difference of velocity dv, causes a shearing stress , where μ is known as

a. absolute viscosity

b. dynamic viscosity

c. viscosity

d. all the above.

15. The flow in open channel is said to be critical if the Froude number is :

a. less than 1.0

b. equal to 1.0

c. greater than 1.0

d. None of these.

16. The Empirical formula  for discharge over large rectangular weirs, is known as

a. Francis formula

b. Bazin formula

c. Rehbook formula

d. Kutter's formula.

17. The coefficient of compressibility of soil, is the ratio of

a. stress to strain

b. strain to stress

c. stress to settlement

d. rate of loading to that of settlement.

18. The maximum possible value nf dry density is referred to as

a. dry density

b. zero air voids

c. saturation dry density

d. all the above.

19. Fundamental relationship between dry density (γd), specific gravity (G), water content (ω) and percentage of air voids (na) is :

a. γd =

b. γd =

c. γd =

d. γd =

20. The minimum water content at which the soil just begins to crumble when rolled into threads 3 mm in diameter, is known

a. liquid limit

b. plastic limit

c. shrinkage limit

d. permeability limit.

21. In a simply supported slab the minimum spacing of distribution reinforcement, should be four times the effective thickness of the slab or

a. 30 cm

b. 40 cm

c. 50 cm

d. 60 cm

22. If the diameter of longitudinal bars of a square column is 16 mm, the diameter of lateral ties should not be less than

a. 5 mm

b. 6 mm

c. 8 mm

d. 10 mm

23. The length of lap in tension reinforcement should not be less than the bar diameter x (actual tension / four times the permissible average bond stress) if it is more than

a. 18 bar diameters

b. 24 bar diameters

c. 30 bar diameters

d. 36 bar diameters

24. Design of a two way slab simply supported on edges and having no provision to prevent the corners from lifting, is made by

a. Marcus formula

b. Rankine Grashoff formula

c. Grashoff formula

d. Rankine-Marcus formula.

25. The floor is rubbed with oxalic acid, for making its surface

a. free from voids

b. glossy

c. durable

d. uniform.

26. In grillage foundations, distance between flanges of grillage beams, is kept

a. 40 cm

b. equal to flange width

c. twice the flange width

d. maximum of (a), (b) and (c).

27. Hydration of cement is due to chemical action of water with

a. Tricalcium silicate and dicalcium silicate

b. Dicalcium silicate and tricalcium aluminate

c. Tricalcium aluminate and tricalcium alumino ferrite

d. All the above.

28. For the construction of thin R.C.C. structures, the type of cement to be avoided, is

a. rapid hardening cement

b. low heat cement

c. blast furnace slag cement

d. sulphate resisting cement.

29. Normal values of overflow rate for plain sedimentation tanks in litres/hr/m2, generally range between

a. 100 to 250

b. 250 to 500

c. 500 to 750

d. 750 to 1000

30. Distribution mains of any water supply, is normally designed for its average daily requirement

a. 100%

b. 150%

c. 200%

d. 225%.

31. Dry water flow in a combined sewer, is

a. industrial sewage

b. domestic sewage

c. storm water

d. inclusive of domestic and industrial sewage but excludes storm water.

32. The quantity of liquid waste which flows in sewers during the period of rainfall, is known

a. sanitary sewage

b. industrial waste

c. storm sewage

d. none of these.

33. The water face of the guide banks, is protected by

a. one men stone pitching

b. two man stone pitching

c. three man stone pitching

d. four man stone pitching

34. In rigid module, the discharge

a. is independent of water levels in the distributary and water course

b. depends upon the water level in distributary

c. depends upon the water level in the water course

d. depends upon the water levels of both, i.e. distributary and water course

35. Pick up the correct statement from the following:

a. Static level of water in an open well indicates the position of the water table

b. Saturated pores of a soil in the capillary zone above water table have negative pressure

c. At water table, hydrostatic pressure is zero

d. All the above.

36. If the irrigation efficiency is 80%, conveyance losses are 20% and the actual depth of watering is 16 cm, the depth of water required at the canal outlet, is

a. 15 cm

b. 20 cm

c. 25 cm

d. 30 cm

37. From the point of tangency before an intersection, the route markers are fixed at a distance of

a. 20 m to 35 m

b. 40 m to 50 m

c. 50 m to 75 m

d. 100 m to 150

38. Super-elevation on roads in snow bound areas, should generally not exceed

a. 12%

b. 10%

c. 7%

d. 6%

39. The usual width of side drains along Highways in hilly region, is

a. 60 cm

b. 70 cm

c. 80 cm

d. 100 cm

40. Which one of the following transportation systems is not used for the conveyance of passengers in India?

a. roads

b. shipping

c. air lines

d. pipe lines.

41. Pick up the correct statement from the following:

a. Cement is paid per 50 kg bag

b. Lime is paid per quintal

c. Brick aggregates is paid per cum

d. All the above.

42. If the formation level of a highway has a uniform gradient for a particular length, and the ground is also having a longitudinal slope, the earthwork may be calculated by

a. Mid-section formula

b. Trapezoidal formula

c. Prismoidal formula

d. All the above.

43. The area is measured correct to the nearest

a. 0.01 sqm

b. 0.02 sqm

c. 0.03 sqm

d. 0.04 sqm

44. For 100 sq. m cement concrete (1 : 2: 4) 4 cm thick floor, the quantity of cement required, is

a. 0.90 m3

b. 0.94 m3

c. 0.98 m3

d. 1.00 m3

45. Modular co-ordination of construction means proper

a. planning

b. designing

c. execution

d. all the above.

46. In CPM analysis,

a. uncertainties are not allowed

b. activities are represented by arrows

c. beginning and end of an activity, are denoted by nodes

d. all the above.

47. Pick up the correct statement from the following:

a. Backward pass is used for calculating the latest occurence time

b. Maximum value of earliest expected time is used if there are more than one value of any event

c. Minimum value of latest occurrence time is used if there are more than one value of any event

d. All the above.

48. The time by which activity completion time can be delayed without affecting the start of succeeding activities, is known as

a. duration

b. total flat

c. free float

d. interfering float.

49. Pick up the correct abbreviation from the following:

a. V.H.F, (very high frequency)

b. L.O.M. (low powered outer marker)

c. L.M.M. (low powered middle marker)

d. All the above

50. Pick up the correct statement from the following:

a. Localizer indicates to the pilot his position with respect to the proposed alignment

b. The glide slope indicates the correct angle of descent

c. L.O.M. and L.M.M. help the pilot to judge his position for the runway

d. All the above.