***Sub Engineer***

1. Ranging in chain survey means

a. establishing an intermediate point on the line

b. determining the distance between end points

c. determining the offset distance

d. none of these

2. The method of finding out the difference in elevation between two points for eliminating the effect of curvature and refraction, is

a. reciprocal levelling

b. precise levelling

c. differential levelling

d. flying levelling

3. In tangential tacheometry, an ordinary level staff is used

a. leaning towards the instrument for inclined sights upward

b. leaning away from the instrument for inclined sights downwards

c. vertical in all cases

d. none of these.

4. Pick up the correct statement from the following :

a. with both handles in his left hand, the chain man throws out the chain with his right hand and the second chain man assists him to free it from knots

b. the follower of the chaining operation should be more experienced than the leader

c. at the end of the tenth chain length, the two chain men meet and the ten arrows are handed over to the leader

d. all the above.

5. Cast iron contains carbon approximately

a. 1.5% to 5.5%

b. 0.05% to 1.75%

c. 0.250%

d. none to these.

6. French polish is

a. oil paint

b. distemper

c. spirit varnish

d. none to these.

7. Bullet proof glass is made of thick glass sheet sandwiched by a layer of

a. steel

b. stainless steel

c. high test plastic

d. chromium plate.

8. The percentage of alumina and silica in good fire clay vary respectively is

a. 25, 75

b. 30, 70

c. 35, 65

d. All of these.

9. The normal and tangential components of stress on an inclined plane through θ° to the direction of the force, will be equal if θ is

a. 45°

b. 30°

c. 60°

d. 90°

10. A material may fail if

a. maximum strain exceeds

b. maximum shear stress exceeds

c. total strain energy exceeds

d. all the above.

11. Every material obeys the Hooke's law within its

a. elastic limit

b. plastic point

c. limit of proportionality

d. none of these.

12. The section modulus of a rectangular section is proportional to

a. area of the section

b. square of the area of the section

c. product of the area and depth

d. product of the area and width

13. Total energy line is

a. pressure head

b. datum head

c. kinetic head

d. All the above.

14. The rise of the liquid along the walls of a revolving cylinder above the initial level, is

a. greater than the depression of the liquid at the axis of rotation

b. lesser than the depression of the liquid at the axis of rotation

c. the same as the depression of the liquid at the axis of rotation

d. none of these.

15. If H is height of the liquid above the sill, the effect of end contractions, according to Francis formula, is

a. 0.1 H

b. 0.2 H

c. 0.3 H

d. 0.4 H

16. Due to decrease of diameter of the droplet, inside pressure intensity

a. increases

b. decreases

c. remains unaffected

d. None of these.

17. The length/diameter ratio of cylindrical specimens used in triaxial test, is generally

a. 1.5

b. 2

c. 2.5

d. 3

18. For a base failure of a slope, depth factor

a. Df = 1

b. Df < 1

c. Df > 1

d. none of these.

19. A partially saturated soil is classified as

a. one phase soil

b. two phase soil

c. three phase soil

d. four phase soil.

20. Pick up the correct statement from the following:

a. Illite bond is stronger than montmorillonite bond

b. Illites do not swell when wet

c. Illites are composed of two silica tetrahedralsheets with a central octahedral sheet

d. All the above.

21. The toe projection of foundation slabs is taken

a. as one third of the base

b. as one sixth of overall height of the wall

c. equal to heel slab

d. below ground surface.

22. In case the factor of safety against sliding is less than 1.5, a portion of slab is constructed downwards at the end of the heel slab, which is known as

a. a key

b. a cut-off wall

c. a rib

d. all the above.

23. The zone in which transverse bending is likely to occur may be obtained by drawing a line from the faces of the column making an angle θ° with horizontal where θ° is

a. 30°

b. 45°

c. 60°

d. none of these.

24. The floor slab of a building is supported on reinforced cement floor beams. The ratio of the end and intermediate spans is kept

a. 0.8

b. 0.9

c. 0.6

d. none of these.

25. The alignment of a cross joint along the plumb line is

a. bed block

b. perpend

c. lintel

d. vertical line.

26. Safe bearing capacity of black cotton soil varies from

a. 2 to 3 t/m2

b. 5 to 7.5 t/m2

c. 8 to 10 t/m2

d. 10 to 12 t/m2.

27. The process of mixing, transporting, placing and compacting concrete using Ordinary Port land Cement should not take more than

a. 30 minutes

b. 40 minutes

c. 60 minutes

d. 75 minutes

28. The ratio of various ingredients (cement, sand, aggregates) in concrete of grade M 200, is

a. 1 : 3 : 6

b. 1 :   : 3

c. 1 : 1 : 2

d. 4 : 2 : 1

29. According to IS : 1172-1963, a minimum of 135 litres of water capita per day, is required for

a. Boarding schools

b. Nurses home and medical quarters

c. hostels

d. all the above.

30. The diameter of pipes in bath rooms and lavatories in domestic water supply, is

a. 6 mm

b. 12 mm

c. 18 mm

d. 24 mm.

31. Traps

a. are water seals which prevent the entry of foul gases

b. are used to trap the rats entering sewers

c. dissolve the foul gases

d. create syphonic action to increase the quick disposal of sewerage.

32. In sewers the highest non-scouring velocity is achieved in

a. glazed bricks sewers

b. cast iron sewers

c. cement concrete sewers

d. stone ware sewers.

33. The depth of the crest of a scouring sluice below the crest of a head regulator, is generally kept

a. 1.20 m

b. 2.20 m

c. 3.20 m

d. 4.90 m

34. To control the silt entry into a distributary at head regulator, King's vanes are provided which are the walls

a. of height l/3rd to l/4th the depth of water in the main canal

b. spaced at interval of  times their heights

c. extending about one metre beyond a line drawn at an inclination of 2 : 1 to the axis of main canal

d. all the above.

35. According to Khosla, the exist gradient of surface flow

a. depends upon the b/d ratio

b. is independent of the b/d ratio

c. is independent of the depths of d/s cut off walls

d. none of these.

36. Pick up the correct statement from the following

a. The capacity of escapes should not be less than the capacity of the canal at its location

b. The escapes must lead the surplus water to natural drainages

c. The escapes are aligned to take advantage of contours of lower values

d. All the above.

37. Thickness of broken centre line markings for a four lane road, is generally kept

a. 12 cm

b. 15 cm

c. 18 cm

d. 20 cm

38. If L metres is the distance between extreme axles of a vehicle, its gross load should not exceed

a. 1526 (L + 5.3) - 14.7 L2

b. 1525 (L + 6.3) - 14.7 L2

c. 1525 (L + 7.3) - 14.7 L2

d. 1526 (L + 8.3) - 14.7 L2

39. If D is the degree of a curve, the percentage reduction of gradient, is

a. 0.02 D

b. 0.03 D

c. 0.04 D

d. 0.005 D

40. Transverse joints are provided at distances varying from

a. 12 m to 18 m

b. 16 m to 24 m

c. 17 m to 27 m

d. 25 m to 35 m

41. The plinth area of a building not includes

a. area of stair cover

b. internal shaft for sanitary installations up to 2 sq m. in area

c. lift and wall including landing

d. area of cantilevered porch.

42. According to ISI method of measurement, the order of the sequence is

a. Length, breadth, height

b. Breadth, length, height

c. Height, length, breadth

d. None of these.

43. A cement concrete road is 1000 m long, 8 m wide and 15 cm thick over the sub-base of 10 cm thick gravel. The box cutting in road crust is

a. 500 m3

b. 1000 m3

c. 1500 m3

d. 2000 m3

44. The item of the brick structure measured in sq.m, is

a. Reinforced brick work

b. Broken glass coping

c. Brick edging

d. Brick work in arches.

45. If TL is the latest allowable event occurrence time, total activity slack(s), is equal to

a. LST-EST

b. LFT-EFT

c. TL-EFT

d. all the above.

46. A CPM family includes

a. CPP (Critical Path Plotted)

b. MCE (Minimum Cost Expenditure)

c. CPS (Critical Path Scheduling)

d. All the above.

47. The difference between the time avail-to do a job and the time required to do the job, is known as

a. event

b. float

c. duration

d. constraint.

48. An event is indicated on the network by a number enclosed in

a. a square

b. a triangle

c. an ellipse

d. all the above.

49. Pick up the correct statement from the following:

a. The wind data of an air port is depicted in the form of a chart known as wind rose

b. The landing and take off of the air craft is made against the wind direction

c. The product of wind loading and power loading is called air craft rating

d. All the above.

50. Pick up the correct statement from the following:

a. Approach areas are measured in horizontal surfaces

b. The imaginary inclined plane which is directly above the approach area is called approach surface

c. Obstruction clearance surface and approach surface are same

d. All the above.