***Sub Engineer***

1. There are two stations A and B. Which of the following statements is correct :

a. the fore bearing of AB is AB

b. the back bearing of AB is BA

c. the fore and back bearings of AB differ by 180°

d. all the above.

2. The  lens  equation   is applicable

a. when the thickness of the lens is small

b. only to conjugate distances along the principal axis

c. the computed quantities are accurate enough for geometrical optics of simple distances

d. all the above.

3. An angle of deflection right, may be directly obtained by setting the instrument to read

a. zero on back station

b. 180° on back station

c. 90°

d. 270° on back station.

4. Contours of different elevations may cross each other only in the case of

a. an over hanging cliff

b. a vertical cliff

c. a saddle

d. an inclined plane.

5. Priming consists of

a. one part of white lead, 8 parts of chalk and four parts of twice boiled linseed oil

b. 8 parts of white lead, one part of chalk and four parts of twice boiled linseed oil

c. one part of white lead, 8 parts of chalk and one part of linseed oil

d. none to these.

6. For light and ornamental casting, the most unsuitable pig iron, is

a. Bessemer pig

b. Grey or foundry pig

c. White or forge pig

d. Mottled pig.

7. Which one of the following is acid resistant asbestos:

a. amosite asbestos

b. anthophylite asbestos

c. crocidolite asbestos

d. All the above.

8. In a rock calcium carbonate predominates. State whether it is :

a. Sillicious rock

b. Argillaceous rock

c. Calcareous rock

d. None of these.

9. For determining the support reactions at A and B of a three hinged arch, points B and C are joined and produced to intersect the load line at D and a line parallel to the load line through A at D'. Distances AD, DD' and AD' when measured were 4 cm, 3 cm and 5 cm respectively. The angle between the reactions at A and B is

a. 30°

b. 45°

c. 60°

d. 90°

10. In the truss shown in the given figure, the force in member BC is

a. 100 t compressive

b. 100 t tensile

c. zero

d. indeterminate

11. The bending moment at C of a portal frame shown in below figure is(1st distance 4m)

a. 4 t-m

b. 28 t-m

c. 16 t-m

d. zero.

12. Struts are load carrying members of a frame structure which are subjected to

a. axial tension loads

b. axial compressive loads

c. torsional loads

d. transverse loads.

13. The ratio of the percentagge error in the discharge and percentage error in the measurement of head over a triangular notch, is

a. 2/3

b. 3/2

c. 2/5

d. 5/2

14. Pick up the correct statement from the following :

a. Discharge over a triangular notch is proportional to H5/2

b. Discharge over a rectangular notch is proportional to H3/2

c. Both (a) and (b)

d. Neither (a) nor (b).

15. In a centrifugual pump casing, the flow of water leaving the impeller, is

a. rectilinear flow

b. radial flow

c. free vortex motion

d. forced vertex.

16. To avoid an interruption in the flow of a syphon, an air vessel is provided

a. at the inlet

b. at the outlet

c. at the summit

d. at any point between intet and outlet.

17. Pick up the correct statement from the following :

a. Liquid limit minus plastic limit, is known as plasticity index of the soil

b. Plastic limit minus shrinkage limit, is known as shrinkage index of the soil

c. The ratio of the plasticity index to the flow limit, is known as toughness index of the soil

d. All the above.

18. Pick up the correct definition from the following:

a. The lateral pressure exerted by the soil when the retaining wall moves away from the back fill, is generally known as active earth pressure of the soil

b. The lateral pressure exerted by the soil when the retaining wail moves towards the soil, is generally known as 'Passive earth pressure of the soil'

c. The lateral pressure exerted by the soil when the retaining wall has no movement relative to the back fill, is known as 'earth pressure at rest of the soil'

d. All the above.

19. The specific gravity of quartz, is

a. 2.65

b. 2.72

c. 2.85

d. 2.90

20. A soil not fully consolidated under the existing over-burden pressure, is called

a. pre-consolidated

b. normally consolidated

c. under-consolidated

d. none of these.

21. Top bars are extended to the projecting parts of the combined footing of two columns L distance apart for a distance of

a. 0.1 L from the outer edge of column

b. 0.1 L from the centre edge of column

c. half the distance of projection

d. one-fourth the distance of projection.

22. Columns may be made of plain concrete if their unsupported lengths do not exceed their least lateral dimension

a. three times

b. four times

c. five times

d. six times.

23. In a combined footing for two columns carrying unequal loads, the maximum hogging bending moment occurs at

a. more loaded column

b. a point equidistant from either column

c. a point of the maximum shear force

d. a point of zero shear force.

24. The radius of a bar bend to form a hook, should not be less than

a. twice the diameter

b. thrice the diameter

c. four times the diameter

d. five times the diameter

25. Pick up the incorrect statement from the following :

a. a long vertical load transferring concrete structure is called a concrete pile

b. in pile which transfers the load to the soil by the friction between the pile and the surrounding soil is called friction pile.

c. the pile which transfers the load to a hard rock bed at certain depth is called load bearing

d. none of the these.

26. A pointed arch which forms isosceles or equilateral triangle, is generally known as

a. three centred arch

b. two centred arch

c. Lancet arch

d. Bull's eye arch.

27. In the method of voids for determination of the quantity of cement paste, it is assumed that

a. Voids in fine aggregates are filled by the cement paste

b. Volume of fine aggregates is equal to total voids in coarse aggregates plus 10% extra

c. Volume of cement paste required is equal to total volume of voids in fine aggregates plus 15% extra

d. All the above.

28. Pick up the correct statement from the following:

a. Gypsum (calcium sulphate) acts as an accelerator

b. Gypsum (calcium sulphate) acts as a retarder

c. Calcium chloride acts as an accelerator

d. Both (c) and (d).

29. The period of cleaning of a slow sand filter, is usually

a. two weeks to three weeks

b. one month to three months

c. three months to six months

d. one year.

30. The population of a city in 2000 is 50,000. The average increase in population over last 8 decades is 7500 and average incremental increase during 8 decades is 750. The population of the city based on incremental method, in the year 2020 will be

a. 60,500

b. 66,500

c. 72,500

d. 76,500

31. Cement concrete sewers are only suitable if non-scouring velocity is between

a. 2.5 to 3.0 m/sec

b. 3.0 to 4.0 m/sec

c. 3.5 to 4.5 m/sec

d. 4.5 to 5.5 m/sec.

32. The gradient of sewers depends upon

a. velocity of flow

b. diameter of the sewer

c. discharge

d. all the above.

33. The most suitable location of a canal head work, is

a. boulders stage of the river

b. delta stage of the river

c. rock stage of the river

d. trough stage of the river.

34. Pick up the correct statement from the following

a. Approach of the water line in a flumed channel section should not be steeper than

b. Departure of the water line from a flumed channel section should not be steeper than 30°

c. Approach of the water line should not be steeper than 30° and departure line not steeper than  in a flumed channel section

d. Approach and departure of the water line in a flumed channel section, should not be steeper than

35. A fall which maintains its depth, is

a. a trapezoidal notch fall

b. a low weir fall

c. a rectangular notch fall

d. all the above.

36. Pick up the incorrect statement from the following

a. Side walls of a venturi head flume are splayed out from the end of the throat at 1 : 10 for a length of 4.5 m

b. Length of side walls should be such that the width of the flume is made equal to 2/3rd the bed width of the distributary

c. Once the width of the flume becomes 2/3rd of the width of the distributary, the splayed walls are increased to 1 in 3 to get full bed width

d. None of these.

37. For a properly designed vehicle, the resistance generally ignored, is

a. rolling resistance

b. grade resistance

c. axle resistance

d. none of these.

38. In a right angle bend of a road provided with a transition throughout, the maximum polar angle will be

a. 15°

b. 20°

c. 30°

d. 45°

39. Camber in pavements is provided by

a. straight line method

b. parabola method

c. straight at the edges and parabolic at the crown

d. all the above.

40. Gradient resistance of moving vehicles along down slopes, is

a. + 7 kg/tonne

b. + 9 kg/tonne

c. - 9 kg/tonne

d. - 7 kg/tonne

41. Pick up the correct statement from the following:

a. In a gully trap, a water seal of 6 to 7.5 cm is provided

b. The gully trap collects waste water from the kitchen, sink, wash basins, etc.

c. The gully trap disconnects the sullage drain from the main drainage system

d. The grating provided over gully traps is 23 cm square.

42. The rate of an item of work depends on

a. Specifications of materials

b. Proportion of mortar

c. Method of construction

d. All the above.

43. The unit of measurement is per quintal for the following:

a. Collapsible gates with rails

b. Rolling shutters

c. Expanded metal wire netting

d. M.S. reinforcement of R.C.C. works.

44. Pick up the correct statement from the following:

a. The earth work of cutting in trenches or borrow pits in fairly uniform ground is measured with the help of average depths of the dead men

b. The earth work in trenches or borrow pits in irregular ground is measured by taking the difference in levels before and after completion of work

c. The earth work in trenches or borrow pits, where neither a nor b is feasible, are measured from the fillings after deduction of voids

d. All the above.

45. The most popular type of organisation used for Civil Engineering Constructions, is

a. line organisation

b. line and staff organisation

c. functional organisation

d. effective organisation.

46. The technique for establishing and maintaining priorities among the various jobs of a project, is known

a. Event flow scheduling technique

b. Critical ratio scheduling

c. Slotting technique for scheduling

d. Short interval scheduling.

47. A Milestone chart

a. shows the interdependencies of various jobs

b. depicts the delay of jobs, if any

c. points outgoing ahead of schedule of jobs, if any

d. none of these.

48. The estimated time required to perform an activity, is known as

a. event

b. dummy

c. duration

d. float.

49. If lift off distance of an craft is 2000 m, the clear way at the end of the runway should not be less than

a. 145 m

b. 152.5 m

c. 162.5 m

d. 172.5 m

50. For night landing, the thresholds are lighted

a. green

b. red

c. white

d. yellow