***Sub Engineer***

1. For high sensivity of the bubble tube

a. a liquid of low surface tension is used

b. the bubble space should be long

c. the bubble tube should not be too narrow

d. all the above.

2. The construction of optical square is based, on the principle of optical

a. reflection

b. refraction

c. double refraction

d. double reflection.

3. In tangential tacheometry, an ordinary level staff is used

a. leaning towards the instrument for inclined sights upward

b. leaning away from the instrument for inclined sights downwards

c. vertical in all cases

d. none of these.

4. The distance between terminal points computed from a subsidiary traverse run between them, is generally known, as

a. traverse leg

b. a base

c. traverse base

d. all the above.

5. Which one of the following is an air binding material ?

a. Gypsum

b. Acid-resistant cement

c. Quick lime

d. All of these.

6. For a 50 kg cement bag water required, is

a. 18.5 litres

b. 20.5 litres

c. 22.5 litres

d. 25 litres.

7. Quick setting cement is produced by adding

a. more amount of gypsum in very fine powdered form

b. aluminium sulphate in very fine powdered form

c. pozzolana in very fine powdered form

d. none of these.

8. Minimum of 40% of iron, is available in

a. Red haemetite

b. Limonite

c. Siderite

d. Black band.

9. A shaft rotating N.R.M. under a torque T, transmits a power

a.  Newton metres/sec

b.  Newton metres/min

c.  Newton metres/min

d.  Newton metres/sec

10. Stress may be expressed in Newtons

a. per millimetre square (N/mm2)

b. per centimetre square (N/cm2)

c. per metre square (N/m2)

d. All of the above.

11. Along the neutral axis of a simply supported beam

a. fibres do not undergo strain

b. fibres undergo minimum strain

c. fibres undergo maximum strain

d. none of these.

12. The shear force on a simply supported beam is proportional to

a. sum of the forces

b. sum of the transverse forces

c. algebraic sum of the transverse forces of the section

d. curvature of the neutral axis.

13. The ratio of the inertia and gravitational force acting in any flow, ignoring other forces, is called

a. Euler number

b. Frode number

c. Reynold number

d. Weber number.

14. Pick up the correct statement from the following :

a. In incompressible flow the density of a fluid remains constant

b. In compressible flow, the density of a fluid changes from point to point

c. In uniform flow, the velocity of a fluid does not change with respect to length of flow direction

d. All the above.

15. The maximum vacuum created at the summit of a syphon is

a. 2.7 m of water

b. 7.4 m of water

c. 5.5 m of water

d. none.

16. Cappoletti weir is a

a. rectangular weir whose length is kept 3 times the height of the water above sill

b. triangular weir whose notch angle is 90°

c. trapezoidal weir, whose sides slope 1 horizontal to 2 verticals

d. a combination of rectangular and triangular weirs.

17. The compressibility of clays, is caused due to:

a. expulsion of double layer water from in between the grains

b. sliping of particles to new positions of greater density

c. bending of particles as elastic sheets

d. all the above.

18. Cohesionless soil is

a. sand

b. silt

c. clay

d. clay and silt.

19. The ratio of the volume of water present in a given soil mass to the total volume of its voids, is known

a. porosity

b. void ratio

c. percentage voids

d. degree of saturation.

20. A direct shear test possesses the following disadvantage:

a. A relatively thin thickness of sample permits quick drainage

b. A relatively thin thickness of sample permits quick dissipation of pore pressure developed during the test

c. As the test progresses the area under shear, gradually changes

d. none of these.

21. In a singly reinforced beam, the effective depth is measured from its compression edge to

a. tensile edge

b. tensile reinforcement

c. neutral axis of the beam

d. longitudinal central axis.

22. If the sides of a slab simply supported on edges and spanning in two directions are equal, the maximum bending moment is multiplied by

a. 0.3

b. 0.4

c. 0.5

d. 0.7

23. For initial estimate for a beam design, the width is assumed

a. 1/20th of span

b. 1/25th of span

c. 1/30th of span

d. 1/40th of span.

24. If the depth of actual neutral axis of a doubly reinforced beam

a. is greater than the depth of critical neutral axis, the concrete attains its maximum stress earlier

b. is less than the depth of critical neutral axis, the steel in the tensile zone attains its maximum stress earlier

c. is equal to the depth of critical neutral axis, the concrete and steel attain their maximum stresses simultanesouly

d. all the above.

25. The brick laid with its breadth parallel to the face of a wall, is known as

a. header

b. stretcher

c. closer

d. none of these.

26. A wooden block fixed on back side of a door frame on its post, is known as

a. cleat

b. stop

c. horn

d. none of these.

27. For concreting tunnel linings, tran-portation of concrete is done by

a. wheel borrows

b. containers

c. pumps

d. belt conveyors.

28. Proper proportioning of concrete, ensures

a. desired durability

b. water tightness of the structure

c. resistance to water

d. all the above.

29. Maximum permissible colour for domestic water supplies, based on Cobalt scale, is

a. 10 P.P.M.

b. 15 P.P.M.

c. 20 P.P.M.

d. 25 P.P.M.

30. The efficiency of sedimentation tank does not depend upon

a. depth of tank

b. length of tank

c. detention period

d. velocity of water.

31. If the side of a square sewer is 1000 mm, the diameter of a hydraulically equivalent circular section, is

a. 1045 mm

b. 1065 mm

c. 1075 mm

d. 1095 mm.

32. If the diameter of a sewer is 100 mm, the gradient required for generating self cleansing velocity, is

a. 1 in 60

b. 1 in 100

c. 1 in 120

d. none of these.

33. Lacy's regime condition is obtained if

a. silt grade in the channel is variable

b. discharge in the channel is variable

c. silt charge in the channel is variable

d. channel flows in unlimited, incoherent alluvium of the same character as that transported material.

34. In gravity canals, F.S.L. is

a. always at the ground level

b. always below the ground level

c. generally 4 to 5 metres above the ground level

d. only a few cm above the ground level.

35. Effective precipitation for a crop may be defined as

a. total precipitation minus the loss due to evaporation

b. total precipitation minus the loss due to infiltration

c. total precipitation during the crop period

d. available water stored in soil within root zone of the crop.

36. In a canal syphon, the flow is

a. under atmospheric pressure

b. pipe flow

c. with critical velocity

d. under negative pressure.

37. Design of horizontal and vertical alignments, super-elevation, sight distance and grades, is worst affected by

a. width of the vehicle

b. length of the vehicle

c. height of the vehicle

d. speed of the vehicle

38. If P is the number of vehicles per day at last census, r is the increase in traffic and n is the number of years passed after last census, number of vehicles A per day for design, is

a. P (1 + r)n

b. P (1 - r)n

c. P (1 + r)-n+5

d. P (1 + r)5n

39. The distance travelled by a moving vehicle during perception and brake reaction times, is known as

a. sight distance

b. stopping distance

c. lag distance

d. none of these.

40. Parapet walls along hill roads, are provided

a. to retain the back filling

b. to prevent the hill from sliding

c. to prevent the wheels of the vehicle from coming on the retaining wall

d. none of these.

41. Pick up the correct statement from the following:

a. The diameter of the pipes is the nominal diameter of internal bore

b. All pipes are measured along the centre line of the pipes in metres

c. Lead caulked joints are enumerated separately

d. All the above.

42. Pick up the correct statement from the following:

a. The earth work of cutting in trenches or borrow pits in fairly uniform ground is measured with the help of average depths of the dead men

b. The earth work in trenches or borrow pits in irregular ground is measured by taking the difference in levels before and after completion of work

c. The earth work in trenches or borrow pits, where neither a nor b is feasible, are measured from the fillings after deduction of voids

d. All the above.

43. Pick up the incorrect statement from the following:

a. Lead is the average horizontal straight distance between the borrow pit and the place of spreading soil

b. The lead is calculated for each block of the excavated area

c. The unit of lead is 50 m for a distance upto 500 m

d. The unit of lead is 1 km where the lead exceeds 2 km.

44. According to ISI method of measurement, the order of the sequence is

a. Length, breadth, height

b. Breadth, length, height

c. Height, length, breadth

d. None of these.

45. The technique for establishing and maintaining priorities among the various jobs of a project, is known

a. Event flow scheduling technique

b. Critical ratio scheduling

c. Slotting technique for scheduling

d. Short interval scheduling.

46. For completion of a project, the critical path of the network represents

a. minimum time

b. maximum time

c. maximum cost

d. minimum cost.

47. A critical ratio scheduling

a. determines the status of each activity

b. adjusts automatically changes in activity progress

c. is a dynamic system

d. none of these.

48. Pick up the correct network for the activities of pouring concrete, erection of form work, removal of form work and curing of concrete from the following:

a. 1. Pouring of Concrete  2. Erection of form work 3. Curing of concrete 4. Removal of form work

b. 1. Erection of form work 2. Pouring of concrete 3. Curing of concrete 4. Removal of form work

c. 1. Removal of form work  2. Erection of form work 3. Pouring of concrete 4. Curing of concrete

d. 1. Pouring of concrete 2. Curing of concrete 3. Erection of form work 4. Removal of form work

49. The length of a runway under standard atmospheric conditions is 1800 m. If the actual reduced level of the site is 1200 m, the design length of the runway is

a. 2360 m

b. 2460 m

c. 2560 m

d. 2660 m

50. According to I.C.A.O. the slope of transitional surface at right angles to the centre line of runway, is kept

a. 1 in 5

b. 1 in 6

c. 1 in 7

d. 1 in 10