***Sub Engineer***

1. Pick up the correct statement from the following :

a. in Galileo's telescope, the rays from the object get refracted at the objective and are intercepted by the eyepiece before a real image is formed

b. a line passing through the optical centre of the objective traversing through the eyepiece, is called line of sight

c. the line of sight which passes through the intersection of cross-lines marked on a diaphragm fixed in front of the eyepiece in a plane at right-angles to the axis, is called the line of collimation

d. all the above.

2. While measuring a chain line between two stations A and B intervened by a raised ground

a. vision gets obstructed

b. chaining gets obstructed

c. both vision and chaining get obstructed

d. all the above.

3. If the length of a transition curve to be introduced between a straight and a circular curve of radius 500 m is 90 m, the maximum deflection angle to locate its junction point, is

a. 1°43' 08"

b. 1°43' 18"

c. 1°43' 28"

d. 1°43' 38"

4. A uniform slope was measured by the method of stepping. If the difference in level between two points is 1.8 m and the slope distance between them is 15 m, the error is approximately equal to

a. cumulative, + 0.11 m

b. compensating, ± 0.11 m

c. cumulative, - 0.11 m

d. none of these

5. The colour of statuary marble used for sculptor's work, is

a. blue

b. white

c. green

d. yellow

6. In arches, stratified stones are placed so that their planes are

a. parallel

b. perpendicular

c. radial

d. none of these.

7. Pick up the incorrect statement from the following:

a. Plastics are chemical resistant

b. Plastics are durable

c. Plastics are ductile

d. Plastics are excellent electric insulators.

8. The melting point of silica is :

a. 1570°C

b. 1630°C

c. 1730°C

d. 1850°C.

9.  is the equation of Euler's crippling load, if

a. both the ends are fixed

b. both the ends are hinged

c. one end is fixed and other end is free

d. one end is fixed and other end is hinged.

10. At any point of a beam, the section modulus may be obtained by dividing the moment of inertia of the section by

a. depth of the neutral axis

b. maximum tensile stress at the section

c. maximum compressive stress at the section

d. none of these.

11. Stress in a beam due to simple bending, is

a. directly proportional

b. inversely proportional

c. curvilinearly related

d. none of these.

12. In a shaft rotated by a couple, the shear force varies

a. from zero at the centre to a maximum at the circumference

b. from minimum at the centre of maximum at the circumference

c. from maximum at the centre to zero at the circumference

d. equally throughout the section

13. In a fluid flow a particle may posses

a. kinetic energy

b. pressure energy

c. initial energy

d. all the above.

14. Chezy's constant  is suggested by

a. Bazin

b. Kutter

c. Manning

d. Powell.

15. If the pressure at the inlet of a pipe is 90 kg/cm2 and pressure drop over the pipe line is 10 kg/cm2, the efficiency of transmission, is

a. 66.6%

b. 77.7%

c. 55.5%

d. 88.8%.

16. For solving network problems of pipes, necessary condition is

a. continuity equation

b. energy equation

c. Darcy-Weisbach equation

d. all the above.

17. A saturated soil sample has water content of 40% and specific gravity of soil particles 2.7. The void ratio of the soil, is

a. 0.4

b. 0.52

c. 1.08

d. none of these.

18. The fundamental equation of air content (ac), degree of saturation (Sr) and void ratio (e), is

a. ac =

b. ac =

c. ac =

d. ac = 1 + e/Sr

19. The minimum water content at which the soil retains its liquid state and also possesses a small shearing strength against flowing, is known

a. liquid limit

b. plastic limit

c. shrinkage limit

d. permeability limit.

20. A pile is being driven with a drop hammer weighing 1800 kg and having a free fall of 1.00 m. If the penetration with last blow is 5 mm, the load carrying capacity of the pile, -according to the Engineering News formula, is

a. 100 tonnes

b. 50 tonnes

c. 20 tonnes

d. 10 tonnes.

21. The number of treads in a flight is equal to

a. risers in the flight

b. risers plus one

c. risers minus one

d. none of these.

22. The length of the lap in a compression member is kept greater than bar diameter x (Permissible stress in bar / Five times the bond stress) or

a. 18 bar diameters

b. 24 bar diameters

c. 30 bar diameters

d. 36 bar diameters

23. If At is the gross area of steel in tension, d is the effective depth of the beam and y is the depth of the centre of gravity of the resultant compression, the moment of resistance M of the beam, is given by

a. M = Atf(d - y)

b. M = Atf(d + y)

c. M =

d. none of these.

24. In a simply supported slab, alternate bars are curtailed at

a. 1/5th of the span

b. 1/6th of the span

c. 1/7th of the span

d. none of these.

25. If height of the first storey of a building is 3.2 m and riser is 13 cm, the number of treads required, is

a. 18

b. 24

c. 25

d. 30.

26. In horizontal D.P.C, thickness of cement concrete (1 : 2 : 4) is

a. 4 cm

b. 6 cm

c. 8 cm

d. 10 cm.

27. The percentage of the aggregate of F.M. 2.6 to be combined with coarse aggregate of F.M. 6.8 for obtaining the aggregates of F.M. 5.4, is

a. 30%

b. 40%

c. 50%

d. 60%.

28. Di-calcium silicate (C2S)

a. generates less heat of hydration

b. hardens rapidly

c. provides less ultimate strength to cement

d. has less resistance to sulphate attack.

29. Turbidity of water may be caused due to

a. Suspended clay

b. Suspended silt

c. finely divided organic material

d. all the above.

30. An aquiclude is

a. a non artesian aquifer

b. an artesian acquifer

c. a confined bed of impervious material between acquifers

d. a large water body under ground.

31. In 1000 kg of sewage, the total solids approximate

a. 0.5 to 1.0 kg

b. 1 kg to 2.0 kg

c. 5 kg to 7.5 kg

d. 7.5 kg to 10 kg.

32. The water carried sewerage system removes

a. domestic sewage

b. industrial sewage

c. storm sewage

d. all the above.

33. The sensitivity of a rigid module, is

a. 1.50

b. 1.00

c. 0.50

d. zero

34. The ratio of the discharge over a trapezoidal crest to a rectangular crest of Sarda falls of identical parameters, is

a. 1.084

b. 1.074

c. 1.064

d. 1.054

35. Meandering of a river generally occurs, in

a. rocky stage

b. delta stage

c. boulder stage

d. trough stage.

36. The useful moisture of soil, is equal to its

a. field capacity

b. saturation capacity

c. moisture content at permanent wilting point

d. difference between filed capacity and permanent wilting point within the root zone of plants.

37. The width of road pavements, depends upon

a. width of traffic lane

b. number of traffic

c. width of median strip

d. all the above.

38. Minimum radius of curvature of National Highways or State highways in hill region free from snow, is kept

a. 50 m

b. 33 m

c. 30 m

d. 25 m

39. The width of the right of way, is influenced by

a. shoulders or berms

b. classification of road

c. availability of funds

d. all the above.

40. Over taking time required for a vehicle with design speed 50 km ph and overtaking acceleration 1.25 m/sec2 to overtake a vehicle moving at a speed 30 km ph, is

a. 5.0 secs

b. 6.12 secs

c. 225.48 secs

d. 30 secs

41. Pick up the correct statement from the following:

a. Additional supervising staff engaged at work site, is called work charged establishment

b. Detailed specifications specify qualities, quantities and the proportions of materials to be used for a particular item

c. The cost per unit at which the article can be procured, from the open market at a given time, is called 'market rate'

d. All the above.

42. The 'centre line method' is specially adopted for estimating

a. Hexagonal buildings

b. Octagonal buildings

c. Other geometrical shaped buildings

d. All the above.

43. If the formation level of a highway has a uniform gradient for a particular length, and the ground is also having a longitudinal slope, the earthwork may be calculated by

a. Mid-section formula

b. Trapezoidal formula

c. Prismoidal formula

d. All the above.

44. The correct prismoidal formula for volume is

a. D [first area + last area + ∑ Even area + 2 ∑ odd areas]

b.  [first area + last area + 4 ∑ Even area + 2 ∑ odd areas]

c.  [first area + last area + 2 ∑ Even area + 4 ∑ odd areas]

d.  [first area + last area + 2 ∑ Even area + 4 ∑ odd areas].

45. CPM is

a. is built of activities oriented programme

b. is based on time estimate

c. is used for repetitive works

d. all the above.

46. The main advantage of line organisation, is :

a. defined responsibilities at all levels

b. rigid discipline in the organisation

c. ability of quick decision at all levels

d. all the above.

47. PERT is

a. limited of event oriented diagrams

b. used for research and development projects

c. based on three time estimates for activities linking up two events

d. all the above.

48. Site order book is used for recording

a. instructions by the executive engineers

b. construction measurements

c. issue of store equipments

d. names of the casual labour.

49. The landing and take off of the air craft is made against the direction of wind. In no case the centre line of the runway should make an angle with the wind direction exceeding

a. 10°

b. 20°

c. 30°

d. 40°

50. To cope up high temperature of 196°C, the taxi ways and aprons are constructed with

a. rubberised tar concrete

b. plain concrete

c. epoxy asphalt concrete

d. all the above.