***Sub Engineer***

1. Accuracy of elevation of various points obtained from contour map is limited to

a.  of the contour interval

b.  th of the contour interval

c.  rd of the contour interval

d.  th of the contour interval.

2. Resolving power of a telescope depends on

a. the diameter of the aperture

b. the pupil aperture of the eye

c. the diameter of the object glass

d. all the above.

3. The systematic errors which persist and have regular effects in the performance of a survey operation, are due to

a. carelessness

b. faulty instrument

c. inattention

d. none of these.

4. A dumpy level was set up at mid-point between pegs A and B, 80 m apart and the staff readings were 1.32 and 1.56. When the level was set up at a point 10 m from A on BA produced, the staff readings obtained at A and B were 1.11 and 1.39. The correct staff reading from this set up at S should be

a. 1.435

b. 1.345

c. 1.425

d. none of these.

5. The portion of the brick cut across its width and having its length equal to that of a full brick, is known as

a. queen closer

b. king closer

c. prince closer

d. none of these.

6. A prime coat is given to steel work with

a. an oxide of iron paint

b. a mixture of white lead and lead paint

c. a special paint

d. cement paint.

7. A pug mill is used for

a. moulding brick earth

b. tempering brick earth

c. providing brick earth

d. all the above.

8. The standard size of masonry bricks, is

a. 19 cm x 9 cm x 9 cm

b. 20 cm x 10 cm x 10 cm

c. 21 cm x 11 cm x 11 cm

d. none of these.

9. There are two hinged semicircular arches A, B and C of radii 5 m, 7.5 m and 10 m respectively and each carries a concentrated load W at their crowns. The horizontal thrust at their supports will be in the ratio of

a. 1 : 1 1/2 : 2

b. 2 : 1 1/2 : 1

c. 1 : 1 : 2

d. none of these.

10. The strain energy due to volumetric strain

a. is directly proportional to the volume

b. is directly proportional to the square of exerted pressure

c. is inversely proportional to Bulk modulus

d. all the above.

11. The force in DB of the truss shown in below figure is

a. W tension

b. 2 W compression

c. 5 W tension

d. 4 W tension.

12. The rise of a parabolic arch at quarter points, is equal to

a.  times the rise of the crown

b.  times the rise of the crown

c.  times the rise of the crown

d.  times the rise of the crown.

13. The imaginary line drawn such that the tangents at its all points indicate the direction of the velocity of the fluid particles at each point, is called

a. path line

b. stream line

c. potential line

d. streak line.

14. The maximum vacuum created at the summit of a syphon is

a. 2.7 m of water

b. 7.4 m of water

c. 5.5 m of water

d. none.

15. Total pressure on the top of a closed cylindrical vessesl completely filled with liquid, is directly proportional to

a. (radius)2

b. (radius)3

c. (radius)4

d. none of these.

16. The discharge through a 100 mm diameter external mouth piece fitted to the side of a large vessel is 0.05948 m3/s. The head over the mouth piece is

a. 2 m

b. 2.5 m

c. 3.0 m

d. 4.0 m.

17. The fundamental equation of specific gravity (G), dry density (γd), unit weight of water (γω) and void ratio (e), is

a. e =

b. G =

c. γd =

d. γω =

18. The liquidity index is defined as a ratio expressed as percentage of

a. plastic limit minus the natural water content, to its plasticity index

b. natural water content minus its plastic limit to its plasticity index

c. natural water content plus its plastic limit to its plasticity index

d. liquid limit minus the natural water content to the plasticity index.

19. Failure of the stability of slopes, generally occurs along

a. slip plane

b. a horizontal surface

c. a curved surface

d. all the surfaces.

20. The earth pressure of a soil at rest, is proportional to (where φ is the angle of internal friction of the soil)

a. tan (45° + φ)

b. tan2 (45° - φ)

c. tan2 (45° + φ)

d. none of these

21. If At is the gross area of steel in tension, d is the effective depth of the beam and y is the depth of the centre of gravity of the resultant compression, the moment of resistance M of the beam, is given by

a. M = Atf(d - y)

b. M = Atf(d + y)

c. M =

d. none of these.

22. If diameter of a reinforcement bar is d, the anchorge value of the hook is

a. 8d

b. 12d

c. 16d

d. none of these.

23. Design of R.C.C. simply supported beams carrying U.D.L. is based on the resultant B.M. at

a. supports

b. mid span

c. every section

d. quarter span.

24. A circular slab subjected to external loading, deflects to form a

a. semi-hemisphere

b. ellipsoid

c. parabolloid

d. none of these.

25. The members which support covering material of a sloping roof, are

a. rafters

b. purlins

c. battens

d. struts.

26. The taper of precast concrete pile should not be more than

a. 1 cm per metre length

b. 2 cm per metre length

c. 4 cm per metre length

d. 5 cm per metre length.

27. To prevent segregation, the maximum height for placing concrete, is

a. 100 cm

b. 125 cm

c. 150 cm

d. 200 cm

28. The mixture of different ingredients of cement, is burnt at

a. 1200°C

b. 1400°C

c. 1600°C

d. 1800°C

29. Surge tanks are used

a. for storage water

b. to increase the velocity in a pipeline

c. as overflow valves

d. to guard against water hammer.

30. Pick up the incorrect statement from the following :

a. Iron salts produce heavy flocs and hence remove more suspended matter

b. Iron salts remove hydrogen sulphides

c. Iron salts can be used over a limited range of pH values

d. Iron salts impart corrosiveness to water.

31. Bio-chemical oxygen demand (BOD) for the first 20 days in generally referred to

a. initial demand

b. first stage demand

c. carbonaceous demand

d. all of these.

32. The normal values of over flow rates for sedimentation tanks using coagulent, ranges between

a. 40, 000 to 50, 000 litres/sqm/day

b. 50, 000 to 60, 000 litres/sqm/day

c. 80, 000 to 100, 000 litres/sqm/day

d. none of these.

33. Canals constructed for draining off water from water logged areas, are known

a. drains

b. inundation canals

c. valley canals

d. contour canals

34. Pick up the incorrect statement from the following:

a. In free flooding irrigation, water is admitted at one corner of a field and is allowed to spread over the entire area

b. In check method of irrigation, the field is divided into smaller compartments and water is admitted to each in turn

c. In furrow irrigation water is admitted between the rows of plants in the field

d. None of these.

35. According to Bligh's creep theory, percolating water flows along

a. straight path under the foundation of the dam

b. circular path under the foundation of the dam

c. the outline of the base of the foundation of the dam

d. none of these.

36. Attracting groynes are built

a. perpendicular to the bank

b. inclined down stream

c. inclined up stream

d. none of these.

37. Formation width of a hill road is the distance between

a. outer end of side drain to outer end of parapet wall

b. outer end of side drain to inner end of parapet wall

c. inner end of side drain to outer end of parapet wall

d. inner end of side drain to inner end of parapet wall

38. If R is the radius of a main circular curve, Δ is the angle of deflection and α is the polar deflection angle of any point, its radial distance is

a. 3 R sin Δ/3 sin 2α

b. 3 R sin Δ/2 sin 3α

c. 3 R sin Δ/3 sin α/2

d. 3 R sin 2Δ sin α

39. Alignment of highways in hilly regions, is decided on

a. number of river crossings

b. natural unstable areas

c. saddles or passes

d. all the above.

40. Full amount of extra width of a pavement on the curve, is provided at

a. beginning of the transition curve

b. centre of the transition curve

c. beginning of the circular curve

d. centre of the circular curve.

41. If the formation level of a highway has a uniform gradient for a particular length, and the ground is also having a longitudinal slope, the earthwork may be calculated by

a. Mid-section formula

b. Trapezoidal formula

c. Prismoidal formula

d. All the above.

42. Pick up the correct statement from the following:

a. The actual expenditure involved to complete a work including incidental, establishment and travelling charges, is called actual cost

b. The formal acceptance by the administrative department for incurring an expenditure on the work, is called administrative approval

c. The order of a competent authority sanctioning a properly detailed estimate of the cost of a work of construction or repair is called technical sanction

d. All the above.

43. The measurement is made for stone work in square metre in case of

a. Columns, lintels, copings

b. Building work

c. Dressed stones in Chajjas

d. (a) and (d) of the above.

44. Pick up the correct statement regarding the centre line method of estimating a building

a. Product of the centre line of the walls and area of cross-section of any item, gives total quantity of the item

b. The centre line is worked out separately for different sections of walls of a building

c. The centre line length is reduced by half the layer of main wall joining the partition wall

d. All the above.

45. Which one of the following represents an event ?

a. concrete cured

b. fixing of door

c. plastering of walls

d. selecting sites

46. While filling the tender for any work, the contractor considers

a. availability of construction materials

b. availability of labour

c. study of specifications

d. all the above.

47. Pick up the incorrect statement from the following:

a. The tail of the arrow indicates the start of the activity

b. The head of the arrow indicates the end of the activity

c. The arrows are drawn to scale from left to right

d. Each activity consumes a given time.

48. In CPM analysis,

a. uncertainties are not allowed

b. activities are represented by arrows

c. beginning and end of an activity, are denoted by nodes

d. all the above.

49. The landing and take off of the air craft is made against the direction of wind. In no case the centre line of the runway should make an angle with the wind direction exceeding

a. 10°

b. 20°

c. 30°

d. 40°

50. The coordinates of the ends of a runway are (5000, 5000) and (8000, 7000). The co-ordinates of an other runway are (4600, 5100) and (7000, 5300). The co-ordinates of the A.R.P. are

a. (6500, 6000)

b. (5800, 5200)

c. (61500, 5600)

d. (8000, 7000)