***Sub Engineer***

1. Diaphragm of a surveying telescope is held inside

a. objective

b. telescope tube at its mid point

c. telescope at the end nearer the eye-piece

d. telescope at its end nearer the objective.

2. Tacheometric formula for horizontal distances using horizontal sights can also suitable be employed for inclined sights through θ by multiplying

a. the constants by cos2 θ

b. the constants by cos θ

c. the constants by sin θ

d. the multiplying constant by cos2 θ and additive constant by cos θ.

3.  is the conventional sign of

a. temple

b. mosque

c. idgah

d. church

4. Volume of the earth work may be calculated by

a. end areas

b. Prismoidal formula

c. Trapezoidal

d. all the above.

5. Pick up the correct statement from the following:

a. The edges of a finished wrought iron, are rough due to red shortage

b. Pig iron (charcoal) is manufactured from magnetic ore (Fe3O4)

c. For the manufacture of wrought iron, non-sulphurous fuel is not necessary

d. All the above.

6. Pick up the metal refractory from the following:

a. Molyblendum

b. Tungsten

c. Zicronium

d. All of these.

7. Which one of the following polymers is obtained from condensation polymerization?

a. phenol formaldehyde

b. carbamide

c. melamine-formaldehyde

d. all of these.

8. The specific gravity of marble, is

a. 2.60

b. 2.66

c. 2.72

d. 3.00.

9. The force in EC of the truss shown in the given figure, is

a. 5t tension

b. 5t compression

c. 4t tension

d. None of these.

10. For beams breadth is constant,

a. depth d ∝ M

b. depth d ∝ M

c. depth d ∝ 3 M

d. depth d ∝

11. Maximum deflection of a

a. simply supported beam carrying a concentrated load W at mid-span is

b. cantilever beam, carrying a uniformly distributed load over span is

c. simply supported beam carrying a uniformly distributed load over the span is

d. All the above.

12. The bending moment is maximum on a section where shearing force

a. is maximum

b. is minimum

c. is equal

d. changes sign.

13. The ratio of frictional factor and coefficient of friction used in general equation for a head loss in a pipe, is

a. 1

b. 2

c. 3

d. 4

14. For the most economical trapezoidal open channel,

a. half of the top width must be equal to one of the sloping sides

b. the hydraulic mean depth must be equal to half the depth of flow

c. the semicircle drawn with top width as diameter must touch the three sides of the channel

d. All of these.

15. The metacentric height of a body equals the distance between

a. the centre of gravity and centre of buoyancy

b. the metacentre and centre of gravity

c. the centre of buoyancy and metacentre

d. none of these.

16. The momentum correction factor (β) for the viscous flow through a circular pipe is

a. 1.33

b. 1.50

c. 1.66

d. 2.00

17. A fundamental equation of void ratio(e), specific gravity (G), water content (ω) and degree of saturation (Sr) is

a. e =

b. ω =

c. G =

d. Sr =

18. The property of a soil which permits water to percolate through it, is called

a. moisture content

b. permeability

c. capillarity

d. none of these.

19. For determining the ultimate bearing capacity of soil, the recommended size of a square bearing plate to be used in load plate test should be 30 to 75 cm square with a minimum thickness of

a. 10 mm

b. 15 mm

c. 20 mm

d. 25 mm

20. The vane shear test is used for the in-situ determination of the undrained strength of the intact fully saturated

a. sands

b. clays

c. gravels

d. highly organic soils.

21. An R.C.C. beam of 6 m span is 30 cm wide and has a lever arm of 55 cm. If it carries a U.D.L. of 12 t per m and allowable shear stress is 5 kg/cm2, the beam

a. is safe in shear

b. is safe with stirrups

c. is safe with stirrups and inclined bars

d. needs revision of section

22. Pick up the incorrect statement from the following: Tensile reinforcement bars of a rectangular beam

a. are curtailed if not required to resist the bending moment

b. are bent up at suitable places to serve as shear reinforcement

c. are bent down at suitable places to serve as shear reinforcement

d. are maintained at bottom to provide at least local bond stress.

23. If T and R are tread and rise respectively of a stair, then

a. 2R + T = 60

b. R + 2T = 60

c. 2R + T = 30

d. R + 2T= 30

24. The reinforced concrete beam which has width 25 cm, lever arm 40 cm, shear force 6t/cm2, safe shear stress 5 kg/cm2 and B.M. 24 mt,

a. is safe in shear

b. is unsafe in shear

c. is over safe in shear

d. needs redesigning.

25. The steel pile which is generally sunk in soft clay or loose sand of low bearing capacity, is

a. pipe pile

b. screw pile

c. disc pile

d. none of these.

26. The ceiling height of a building is

a. between ceiling and ground level

b. between ceiling and floor level

c. upto roof above ground level

d. upto ceiling from the ground level.

27. The ratio of various ingredients (cement, sand, aggregates) in concrete of grade M 200, is

a. 1 : 3 : 6

b. 1 :   : 3

c. 1 : 1 : 2

d. 4 : 2 : 1

28. The bulk density of aggregates does not depend upon :

a. specific gravity of aggregates

b. grading of aggregates

c. size and shape of the container

d. none of these.

29. E. Coli bacterias die in water having pH greater than

a. 6.5

b. 7.5

c. 8.5

d. 9.5

30. Pick up the correct statement from the following regarding radial flow centrifugal pumps :

a. In involute type of radial flow centrifugal pump, the impeller discharges into a gradually expanding spiral casing

b. In turbine type of radial flow centrifugal pumps, the impeller is surrounded by stationary guide vanes to reduce the velocity of water

c. The efficiency of turbine type of radial flow centrifugal pump, is always higher than that of volute type

d. All the above.

31. In a fully mechanised composting plant, involves

a. mechanized receipt

b. mechanized segregation

c. mechanized pulverising of refuse

d. all of these.

32. Dried sewage after treatment is used as

a. fertilizer

b. building material

c. chemical for lowering B.O.D.

d. base material for paints.

33. If the straight sides of a triangular section of a lined canal with circular bottom of radius D, make an angle θ with horizontal, the hydraulic mean depth is

a. D/2

b. D/3

c. D/4

d. D/5

34. To control the silt entry into a distributary at head regulator, King's vanes are provided which are the walls

a. of height l/3rd to l/4th the depth of water in the main canal

b. spaced at interval of  times their heights

c. extending about one metre beyond a line drawn at an inclination of 2 : 1 to the axis of main canal

d. all the above.

35. For diversion of flood water of rivers, the type of canal constructed, is

a. ridge canal

b. perennial canal

c. inundation canal

d. canal.

36. For a given discharge in a channel, Blench curves give the relationship between the loss of head (HL) and

a. specific energy up-stream

b. specific energy down-stream

c. critical depth of water down-stream

d. depth of water down-stream

37. If the elevations along a road increase, the slope of the road along the longitudinal direction, is known as

a. gradient

b. grade

c. positive grade

d. negative grade.

38. To indicate proper control of consistency of a freshly mixed concrete for pavement construction, the slump should be between

a. 4 to 6 cm

b. 5 to 7 cm

c. 7 to 10 cm

d. 10 to 12 cm

39. The difference in gradients after full super-elevation and the initial alignment of a road, is known as

a. rising gradient

b. compensated gradient

c. differential gradient

d. none of these.

40. If cross slope of a country is 10% to 25%, the terrain is classified as

a. rolling

b. mountainous

c. steep

d. plain.

41. The expected out turn for earth work in excavation in ordinary soil per mazdoor per day is

a. 1.00 cubic meter

b. 2.00 cubic meter

c. 3.00 cubic meter

d. 4.00 cubic meter

42. The trap which is provided to disconnect the house drain from the street sewer is called

a. Intercepting trap

b. Interception manhole

c. Intercepterchamber

d. All the above.

43. Size, capacity and materials need be specified for

a. Stop-cocks

b. Ferrules

c. Ball valves

d. All the above.

44. The value of 'B' of Indian type W.C. shown in the given figure is :

a. 45 cm

b. 50 cm

c. 30 cm

d. 25 cm

45. Frederick W. Taylor introduced a system of working known as

a. line organisation

b. line and staff organisation

c. functional organisation

d. effective organisation.

46. The estimated time required to perform an activity, is known as

a. event

b. dummy

c. duration

d. float.

47. Works costing less than Rs. 20,000 are treated as

a. projects

b. major projects

c. minor projects

d. all the above.

48. If the total float and duration of an activity are 5 and 10 days respectively, the particular activity can be

a. started 5 days later

b. completed 5 days later

c. performed at slower rate in 15 days

d. all the above.

49. According to I.C.A.O. all markings on the runways are painted white and on taxiways

a. black

b. red

c. yellow

d. green

50. To cope up high temperature of 196°C, the taxi ways and aprons are constructed with

a. rubberised tar concrete

b. plain concrete

c. epoxy asphalt concrete

d. all the above.