***Sub Engineer***

1. Pick up the correct statement from the following :

a. A level line lies in level surface

b. A horizontal surface is normal to the direction of gravity at only one point

c. A horizontal line is tangential to the level surface

d. All the above.

2. Which one of the following procedures for getting accurate orientation is the most distinctive feature of the art of plane tabling

a. radiation

b. intersection

c. traversing

d. resection.

3. In an internal focusing telescope

a. the objective is at a fixed distance from the diaphragm

b. the focusing is done by the sliding of a divergent lens.

c. the focusing divergent lens is situated at about the middle of the tube

d. all the above.

4. The combined effect of curvature and refraction over a distance L kilometres is

a. 67.2 L2 mm

b. 76.3 L2 mm

c. 64.5 L2 mm

d. none of these.

5. Brittleness of cold is due to an excess of

a. sulphur

b. carbon

c. phosphorus

d. silicon.

6. The main constituent of fly-ash, is

a. aluminium oxide

b. silica

c. ferrous oxide

d. All of these.

7. To retard the initial setting time of cement, the compound responsible, is

a. Tricalcium silicate

b. Gypsum

c. Di-calcium silicate

d. Tri calcium aluminate.

8. The commonly used lime in white washing, is

a. fat lime

b. hydraulic lime

c. lime

d. quick lime.

9. Maximum shear stress theory for the failure of a material at the elastic limit, is known

a. Guest's or Trecas' theory

b. St. Venant's theory

c. Rankine's theory

d. Haig's theory

10. Keeping the depth d constant, the width of a cantilever of length l of uniform strength loaded with a uniformly distributed load w varies from zero at the free end and

a.  at the fixed end

b.  at the fixed end

c.  at the fixed end

d.  at the fixed end

11. The moment diagram for a cantilever whose free end is subjected to a bending moment, will be a

a. triangle

b. rectangle

c. parabola

d. cubic parabola.

12. In a three hinged arch, the bending moment will be zero

a. at right hinge only

b. at left hinge only

c. at both right and left hinges

d. at all the three hinges.

13. The pressure rise due to water hammer depends upon

a. the length of pipe

b. time taken to close the valve

c. the elasticity of the pipe material

d. All of above.

14. Most economical section of a circular channel for maximum discharge

a. hydraulic mean depth = 0.286 diameter of circular section

b. wetted perimeter = 2.6 diameter of circular section

c. wetted perimeter = 2.83 depth of water

d. all the above.

15. The discharg formula Q = Cd 2gH x A is used for rectangular

a. small orifices only

b. large orifices only

c. small and large orifices only

d. for all types of orifices.

16. The dimensions MLT-2 refers to

a. specific weight

b. force

c. discharge

d. none of these.

17. The shearing force acting along the slice of a curved surface of slippage, causes the soil to slide

a. down at the centre

b. down at the toe

c. upward at the centre

d. none of these.

18. The minimum water content at which the soil just begins to crumble when rolled into threads 3 mm in diameter, is known

a. liquid limit

b. plastic limit

c. shrinkage limit

d. permeability limit.

19. Pick up the correct statement from the following :

a. To an agriculturist, soil is the substance existing on the earth's surface, which grows and develops plants

b. To a geologist, soil is the material in a relatively thin surface zone within which roots occur, and rest of the crust is termed as rock irrespective of hardness

c. To an engineer, soil is the unaggregated and uncemented deposits of minerals and organic particles covering the earth's crust

d. All the above.

20. Stoke's law does not hold good if the size of particle is smaller than

a. 0.0002 mm

b. 0.002 mm

c. 0.02 mm

d. 0.2 mm

21. For a circular slab carrying a uniformly distributed load, the ratio of the maximum negative to maximum positive radial moment, is

a. 2

b. 3

c. 4

d. 5

22. In the zone of R.C.C. beam where shear stress is less than 5 kg/cm2, nominal reinforcement is provided at a pitch of

a. one-half lever arm of the section

b. one-third lever arm of the section

c. lever arm of the section

d. one and half lever arm of the section.

23. If the diameter of longitudinal bars of a square column is 16 mm, the diameter of lateral ties should not be less than

a. 5 mm

b. 6 mm

c. 8 mm

d. 10 mm

24. Pick up the correct statement from the following:

a. A pile is a slender member which transfers the load through its lower end on a strong strata

b. A pile is a slender member which transfers its load to the surrounding soil

c. A pile is a slender member which transfers its load by friction

d. A pile is a cylindrical body of concrete which transfers the load at a depth greater than its width.

25. Under reamed piles are generally used for

a. factory buildinp

b. transmission linetowers

c. tall structures.

d. All the above.

26. The pile which is provided with a bulb filled with concrete at its lower end, is known as

a. Mac-Arthur pile

b. Raymond pile

c. Franki pile

d. none of these.

27. Construction joints are provided

a. where the member is supported by other member

b. at 18 m apart in huge structures

c. in concrete wall at sill level of windows

d. all the above.

28. Ordinary concrete is not used for concrete grade

a. M 150

b. M 200

c. M 250

d. M 400

29. Growth of population can be conveniently represented by

a. a semi-logarithmic curve

b. a logistic curve

c. a straight line curve

d. all the above.

30. Slow sand filter is used if maximum turbidity of raw water is less than

a. 20 gm/litre

b. 30 gm/litre

c. 40 gm/litre

d. 50 gm/litre.

31. For house drainage minimum gradient is

a. 1 in 60

b. 1 in 80

c. 1 in 10

d. 1 in 400

32. In very first stage of decomposition of the organic matter in sewage

a. nitrites are formed

b. nitrates are formed

c. carbondioxide is formed

d. ammonia is formed.

33. In gravity canals, F.S.L. is

a. always at the ground level

b. always below the ground level

c. generally 4 to 5 metres above the ground level

d. only a few cm above the ground level.

34. The discharge over a trapezoidal crest of a Sarda fall with free over fall conditions, is

a. 0.45 2g . LH3/2 (H/B)1/3

b. 0.45 2g . LH2/3 (H/B)2/3

c. 0.45 2g . LH3/2 (H/B)1/6

d. 0.45 2g . LH1/6 (H/B)3/2

35. In Montague type fall

a. a straight glacis is provided

b. a circular glacis is provided

c. a parabolic glacis is provided

d. no glacis is provided.

36. For the stability of a structure against seepage pressure according to Khosla's creep theory, the critical gradient is

a. 0.25

b. 0.50

c. 0.75

d. 1.00

37. The minimum cross fall of shoulders is kept

a. 1.0%

b. 1.5%

c. 2.5%

d. 3%

38. At a road junction, 7 cross conflict points are severe if

a. both are one-way roads

b. both are two-way roads

c. one is two-way road and other is one-way road

d. none of these.

39. Horizontal curves on highways are provided

a. to break the monotony of driving

b. to discourage the tendency to increase speed

c. to decrease the mental strain on drivers

d. all the above.

40. Alignment of highways in hilly regions, is decided on

a. number of river crossings

b. natural unstable areas

c. saddles or passes

d. all the above.

41. Pick up the correct statement from the following:

a. Cement is paid per 50 kg bag

b. Lime is paid per quintal

c. Brick aggregates is paid per cum

d. All the above.

42. Pick up the correct statement from the following:

a. In order to check up the average depth of excavation, 'Dead mans' are left at the mid-widths of borrow pits

b. The earthwork calculation in excavation is made from the difference in levels obtained with a level

c. The earth work in excavation to form the road embankment includes the formation of correct profile and depositing the soil in layers

d. All the above.

43. The following item of earth work is not measured separately.

a. Site clearence

b. dead men

c. Steps in deep exavaction

d. All the above.

44. Pick up the correct statement in case of water supply.

a. Cutting through walls and floors are included with the item

b. Pipes are classified according to their sizes and quality

c. In laying pipes, the method of jointing and fixing is specifically specified

d. All the above.

45. The time which results in the leasi, possible construction cost of an activity, is known

a. normal time

b. slow time

c. crash time

d. standard time.

46. Pick up the correct statement from the following:

a. Latest occurrence time is denoted by TL

b. Contractual obligation time is denoted by Ts

c. Latest occurrence time is taken as contractual obligation time

d. All the above.

47. The final selection of a construction site, is done by

a. local civil authority representative

b. representative of engineer authority

c. representative of administration

d. all the above.

48. Residential buildings are treated as

a. light construction

b. heavy construction

c. industrial construction

d. private construction.

49. Beaufort scale is used to determine

a. strength of winds

b. direction of winds

c. height of air-crafts

d. none of these.

50. If lift off distance of an craft is 2000 m, the clear way at the end of the runway should not be less than

a. 145 m

b. 152.5 m

c. 162.5 m

d. 172.5 m