***Sub Engineer***

1. The least count of a vernier scale is

a. sum of the smallest divisions of main and vernier scales

b. value of one division of the primary scale divided by total number of divisions of vernier scale

c. value of one division of vernier scale divided by total number of divisions of primary scale

d. none of these.

2. Ranging is an operation of

a. reconnaissance

b. judging the distance

c. determination of slope

d. establishing intermediate points between terminals.

3. The 'fix' of a plane table station with three known points, is bad if the plane table station lies

a. in the great triangle

b. outside the great triangle

c. on the circumference of the circumscribing circle

d. none of these.

4. While rotating the theodolite in the horizontal plane, the bubble of the bubble tube takes up the same position in its tube, it indicates

a. the rotation axis is vertical

b. the trummion axis is horizontal

c. the line of collimation is perpendicular to vertical axis

d. none of the above

5. Pick up the constituent of good brick earth whose excess causes the raw bricks shrink and warp during drying and burning, from the following:

a. Alumina

b. Lime

c. Ironoxide

d. Magnesia.

6. Dry rot

a. reduces the strength of timber

b. reduces the timber to powder

c. shrinks the timber

d. spoils the appearance of timber.

7. Geologically, marble is known as

a. sedimentary rock

b. igneous rock

c. metamorphic rock

d. stratified rock.

8. Veneering means

a. carving out designs on timber planks

b. chemically treating timber planks

c. thick   layer   of superior wood glued to inferior wood

d. thin   layer   of superior   wood glued to inferior wood.

9. The horizontal thrust on the ends of a two hinged semicircular arch of radius R carrying

a. a uniformly distributed load ω per unit run over its right half span, is

b. a uniformly distributed load ω per unit run over its entire span is

c. a distributed load varying from zero at the left end to ω per unit horizontal run at the right end, is

d. all the above.

10. A steel bar 20 mm in diameter simply-supported at its ends over a total span of 40 cm carries a load at its centre. If the maximum stress induced in the bar is limited to N/mm2, the bending strain energy stored in the bar, is

a. 411 N mm

b. 511 N mm

c. 611 Nmm

d. 711 N mm

11. The stress in the wall of a cylinder in a direction normal to its longitudinal axis, due to a force acting along the circumference, is known as

a. longitudinal stress

b. hoop stress

c. circumferential stress

d. ultimate stress.

12. If the beam is supported so that there are only three unknown reactive elements at the supports. These can be determined by using the following fundamental equation of statics

a. ∑V = 0

b. ∑H = 0; ∑H = 0

c. ∑H = 0; ∑V = 0; ∑M = 0

d. ∑M = 0; ∑H = 0

13. A cylinder 3 m in diameter and 4 m long retains water one side as shown in the below figure. If the weight of the cylinder is 2000 kgf, the horizontal reaction at B is

a. 10,000 kgf

b. 15,000 kgf

c. 20,000 kgf

d. 18,000 kgf.

14. When the whole fluid mass rotates either due to fluid pressure or gravity or rotation previously imparted, the motion is known as

a. free vortex

b. forced vortex

c. non-potential vortex

d. rotational vortex.

15. For the two dimensional flow, the stream function is given by ψ = 2xy. The velocity at a point (3, 4) is

a. 8 m/sec

b. 10 m/sec

c. 12 m/sec

d. 15 m/sec.

16. Energy equation is usually applicable to

a. non-uniform flow

b. turbulent flow

c. laminar flow

d. steady flow.

17. A plane inclined at an angle φ to the horizontal at which the soil is expected to stay in the absence of any lateral support, is known as

a. natural slope line

b. repose line

c. the φ line

d. all the above.

18. Pick up the correct statement from the following:

a. The permeability of the coarse-grained soils may be reduced by grouting.

b. The process of injecting fluids (i.e. grouts) into the pores space of the soil, is called grouting.

c. The grouting increases the soil strength.

d. All the above.

19. Soils containing organic matters

a. are of spongy nature

b. swell with decrease of moisture

c. shrink with increase of moisture content

d. none of these.

20. 'Loess' is silty clay formed by the action of

a. water

b. glacier

c. wind

d. gravitational force.

21. Lapped splices in tensile reinforcement are generally not used for bars of size larger than

a. 24 mm diameter

b. 30 mm diameter

c. 36 mm diameter

d. 32 mm diameter

22. The effective span of a simply supported slab, is

a. distance between the centres of the bearings

b. clear distance between the inner faces of the walls plus twice the thickness of the wall

c. clear span plus effective depth of the slab

d. none of these.

23. If the maximum shear stress at the end of a simply supported R.C.C. beam of 16 m effective span is 10 kg/cm2, the length of the beam having nominal reinforcement, is

a. 4 cm

b. 6 m

c. 8 m

d. 10 m

24. The shear reinforcement in R.C.C. is provided to resist

a. vertical shear

b. horizontal shear

c. diagonal compression

d. diagonal tension.

25. To construct a 10 cm thick partition wall, you will prefer

a. English bond

b. Flemish bond

c. Header bond

d. Stretcher bond.

26. Which one of the following piles has a cast iron shoe even after removal of the hollow cylindrical steel casing

a. pedastal pile

b. Franki pile

c. vibro pile

d. both (a) and (d) of the above.

27. The shrinkage of concrete

a. is proportional to water content is the mix

b. is proportional to cement concrete

c. increases with age of concrete

d. all the above.

28. While compacting the concrete by a mechanical vibrator, the slump should not exceed

a. 5.0 cm

b. 7.5 cm

c. 10 cm

d. 15 cm

29. Pick up the correct statement from the following :

a. Even if ports are closed, the wet intake tower will have water filled up to reservoir level

b. The dry intake towers are generally constructed with heavier construction to resist buoyant forces

c. No buoyant force acts on wet intake towers

d. All the above.

30. Most commonly used section in grade aqueducts, is

a. circular

b. rectangular

c. parabolic

d. horse shoe section.

31. Traps

a. are water seals which prevent the entry of foul gases

b. are used to trap the rats entering sewers

c. dissolve the foul gases

d. create syphonic action to increase the quick disposal of sewerage.

32. A nuisance is experienced in diluting water if dilution factor is less than

a. 60

b. 40

c. 20

d. 10

33. The intensity of irrigation means

a. percentage of culturable commanded area to be irrigated annually

b. percentage of gross commanded area to be irrigated annually

c. percentage of the mean of culturable commanded area and the gross commanded area to be irrigated annually

d. total depth of water supplied by the number of waterings.

34. Effective precipitation for a crop may be defined as

a. total precipitation minus the loss due to evaporation

b. total precipitation minus the loss due to infiltration

c. total precipitation during the crop period

d. available water stored in soil within root zone of the crop.

35. Pick up the correct statement from the following

a. The capacity of escapes should not be less than the capacity of the canal at its location

b. The escapes must lead the surplus water to natural drainages

c. The escapes are aligned to take advantage of contours of lower values

d. All the above.

36. If the straight sides of a triangular section of a lined canal with circular bottom of radius R, make an angle θ with horizontal, the perimeter of the canal is

a. 2R(θ + tan θ)

b. R(θ + cos θ)

c. 2R(θ + cos θ)

d. none of these.

37. The basic formula for determination of pavement thickness was first suggested by

a. Spanglar

b. Picket

c. Kelly

d. Goldbeck

38. Hill roads are

a. provided camber along spur curves

b. provided camber along valley curves

c. seldom provided a camber

d. None of these.

39. If d is the thickness of a concrete pavement, the equivalent radius b of resisting section for an interior loading, is

a. b = 0.6 + d2 - 0.675 d

b. b = 1.6 + d2 + 0.675 d

c. b = 1.6 + d2 - 3.675 d

d. b = 1.6 + d2 - 0.675 d

40. The maximum comfortable retardation applied to moving vehicles, is

a. 3.42 m/sec2

b. 4.42 m/sec2

c. 5.56 m/sec2

d. 7.80 m/sec2

41. The most reliable estiamte is

a. Detailed estimate

b. Preliminary estimate

c. Plinth area estimate

d. Cube rate estimate

42. The measurement is made in square metre in case of

a. R.C.C. structure

b. Hollow concrete block wall

c. Concrete fencing posts

d. None of these.

43. Carpet area does not include the area of

a. verandah, corridor and passage

b. bath room and lavatory

c. kitchen and pantry

d. All the above.

44. Pick up the incorrect statement from the following:

a. Dimensions are measured to the nearest 0.01 m

b. Areas are measured to the nearest 0.01 sq.m

c. Cubic contents are measured to the nearest 0.1 cm m

d. Weights are measured to the nearest 0.001 tonnes

45. If TL is the latest allowable event occurrence time, total activity slack(s), is equal to

a. LST-EST

b. LFT-EFT

c. TL-EFT

d. all the above.

46. Pick up the correct statement from the following:

a. The difference of latest occurence time and earliest expected time, is called slack

b. The activities connecting the events having zero slack, lie on the critical path

c. The critical path consumes the maximum time

d. All the above.

47. Railway projects are treated as

a. light construction

b. heavy construction

c. industrial construction

d. none of these.

48. The performance of a specific task in CPM, is known

a. Dummy

b. Event

c. Activity

d. Contract.

49. From the end of an instrumental runway, the approach surface rises outwards

a. 1 in 20

b. 1 in 30

c. 1 in 40

d. 1 in 50

50. The coordinates of the ends of a runway are (5000, 5000) and (8000, 7000). The co-ordinates of an other runway are (4600, 5100) and (7000, 5300). The co-ordinates of the A.R.P. are

a. (6500, 6000)

b. (5800, 5200)

c. (61500, 5600)

d. (8000, 7000)