***Sub Engineer***

1. In optical reading instruments

a. the vertical circle is usually continuous from 0° to 359°

b. the readings increase when the telescope is elevated in the face left position

c. the readings decrease when the telescope is elevated in the face right position

d. all the above.

2. The included angles of a theodolite traverse, are generally measured

a. clockwise from the forward station

b. anti-clockwise from the back station

c. anti-clockwise from the forward station

d. clockwise from the back station.

3. If the whole circle bearing of a line is 270°, its reduced bearing is

a. N 90° W

b. S 90° W

c. W 90°

d. 90° W.

4. The diaphragm of a stadia theodolite is fitted with two additional

a. horizontal hairs

b. vertical hairs

c. horizontal and two vertical hairs

d. none of these.

5. Duco paints are

a. cellulose paints

b. emulsion paints

c. bituminous paints

d. oil paints.

6. Bitumen emulsion is

a. a liquid containing bitumen in suspension

b. a paint

c. used as anti-corrosive paint

d. all the above.

7. Cement paints usually

a. contain 5% to 10% colour pigments

b. are prepared with white cement

c. contain 5% sodium chloride

d. all the above.

8. Linseed oil is used in paints as

a. thinner

b. vehicle

c. base

d. drier.

9. A material is said to be perfectly elastic if

a. it regains its original shape on removal of the load

b. It regains its original shape partially on removal of the load

c. it does not regain its original shape at all

d. none of these.

10. The ratio of crippling loads of a column having both the ends fixed to the column having both the ends hinged, is

a. 1

b. 2

c. 3

d. 4

11. For a stable frame structure, number of members required, is

a. twice the number of joints minus three

b. twice the number of joints minus two

c. twice the number of joints minus one

d. none of these.

12. In rectangular columns (cross-section b x h), the core is a

a. square of length b/2

b. rhombus of length h/2

c. rhombus of diagonals b/3 and h/3

d. none of the these.

13. The Empirical formula  for discharge over large rectangular weirs, is known as

a. Francis formula

b. Bazin formula

c. Rehbook formula

d. Kutter's formula.

14. A cylinder 3 m in diameter and 4 m long retains water one side as shown in the below figure. If the weight of the cylinder is 2000 kgf, the vertical reaction at A is

a. 14,137 kgf

b. 5,863 kgf

c. 20,000 kgf

d. 18,000 kgf.

15. The length of hydraulic jump is roughly

a. 2 to 3 times its height

b. 3 to 5 times its height

c. 5 to 7 times its height

d. None of these.

16. An independent mass of a fluid does not posses

a. elevation energy

b. kinetic energy

c. pressure energy

d. none of these.

17. The minimum water content at which the soil just begins to crumble when rolled into threads 3 mm in diameter, is known

a. liquid limit

b. plastic limit

c. shrinkage limit

d. permeability limit.

18. The vane shear test is used for the in-situ determination of the undrained strength of the intact fully saturated

a. sands

b. clays

c. gravels

d. highly organic soils.

19. If the back fill is having a uniform surcharge of intensity q per unit area, the lateral pressure will be

a. q times the lateral pressure within the surface

b. 1/q times the lateral pressure within the surface

c. equal to a fill of height Z equal to q/r, where r is the density of the backfill

d. none of these.

20. Pick up the correct statement from the following:

a. Failure plane carries maximum shear stress

b. Failure plane does not carry maximum shear stress

c. Failure plane carries shear stress equal to maximum shear stress

d. None of these.

21. An R.C.C. column is treated as short column if its slenderness ratio is less than

a. 35

b. 40

c. 50

d. 60

22. If T and R are tread and rise respectively of a stair, then

a. 2R + T = 60

b. R + 2T = 60

c. 2R + T = 30

d. R + 2T= 30

23. The maximum ratio of span to depth of a slab simply supported and spanning in two directions, is

a. 30

b. 35

c. 40

d. 15

24. The neutral axis of a T-beam exists

a. within the flange

b. at the bottom edge of the slab

c. below the slab

d. all the above.

25. For each storey of a building, the depth of exploration should be

a. 1 metre

b. 2 metres

c. 3 metres

d. 4 metres.

26. Pick up the correct statement from the following :

a. Double-flemish bond is suitable for brick masonry to give uniform face appearance

b. The double-flemish bond is used for the construction of single brick residential building

c. The stretcher bond is used for the construction of half brick masonry brick

d. all the above.

27. The process of proper and accurate measurement of concrete ingredients for uniformity of proportion, is known

a. curing

b. mixing

c. batching

d. none of these.

28. High temperature

a. increases the strength of concrete

b. decreases the strength of concrete

c. has no effect on the strength of concrete

d. none of these.

29. Normal values of overflow rate for plain sedimentation tank, is

a. 250 to 500 litres/hr/m2

b. 500 to 750 litres/hr/m2

c. 750 to 1000 litres/hr/m2

d. 1000 to 1250 litres/hr/m2.

30. The least thickness of class B cast iron (spun) pipe, is

a. 7.2 mm

b. 7.9 mm

c. 8.6 mm

d. 10 mm.

31. Pick up the correct statement from the following :

a. Medium sewers are cleaned by cane rodding

b. Medium sewers may be cleaned by pills

c. Large sewers are cleaned by removing the deposited materials

d. All the above.

32. The minimum recommended diameter of sewers, is

a. 5 cm

b. 10 cm

c. 15 cm

d. 20 cm.

33. The sensitivity of a rigid module, is

a. 1.50

b. 1.00

c. 0.50

d. zero

34. The ratio of the rate of change of discharge of an outlet and parent channel, is known as

a. efficiency

b. sensitivity

c. flexibility

d. modular limit.

35. For a given discharge in a channel, Blench curves give the relationship between the loss of head (HL) and

a. specific energy up-stream

b. specific energy down-stream

c. critical depth of water down-stream

d. depth of water down-stream

36. In a concrete canal the approximate permissible velocity of water should not exceed

a. 1 m/sec

b. 1.5 m/sec

c. 2 m/sec

d. 2.5 m/sec

37. To ensure that bullock carts may not overturn on curves, the maximum value of super-elevation, recommended by I.R.C., is

a. 1 in 12

b. 1 in 15

c. 1 in 20

d. 1 in 25

38. To prevent compressive stresses in a rigid concrete pavement, the joint prvoided, is

a. expansion joint

b. contraction joint

c. hinged joint

d. all the above.

39. If the cross slope of a country is 25% to 60%, the terrain is classified as

a. plain

b. rolling

c. steep

d. mountainous.

40. If no super elevation is provided on a road along curves, pot holes may develop at

a. inner edge of the road

b. outer edge of the road

c. centre of the road

d. no where on the road.

41. If B is the width of formation, d is the height of the embankment, side slope S : 1, for a highway with no transverse slope, the area of cross-section is

a. B + d + Sd

b. Bd + Sd2

c. B x d - Sd1/2

d. (Bd + Sd2)

42. The reduced levels of points, 30 metres apart along the longitudinal section of a road portion between chainages 5 and 9 are shown in the given figure. If there is a uniform up-gradient of the road 120 in 1, the chainage of the point with no filling or cutting is

a. (6 + 15) chains

b. (6 + 12) chains

c. (6 + 18) chains

d. None of these.

43. Carpet area does not include the area of

a. verandah, corridor and passage

b. bath room and lavatory

c. kitchen and pantry

d. All the above.

44. The cost of the earthwork in excavation for the surface drain of cross-section shown in the given figure for a total length of 5 metres @ Rs. 450% cum, is

a. Rs. 400

b. Rs. 425

c. Rs. 450

d. Rs. 500

45. Pick up the incorrect statement from the following:

a. The beginning and end of a job, are called events

b. The activity which consumes maximum time, is called a node

c. Logically and sequentially connected activities and events form a network

d. None of these.

46. Frederick W. Taylor introduced a system of working known as

a. line organisation

b. line and staff organisation

c. functional organisation

d. effective organisation.

47. The time by which activity completion time can be delayed without affecting the start of succeeding activities, is known as

a. duration

b. total flat

c. free float

d. interfering float.

48. The first method invented for planning projects, was

a. Bar chart method

b. Milestone chart

c. Critical path method (CPM)

d. Programme Evaluation and Review Technique (PERT)

49. For the proposed air port, the survey project provides

a. topograhic plan

b. grading plan

c. drainage plan

d. all the above.

50. According to I.C.A.O. the recommended length of air ports is decided on

a. sea level elevation

b. standard sea level temperature (15°C)

c. effective gradient percentage

d. all the above.