***Sub Engineer***

1. The probable error of the adjusted bearing at the middle is

a.  rn

b.  rn

c.  rn

d.  rn.

2. In setting up a plane table at any station

a. levelling is done first

b. centering is done first

c. both levelling and centering are done simultaneously

d. orientation is done first.

3. Metric chains are generally available in

a. 10 m and 20 m length

b. 15 m and 20 m length

c. 20 m and 30 m length

d. 25 m and 100 m length

4. Pick up the correct statement from the following :

a. in Galileo's telescope, the rays from the object get refracted at the objective and are intercepted by the eyepiece before a real image is formed

b. a line passing through the optical centre of the objective traversing through the eyepiece, is called line of sight

c. the line of sight which passes through the intersection of cross-lines marked on a diaphragm fixed in front of the eyepiece in a plane at right-angles to the axis, is called the line of collimation

d. all the above.

5. The minimum compressive strength of 1st class bricks should be

a. 90 kg/cm2

b. 100 kg/cm2

c. 120 kg/cm2

d. 130 kg/cm2.

6. Wrought iron contains carbon about

a. 1.5% to 5.5%

b. 0.5% to 1.75%

c. 0.1% to 0.25%

d. none to these.

7. Chlorite, a green colour mineral is mainly derived from the decomposition of

a. augite

b. biotite

c. horn blende

d. All of these.

8. The compound of Portland cement which reacts immediately with water and also sets first is

a. Tri-calcium silicate

b. Di-calcium silicate

c. Tri-calcium aluminate

d. Tetra calcium alumino ferrite.

9. The assumption in the theory of bending of beams, is :

a. material is isotropic

b. Young's modulus is same in tension as well as in compression

c. each layer is independent to expand or to contract

d. all the above.

10. The force in BF of the truss shown in given figure, is

a. 4t compression

b. 4.5t tension

c. 4.5t compression

d. zero.

11. The law which states, "within elastic limits strain produced is proportional to the stress producing it", is known as

a. Stress law

b. Hooke's law

c. Poisson's law

d. none of these.

12. When a rectangular beam is loaded transversely, the maximum compressive stress develops on

a. bottom fibre

b. top fibre

c. neutral axis

d. every cross-section.

13. In a venturimeter, the divergent cone is kept

a. shorter than convergent cone

b. equal to covergent cone

c. longer than convergent cone

d. none of these.

14. A jet projected at an angle of 45θ, 40 m from the foot of a vertical column, just reaches the top of the column. The height of the column is

a. 20 m

b. 30 m

c. 40 m

d. 60 m.

15. The value of momentum correction factor (β) for a laminar flow through a circular pipe, is

a. 2/3

b. 3/4

c. 4/3

d. 3/2

16. A pipe of 0.1 m2 cross sectional area suddenly enlarges to 0.3 m2 cross-sectional area. If the discharge of the pipe is 0.3 m3 /sec, the head loss is

a. 2/g m of water

b. g/2 m of water

c. 1g m of water

d. g m of water.

17. The passive earth pressure of a soil, is proportional to (where φ is the angle of internal friction of the soil.)

a. tan (45° - φ)

b. tan (45° + φ)

c. tan2 (45° - φ

d. tan2 (45° + φ)

18. The ratio of volume of air voids to the volume of total voids, is known as

a. air content

b. percentage air voids

c. percentage voids

d. porosity.

19. Pick up the correct statement from the following:

a. In hydrometer method, weight Wd per ml of suspension is found directly

b. In pipette analysis, weight Wd per ml of suspension is found indirectly

c. In pipette analysis, weight Wd per ml of suspension is found directly

d. None of these.

20. A moist soil sample weighing 108 g has a volume of 60 cc. If water content is 25% and value of G = 2.52, the void ratio is

a. 0.55

b. 0.65

c. 0.75

d. 0.80

21. An R.C.C. column is treated as short column if its slenderness ratio is less than

a. 35

b. 40

c. 50

d. 60

22. An under-reinforced section means

a. Steel is provided at the under side only

b. Steel provided is insufficient

c. Steel provided on one face only

d. Steel will yield first.

23. The zone in which transverse bending is likely to occur may be obtained by drawing a line from the faces of the column making an angle θ° with horizontal where θ° is

a. 30°

b. 45°

c. 60°

d. none of these.

24. If the diameter of the main reinforcement in a slab is 16 mm, the concrete cover to main bars is

a. 13 mm

b. 14 mm

c. 15 mm

d. 16 mm

25. The arrangement made to support an unsafe structure temporarily, is known as

a. shoring

b. scaffolding

c. underpinning

d. jacking

26. For different layers of cement concrete floor. Pick up the incorrect statement from the following :

a. The lowest layer consists of consolidated ground

b. A 10 cm thick clean sand is laid on consolidated ground

c. A 10 cm lime concrete (1 : 4 : 8) is laid on clean sand

d. A 10 cm thick cement concrete (1 : 2 : 4) is laid on top layer.

27. Curing a concrete for long period ensures better

a. strength

b. water resistance

c. water tightness and durability

d. all the above.

28. Concrete mainly consists of

a. aggregates

b. admixture

c. water

d. all the above.

29. Rapid gravity filter can only remove turbidity of water upto

a. 25 to 30 gm/litre

b. 30 to 35 cm/litre

c. 35 to 40 gm/litre

d. 50 gm/litre.

30. Water to the consumers may be supplied from

a. infiltration galleries connected to sump well

b. infiltration well dug out on the banks of rivers

c. ranney wells sunk to the water level

d. all the above.

31. The moisture content of a sludge is reduced from 90% to 80% in a sludge digestion tank. The percentage decrease in the volume of sludge, is

a. 25%

b. 50%

c. 10%

d. 5%.

32. If D.O. concentration falls down to zero in any natural drainage, it indicates the zone of

a. active decomposition

b. recovery

c. cleaner water

d. none of these.

33. For cereal crops the most commonly adopted method of irrigation, is

a. free flowing method

b. check method

c. furrow method

d. sprinkling method.

34. If H and d are the water depth and drop in the bed level at a Sarda fall, the width B of the trapezoidal crest, is given by

a. B = 0.22 H + d

b. B = 0.33 H + d

c. B = 0.44 H + d

d. B = 0.55 H + d

35. F.S.L. of a canal at its head with respect to parent channel is kept

a. at the same level

b. 15 cm lower

c. 15 cm higher

d. none of these.

36. The depth of the crest of a scouring sluice below the crest of a head regulator, is generally kept

a. 1.20 m

b. 2.20 m

c. 3.20 m

d. 4.90 m

37. The usual width of parapet walls along Highways in hilly region, is

a. 60 cm

b. 70 cm

c. 80 cm

d. 100 cm

38. Bottom-most component of a flexible pavement, is

a. subgrade

b. sub-base

c. base

d. base course.

39. Traffic census is carried out for

a. road parking study

b. traffic volume study

c. origin and destination study

d. all the above.

40. Pick up the incorrect statement from the following. The width of the right-of-way is decided so as to accommodate

a. side slopes

b. horizontal curve

c. vertical curve

d. provision of future widening.

41. Pick up the correct statement from the following:

a. The actual expenditure involved to complete a work including incidental, establishment and travelling charges, is called actual cost

b. The formal acceptance by the administrative department for incurring an expenditure on the work, is called administrative approval

c. The order of a competent authority sanctioning a properly detailed estimate of the cost of a work of construction or repair is called technical sanction

d. All the above.

42. Pick up the correct statement regarding the centre line method of estimating a building

a. Product of the centre line of the walls and area of cross-section of any item, gives total quantity of the item

b. The centre line is worked out separately for different sections of walls of a building

c. The centre line length is reduced by half the layer of main wall joining the partition wall

d. All the above.

43. For 100 sq. m cement concrete (1 : 2: 4) 4 cm thick floor, the quantity of cement required, is

a. 0.90 m3

b. 0.94 m3

c. 0.98 m3

d. 1.00 m3

44. The floor area includes the area of the balcony up to

a. 100%

b. 75%

c. 50%

d. 25%

45. Pick up the incorrect statement from the following:

a. The tail of the arrow indicates the start of the activity

b. The head of the arrow indicates the end of the activity

c. The arrows are drawn to scale from left to right

d. Each activity consumes a given time.

46. The Overall in-charge of an organisation at the site responsible for the execution of the works, is

a. Engineer

b. Junior Engineer

c. Sub overseer

d. Assistant Engineer.

47. The main principle of an organisation, is

a. cohegency

b. effective control at all levels

c. delegation of authority

d. all the above.

48. A CPM family includes

a. CPP (Critical Path Plotted)

b. MCE (Minimum Cost Expenditure)

c. CPS (Critical Path Scheduling)

d. All the above.

49. The landing and take off of the air craft is made against the direction of wind. In no case the centre line of the runway should make an angle with the wind direction exceeding

a. 10°

b. 20°

c. 30°

d. 40°

50. Pick up the correct statement from the following:

a. The speed of the aircraft relative to the ground, is called cruising speed

b. The speed of the aircraft relative to wind, is called air speed

c. When wind is blowing the direction of the flight, air speed is less than cruising speed

d. All the above.