***Sub Engineer***

1. The slope correction may be ignored if

a. the slope of the ground is less than 3°

b. to slope of the ground is say 1 in 19

c. both (a) and (b)

d. neither (a) nor (b)

2. The additional lines which are measured to show the correctness of the chain surveying are called:

a. check clines

b. proof lines

c. tie lines

d. all of these.

3. The imaginary line passing through the intersection of cross hairs and the optical centre of the objective, is known as

a. line of sight

b. line of collimation

c. axis of the telescope

d. none of these.

4. The slope correction for a 3° slope for a length of 100 m, is

a. -0.11 m

b. -0.12 m

c. -1.87 m

d. -0.137 m

5. The filler used in plastic bitumen, is

a. talc powder

b. asbestos powder

c. plastic powder

d. none of these.

6. The normal curing period for lime mortar, is:

a. 3 days

b. 7 days

c. 10 days

d. 14 days.

7. Resins are

a. soluble in spirit

b. used in varnishes

c. left behind on evaporation of oil

d. all the above.

8. Pick up the correct statement from the following:

a. Horn blende mineral is brittle

b. Muscovite is also known as white mica and potashmica

c. Biotite is also known as blackmica

d. All the above.

9. The force in CD of the truss shown in given figure, is

a. 3t tension

b. zero

c. 1.5t compression

d. 1.5t tension

10. A three hinged arch is generally hinged at its supports and

a. at one quarter span

b. at the crown

c. any where in the rib

d. none of these.

11. In a continuous bending moment curve the point where it changes sign, is called

a. point of inflexion

b. point of contraflexture

c. point of virtual hinge

d. all the above.

12. The range within which a load can be applied on a rectangular column, to avoid any tensile stress, is

a. one-third of the base

b. one-fourth of the base

c. one-fifth of the base

d. one sixth of the base on either side of centroid.

13. The ratio of maximum velocity to average velocity of viscous fluid through a circular pipe is

a. 0.5

b. 0.75

c. 1.25

d. 2.00

14. If Cv, Cc, Cd and Cr are the hydraulic coefficients of an orifice, then

a. Cd = Cc . Cv

b. Cr = 1 + Cv2/Cd

c. Cv = Cc + Cd

d. Cc = Cv/Cd

15. In case of laminar flow through a circular pipe,

a. momentum correction factor is 1.33

b. energy correction factor is 2.00

c. both (a) and (b)

d. Neither (a) nor (b).

16. A floating body attains stable equilibrium if its metacentre is

a. at the centroid

b. above the centroid

c. below the centroid

d. anywhere.

17. The ratio of emax and emin of silty sand, is

a. 5

b. 3.0

c. 3.5

d. 4.0

18. For determing the specific gravity of soil solids, using a pycnometer of 500 cc., the following data is available : Weight of dry empty pycnometer = 125 g Weight of dried soil and pycnometer = 500 g Weight of dried soil and distilled = 850 g water filled in pycnometer up to top The specific gravity of soil soilds, is

a. 2.0

b. 2.25

c. 2.50

d. 2.75

19. The ratio of the weight of given volume of soil solids to the weight of an equal volume of distilled water at the given temperature, is known

a. porosity

b. specific gravity

c. void ratio

d. water content.

20. Pick up the correct statement from the following :

a. To an agriculturist, soil is the substance existing on the earth's surface, which grows and develops plants

b. To a geologist, soil is the material in a relatively thin surface zone within which roots occur, and rest of the crust is termed as rock irrespective of hardness

c. To an engineer, soil is the unaggregated and uncemented deposits of minerals and organic particles covering the earth's crust

d. All the above.

21. The maximum shear stress (qmax) in a rectangular beam is

a. 1.50 times the average

b. 1.75 times the average

c. 2.0 times the average

d. 2.5 times the average.

22. The stem of a cantilever retaining wall which retains earth level with top is 6 m. If the angle of repose and weight of the soil per cubic metre are 30° and 2000 kg respectively, the effective width of the stem at the bottom, is

a. 52.5

b. 53.5

c. 54.5

d. 55.5

23. The percentage of minimum reinforcement of the gross sectional area in slabs, is

a. 0.12%

b. 0.15%

c. 0.18%

d. 0.20%

24. The length of the straight portion of a bar beyond the end of the hook, should be at least

a. thrice the diameter

b. four times the diameter

c. five times the diameter

d. seven times the diameter.

25. Open test pit is only suitable upto a depth of

a. 2 metres

b. 2.5 metres

c. 3 metres

d. none of the above.

26. The vertical side member of a shutter frame, is known

a. style

b. reveal

c. mullion

d. post.

27. The grade of concrete M 150 means that compressive strength of a 15 cm cube after 28 days, is

a. 150 kg/cm2

b. 200 kg/cm2

c. 250 kg/cm2

d. 300 kg/cm2

28. Pick up the incorrect statement applicable to the field test of good cement.

a. The colour of the cement is bluish

b. A handful of cement thrown into a bucket of water should sink immediately

c. By rubbing cement in between fingers, one should feed rough

d. All the above.

29. Disappearance of pink colour of water of a well due of KMnO4 indicates that water contains

a. alkalinity

b. turbidity

c. organic matter

d. all the above.

30. The duration of contact of chlorine with water before it is served to the first consumer, should be at least

a. 15 to 20 minutes

b. 20 to 30 minutes

c. 30 to 40 minutes

d. one hour.

31. It is customary to design a sewer for D.W.F. on the basis of

a. average demand

b. twice the average demand

c. thrice the average demand

d. four times the average demand.

32. Imhoff cone is used to determine

a. settlable solids

b. suspended solids

c. dissolved solids

d. none of these.

33. In a canal syphon, the flow is

a. under atmospheric pressure

b. pipe flow

c. with critical velocity

d. under negative pressure.

34. In a concrete canal the approximate permissible velocity of water should not exceed

a. 1 m/sec

b. 1.5 m/sec

c. 2 m/sec

d. 2.5 m/sec

35. The state of the soil when plants fail to extract sufficient water for their requirements, is

a. maximum saturated point

b. permanent wilting point

c. ultimate utilisation point

d. none of these.

36. The ratio of the discharge over a trapezoidal crest to a rectangular crest of Sarda falls of identical parameters, is

a. 1.084

b. 1.074

c. 1.064

d. 1.054

37. Traffic census is carried out for

a. road parking study

b. traffic volume study

c. origin and destination study

d. all the above.

38. Maximum number of passenger cars that can pass a given point on a road during one hour under the most ideal road way and traffic conditions, is known as

a. traffic density

b. basic capacity of traffic lane

c. possible capacity of traffic lane

d. all the above.

39. For a properly designed vehicle, the resistance generally ignored, is

a. rolling resistance

b. grade resistance

c. axle resistance

d. none of these.

40. The traffic manoeuvre means

a. diverging

b. merging

c. crossing

d. all the above.

41. Pick up the incorrect statement regarding a master trap from the following :

a. It is provided a cleaning eye at the top of the trap

b. The height of fresh air inlet pipe fixed vertically with wall is 3 m

c. The mica flap valve which opens inwards only, is fitted at the top of the inlet pipe

d. The water seal is less than that of ordinary traps.

42. Pick up the correct statement from the following:

a. Plastering is measured in sq.m

b. Glazing is measured in sq.m

c. Striking is measured in sq.m

d. All the above.

43. Pick up the correct statement from the following:

a. Additional supervising staff engaged at work site, is called work charged establishment

b. Detailed specifications specify qualities, quantities and the proportions of materials to be used for a particular item

c. The cost per unit at which the article can be procured, from the open market at a given time, is called 'market rate'

d. All the above.

44. The expected out turn of cement concrete 1 : 2 : 4 per mason per day is

a. 1.5 m3

b. 2.5 m3

c. 3.5 m3

d. 5.0 m3

45. In CPM analysis,

a. uncertainties are not allowed

b. activities are represented by arrows

c. beginning and end of an activity, are denoted by nodes

d. all the above.

46. Various activities of a project, are shown on bar charts by

a. vertical lines

b. horizontal lines

c. dots

d. crosses.

47. A critical ratio scheduling

a. determines the status of each activity

b. adjusts automatically changes in activity progress

c. is a dynamic system

d. none of these.

48. The time by which activity completion time can be delayed without affecting the start of succeeding activities, is known as

a. duration

b. total flat

c. free float

d. interfering float.

49. Pick up the component not applicable to aeroplanes

a. engines

b. air screw

c. fuse lage

d. none of these.

50. The bearing of the longest line of a wind rose is S 45° E, the bearing of the runway will be numbered

a. 315°

b. 13

c. 31

d. both (c) and (d)