***Sub Engineer***

1. If the long chord and tangent length of a circular curve of radius R are equal the angle of deflection, is

a. 60°

b. 90°

c. 120°

d. 150°.

2. There are two stations A and B. Which of the following statements is correct :

a. the fore bearing of AB is AB

b. the back bearing of AB is BA

c. the fore and back bearings of AB differ by 180°

d. all the above.

3. If L is in kilometres, the curvature correction is

a. 58.2 L2 mm

b. 64.8 L2 mm

c. 74.8 L2 mm

d. 78.4 L2 mm.

4. If a linear traverse follows a sharp curve round a large lake where it is difficult to have long legs, the accuracy of the traverse may be improved by

a. taking short legs

b. making repeated observations of angular and linear measurements

c. making a subsidiary traverse to determine the length of a long leg

d. all the above.

5. The commonly used lime in white washing, is

a. fat lime

b. hydraulic lime

c. lime

d. quick lime.

6. Wrought iron is manufactured from pig iron by

a. puddling

b. shingling

c. rolling

d. all the above.

7. Iron ore may contain

a. silicon

b. sulphur

c. phosphorus and maganese

d. all the above.

8. If water required for 1 bag of cement is 30 litres, the water cement ratio is :

a. 0.40

b. 0.50

c. 0.60

d. None of these.

9. Keeping the depth d constant, the width of a cantilever of length l of uniform strength loaded with a uniformly distributed load w varies from zero at the free end and

a.  at the fixed end

b.  at the fixed end

c.  at the fixed end

d.  at the fixed end

10. The point of contraflexure is the point where

a. B.M. changes sign

b. B.M. is maximum

c. B.M. is minimum

d. S.F. is zero.

11. A cantilever carries is uniformly distributed load W over its whole length and a force W acts at its free end upward. The net deflection of the free end will be

a. zero

b.  upward

c.  downward

d. none of these.

12. The stress necessary to initiate yielding, is considerably

a. more than that necessary to continue it

b. less than that necessary to continue it

c. more than that necessary to stop it

d. less than that necessary to stop it.

13. Capillary rise of water is

a. directly proportional to surface tension

b. inversely proportional to water density

c. inversely proportional to diameter of the tube

d. All of these.

14. If the total head of the nozzle of a pipe is 37.5 m and discharge is 1 cumec, the power generated is

a. 400 H.P.

b. 450 H.P.

c. 500 H.P.

d. 550 H.P.

15. In order to avoid capillary correction, the minimum diameter of a manometer used for measuring pressure, should be

a. 4 mm

b. 6 mm

c. 8 mm

d. 10 mm.

16. Flow in pipes is turbulent if Reynold number is

a. less than 2100

b. more than 3000

c. between 2100 and 3000

d. none of these.

17. Pick up the correct statement from the following:

a. At low value of water content most soils tend to be stiff.

b. At high water content, the dry density decreases with an increase of water content.

c. An airless soil will have maximum possible value of dry density for the given water content

d. All the above.

18. The shear strength in plastic undrained clay, is due to

a. inter-granular friction

b. internal friction

c. cohesion

d. none of these.

19. A coarse-grained soil has a voids ratio 0.75, and specific gravity as 2.75. The critical gradient at which quick sand condition occurs, is

a. 0.25

b. 0.50

c. 0.75

d. 1.00

20. The ratio of emax and emin of silty sand, is

a. 5

b. 3.0

c. 3.5

d. 4.0

21. In favourable circumstances a 15 cm concrete cube after 28 days, attains a maximum crushing strength

a. 200 kg/cm2

b. 300 kg/cm2

c. 400 kg/cm2

d. 500 kg/cm2

22. The thickness of the flange of a Tee beam of a ribbed slab is assumed as

a. width of the rib

b. depth of the rib

c. thickness of the concrete topping 0d) half the thickness of the rib

d. twice the width of the rib.

23. On an absolutely rigid foundation base, the pressure will

a. be more at the edges of the foundation

b. be uniform

c. not be uniform

d. be zero at the centre of the foundation.

24. For a continuous floor slab supported on beams, the ratio of end span length and intermediate span length, is

a. 0.6

b. 0.7

c. 0.8

d. 0.9

25. The Auger boring method is  not suitable for

a. cemented soil

b. vary soft soil

c. fully saturated cohesionless soils

d. all the above.

26. For constructing a terrazo floor. Pick up the incorrect statement from the following :

a. a 32 mm thick layer of cement concrete (1 : 2 : 4) is laid on the base course and the surface is made smooth by trowelling

b. glass strips are driven into the layer according to the pattern required

c. after final grinding is over, oxalic acid mixed with water is spread over and rubbed hard with soft material

d. none of these.

27. The top diameter, bottom diameter and the height of a slump mould are :

a. 10 cm, 20 cm, 30 cm

b. 10 cm, 30 cm, 20 cm

c. 20 cm, 10 cm, 30 cm

d. 20 cm, 30 cm, 10 cm

28. Expansion joints are provided if the length of concrete structures exceeds

a. 15 m

b. 15 m

c. 35 m

d. 45 m

29. For plain chlorination of water, the quantity of chlorine used, is

a. 0.2 mg/litre

b. 0.3 mg/litre

c. 0.4 mg/litre

d. 0.5 mg/litre.

30. The average domestic consumption under normal conditions in an Indian city per day per person, is

a. 115 litres

b. 125 litres

c. 135 litres

d. 150 litres.

31. In SI units the power of sound is represented in

a. kgs

b. joules

c. neutons

d. watts.

32. Setting out the alignments of sewers may start from

a. city

b. out-fall

c. tail end

d. any point.

33. Pick up the incorrect statement from the following. Culturable commanded area is the gross area of an irrigation canal system less

a. populated area

b. alkaline area

c. forest area

d. fallow land.

34. The useful moisture of soil, is equal to its

a. field capacity

b. saturation capacity

c. moisture content at permanent wilting point

d. difference between filed capacity and permanent wilting point within the root zone of plants.

35. Effective precipitation for a crop may be defined as

a. total precipitation minus the loss due to evaporation

b. total precipitation minus the loss due to infiltration

c. total precipitation during the crop period

d. available water stored in soil within root zone of the crop.

36. The optimum depth of kor watering for a rice crop, is

a. 19.0 cm

b. 17.5 cm

c. 13.5 cm

d. 12.0 cm

37. If no super elevation is provided on a road along curves, pot holes may develop at

a. inner edge of the road

b. outer edge of the road

c. centre of the road

d. no where on the road.

38. Longitudinal pavement lines marked broken in white paint

a. are for the guidance of the drivers

b. are not to be crossed over

c. may be crossed over at the discretion of the driver

d. all the above.

39. The minimum vertical clearance of overhanging cliffs or any other structure above the surface of a hill road, should be

a. 4 m

b. 5 m

c. 6 m

d. 4.5 m

40. If the rate of change of the super-elevation along a curved portion of a 7 metre wide road is 1 in 150 and the maximum super-elevation allowed is 1 in 15, the maximum length of the transition curve to be provided at either end, is

a. 70 m

b. 75 m

c. 80 m

d. 90 m

41. The detention period in a septic tank is assumed

a. 20 minutes

b. 25 minutes

c. 30 minutes

d. 40 minutes

42. The assumption on which the trapezoidal formula for volumes is based, is

a. The end sections are parallel planes

b. The mid-area of a pyramid is half the average area of the ends

c. The volume of the prismoidal is over-es-timted and hence a prismoidal correction is applied

d. All the above.

43. The expected out turn for earth work in excavation in ordinary soil per mazdoor per day is

a. 1.00 cubic meter

b. 2.00 cubic meter

c. 3.00 cubic meter

d. 4.00 cubic meter

44. The slope of the oultet of 'P trap' below the horizontal is kept

a. 8°

b. 10°

c. 12°

d. 14°

45. A Milestone chart

a. shows the interdependencies of various jobs

b. depicts the delay of jobs, if any

c. points outgoing ahead of schedule of jobs, if any

d. none of these.

46. Construction team means

a. an architect

b. an owner

c. a contractor

d. all the above.

47. The time by which activity completion time can be delayed without affecting the start of succeeding activities, is known as

a. duration

b. total flat

c. free float

d. interfering float.

48. A construction schedule is prepared after collecting

a. output of labour

b. output of machinery

c. quantity of various items

d. all the above.

49. Airport elevation is the reduced level above M.S.L. of

a. control tower

b. highest point of the landing area

c. lowest point of the landing area

d. none of these.

50. To cope up high temperature of 196°C, the taxi ways and aprons are constructed with

a. rubberised tar concrete

b. plain concrete

c. epoxy asphalt concrete

d. all the above.