***Sub Engineer***

1. Volume of the earth work may be calculated by

a. end areas

b. Prismoidal formula

c. Trapezoidal

d. all the above.

2. The operation of resection involves the following stepsrough orientation of the plane tablethe three lines form a triangle of errordrawing lines back through the three control pointsselect a point in the triangle of error such that each ray is equally rotated either clockwise or anti clockwisethe points obtained by three rays is the correct location. The correct sequence is

a. 1, 3, 2, 4, 5

b. 1, 2, 3, 4, 5

c. 1, 4, 3, 2, 5

d. 1, 4, 2, 3, 5

3. The bearing of lines OA and OB are 16° 10' and 332° 18', the value of the included angle BOA is

a. 316° 10'

b. 158° 28'

c. 348° 08'

d. 43°  52'

4. The distances AC and BC are measured from two fixed points A and B whose distance AB is known. The point C is plotted by intersection. This method is generally adopted in

a. chain surveying

b. traverse method of surveys

c. triangulation

d. none of these.

5. To give a brilliant finish, the type of varnish used, is

a. spirit varnish

b. turpentine varnish

c. oil varnish

d. none of these.

6. Glazing of clay products, is done

a. to improve their appearance

b. to protect them from atmospheric effect

c. to protect them from corrosive action

d. all the above.

7. Acrylic sheets

a. are generally unaffected by most household detergents

b. possess the light transmission rate of 93%

c. are available in various shapes

d. all the above.

8. In stone masonry, stones (stratified rocks) are so placed that the direction of pressure to the plane of bedding is

a. right angles

b. 45°

c. 60°

d. parallel

9. The ratio of lateral strain to axial strain of a homogeneous material, is known

a. Yield ratio

b. Hooke's ratio

c. Poisson's ratio

d. Plastic ratio.

10. The horizontal thrust on the ends of a two hinged semicircular arch of radius R carrying

a. a uniformly distributed load ω per unit run over its right half span, is

b. a uniformly distributed load ω per unit run over its entire span is

c. a distributed load varying from zero at the left end to ω per unit horizontal run at the right end, is

d. all the above.

11. A cantilever carries is uniformly distributed load W over its whole length and a force W acts at its free end upward. The net deflection of the free end will be

a. zero

b.  upward

c.  downward

d. none of these.

12. For keeping the stress wholly compressive the load may be applied on a circular column anywhere within a concentric circle of diameter

a. d/3

b. d/4

c. d/8

d. d/10

13. For solving the problems is hydraulic engineering, the velocity used is

a. velocity at the centre of pipe section

b. average velocity of flow over a section

c. mean of the velocities at the centre and that along the pipe surface

d. none of these.

14. An open container filled with water is moved vertically downward with a uniform linear acceleration. The pressure at its bottom will be

a. greater than static pressure

b. equal to static pressure

c. lesser than static pressure

d. none of these.

15. The notch angle for maximum discharge over a triangular notch, is

a. 30°

b. 60°

c. 90°

d. 120°

16. An error of 1% in measuring the head of water over the crest of a triangular notch, produces an error in the discharge which is equal to

a. 1.25%

b. 1.5%

c. 2.0%.

d. 2.5%

17. Factor of safety against sliding of a slope, is the ratio of

a. actual cohesion to that required to maintain stability of slope

b. shear strength to shear stress along the surface

c. neither (a) nor (b)

d. both (a) and (b).

18. The water content in a soil sample when it continues to loose weight without loosing the volume, is called

a. Shrinkage limit

b. Plastic limit

c. liquid limit

d. semi-solid limit.

19. For testing a saturated clay for shear strength, the test recommended, is

a. direct shear test

b. triaxial compression test

c. unconfined compression test

d. all the above.

20. In active state of plastic equilibrium in a non cohesive soil with horizontal ground surface

a. major principal stress is horizontal

b. minor principal stress is vertical

c. major principal stress is vertical

d. minor and major principal stresses are equally inclined to horizontal.

21. A per IS : 1343, total shrinkage for a pretensioned beam, is

a. 3.0 x 10-3

b. 3.0 x 10-4

c. 3.0 x 10-5

d. 3.5 x 10-5

22. Pick up the incorrect statement from the following: Tensile reinforcement bars of a rectangular beam

a. are curtailed if not required to resist the bending moment

b. are bent up at suitable places to serve as shear reinforcement

c. are bent down at suitable places to serve as shear reinforcement

d. are maintained at bottom to provide at least local bond stress.

23. If H is the overall height of a retaining wall retaining a surcharge, the width of the base slab usually provided, is

a. 0.4 H

b. 0.5 H

c. 0.6 H

d. 0.7 H

24. If the shear stress in a R.C.C. beam is

a. equal or less than 5 kg/cm2, no shear reinforcement is provided

b. greater than 4 kg/cm2, but less than 20 kg/cm2, shear reinforcement is provided

c. greater than 20 kg/cm2, the size of the section is changed

d. all the above.

25. The position of a brick when laid on its side 9 cm x 9 cm with its frog in the vertical plane, is called

a. brick on edge

b. brick on end

c. brick on bed

d. brick held vertically.

26. The type of pointing in which a V-shaped projection outside the wall surface, is provided, is called

a. recessed pointing

b. weather pointing

c. V-pointing

d. tuck pointing.

27. The condition not applicable to water cement ratio law, is

a. concrete specimens may be tested at any temperature

b. concrete specimens need be of same age

c. concrete specimens need be of same size

d. none of these.

28. For given water content, workability decreases if the concrete aggregates contain an excess of

a. flat particles

b. elongated particles

c. flaky paticles

d. all the above.

29. Pick up the correct statement from the following :

a. The internal pressure in a pipe running full is equal to vertical ordinate between hydraulic gradient line and centre of the pipe

b. In pressure pipes with water at rest, the pressure is equal to water head

c. Sudden closure of a valve, causes the water hammer pressure

d. All the above.

30. The most important and widely used tube well in India, is

a. strainer well

b. cavity well

c. slotted well

d. perforated pipe well.

31. The ratio of design discharge to the surface area of a sedimentation tank is called

a. surface loading

b. overflow rate

c. overflow velocity

d. all of these.

32. If the length of overland flow from the critical point to the mouth of drain is 13.58 km and difference in level between the critical point and drain mouth is 10 m, the inlet time is

a. 2 hours

b. 4 hours

c. 6 hours

d. 8 hours.

33. The main function of a diversion head works of a canal from a river, is

a. to control floods

b. to store water

c. to raise water level

d. all the above.

34. A counter berm is

a. a horizontal benching provided on the inside slope

b. a horizontal benching provided on the outside slope

c. a vertical benching provided on the outer edge of the bank

d. a vertical benching provided on the inner edge of the bank.

35. The main cause of silting up a channel,

a. inadequate slope

b. defective head regulator

c. defective outlets

d. all the above.

36. Process of meandering is due to

a. sediment load of streams

b. discharge and hydraulic properties of streams

c. relative erodibility of the bed and banks

d. all the above.

37. In water bound macadam roads, binding material, is

a. sand

b. stone dust

c. cement

d. brick dust.

38. Set-back distance is the distance between

a. road land boundary and building line

b. road land boundary and control line

c. building line and control line

d. road land boundary and control line.

39. For maximum strength and durability minimum percentage of cement, by weight is

a. 20%

b. 25%

c. 30%

d. 33%

40. For night travel, the length of a valley curve should be such that the head light beam distance is the same as

a. stopping sight distance

b. overtaking sight distance

c. sum of (a) and (b)

d. difference of (a) and (b)

41. The excavation exceeding 1.5 m in width and 10 sq.m in plan area with a depth not exceeding 30 cm, is termed as

a. Excavation

b. Surface dressing

c. Cutting

d. Surface excavation.

42. The expected out turn of half brick partition wall per mason per day is

a. 1.5 m3

b. 2.0 m3

c. 4.0 m2

d. 5.0 m2

43. According to ISI method of measurement, the order of the sequence is

a. Length, breadth, height

b. Breadth, length, height

c. Height, length, breadth

d. None of these.

44. Pick up the incorrect statement from the following:

a. The built up covered area at the floor level of any storey of a building is called plinth area

b. The usable covered area of the rooms of any storey of a building is called carpet area

c. The carpet area of a building along with area of its kitchen, pantry, store, lavatory, bath room and glazed verandah, is called floor area

d. None of these.

45. The most popular type of organisation used for Civil Engineering Constructions, is

a. line organisation

b. line and staff organisation

c. functional organisation

d. effective organisation.

46. The time which results in the leasi, possible construction cost of an activity, is known

a. normal time

b. slow time

c. crash time

d. standard time.

47. Railway projects are treated as

a. light construction

b. heavy construction

c. industrial construction

d. none of these.

48. Frequency distribution curves

a. having a single lump, are called uninodal curves

b. if symmetrical, are called normal curves

c. if not symmetrical, are called skew curves

d. all the above.

49. The bearing of the runway at threshold is 290°, the runway number is

a. N 70° W

b. 290°

c. 29°

d. W 20° N

50. Pick up the correct statement from the following:

a. The distance between the points of intersection of the extreme tangents to the transition curve is kept greater than 7500 m x sum of grade changes at the point of intersection

b. The rate of change of grade is limited to 0.3% per 30 m length of the curve

c. According to I.C.A.O. the maximum longitudinal gradient along a runway is limited to 1.5%

d. All the above.