***Sub Engineer***

1. The orthographical projection of a traverse leg upon the reference meridian, is known as

a. departure of leg

b. latitude to the leg

c. co-ordinate of the leg

d. bearing of the leg.

2. Staff readings on pegs x and y from X station are 1.755 m and 2.850 m, and from station Y on staff head at Y and X are 0.655 m and 1.560 m. If reduced level of X is 105.5 m, the reduced level of Y is

a. 104.0 m

b. 104.5 m

c. 105.0 m

d. 105.5 m

3. The operation of making the algebraic sum of latitudes and departures of a closed traverse, each equal to zero, is known

a. balancing the sights

b. balancing the departures

c. balancing the latitudes

d. balancing the traverse.

4. Volume of the earth work may be calculated by

a. end areas

b. Prismoidal formula

c. Trapezoidal

d. all the above.

5. Acrylic sheets

a. are generally unaffected by most household detergents

b. possess the light transmission rate of 93%

c. are available in various shapes

d. all the above.

6. Initial setting of cement is caused due to

a. Tri-calcium silicate

b. Di-calcium silicate

c. Tri-calcium aluminate

d. Tetra calcium alumino ferrite.

7. Pick up the incorrect statement from the following:

a. Plastics are chemical resistant

b. Plastics are durable

c. Plastics are ductile

d. Plastics are excellent electric insulators.

8. Cast iron contains carbon approximately

a. 1.5% to 5.5%

b. 0.05% to 1.75%

c. 0.250%

d. none to these.

9. A simply supported rolled steel joist 8 m long carries a uniformly distributed load over it span so that the maximum bending stress is 75 N/mm2. If the slope at the ends is 0.005 radian and the value of E = 0.2 x 106 N/mm2, the depth of the joist, is

a. 250 mm

b. 300 mm

c. 350 mm

d. 400 mm

10. Keeping the depth d constant, the width of a cantilever of length l of uniform strength loaded with a uniformly distributed load w varies from zero at the free end and

a.  at the fixed end

b.  at the fixed end

c.  at the fixed end

d.  at the fixed end

11. A simply supported beam (l + 2a) with equal overhangs (a) carries a uniformly distributed load over the whole length, the B.M. changes sign if

a. l > 2a

b. l < 2a

c. l = 2a

d. l = 4a

12. For the same height, the bottom width for no tension,

a. for triangular section is more than rectangular section

b. for rectangular section is more than triangular section

c. for triangular section is same as that of a rectangular section

d. none of these.

13. The horizontal component of the force on a curved surface is equal to

a. weight of liquid vertically below the curved surface

b. force on a vetical projection of the curved surface

c. product of pressure at its centroid and the area

d. weight of liquid retained by the curved area.

14. Atmospheric pressure is equal to water column head of

a. 9.81 m

b. 5.0 m

c. 10.30 m

d. 7.5 m.

15. The radius of gyration of the water line of a floating ship is 4 m and its metacentric height is 72.5 cm. The period of oscillation of the ship, is

a. 2π

b. 3π

c. 4π

d. π/2

16. The instrument used for measuring the velocity of flow, is known as

a. venturimeter

b. orifice meter

c. pitot tube

d. none of these.

17. The direct shear test suffers from the following disadvantage:

a. Pore water pressure cannot be measured

b. Shear stress on the failure plane is not uniform.

c. The area under the shear and vertical loads does not remain constant throughout the test

d. All the above.

18. 260 g of wet soil was taken in a pycnometer jar of weight 400 g in order to find the moisture content in the soil, with specific gravity of soil particles 2.75. The weight of soil and remaining water filled in pycnometer without air bubbles was 1415 g and the weight of pycnometer filled with water alone was 1275 g. The moisture content in the soil is

a. 24.2%

b. 18.2%

c. 53.8%

d. none of these.

19. A moist soil sample of volume 60 cc. weighs 108 g and its dried weight is 86.4 g. If its absolute density is 2.52, the degree of saturation is

a. 54%

b. 64%

c. 74%

d. 84%

20. You are given a sample of soil containing coarse grains to determine its water content, you will use

a. pycnometer

b. oven-drying method

c. calcium carbide method

d. alcohol method.

21. The width of the flange of a T-beam, which may be considered to act effectively with the rib depends upon

a. overall thickness of the rib

b. centre to centre distance between T-beams

c. span of the T-beam

d. all the above.

22. Pick up the correct statement from the following:

a. Lateral reinforcement prevents the shearing of concrete on diagonal plane

b. Lateral reinforcement stops breaking away of concrete cover, due to buckling

c. Lateral reinforcement in R.C.C. columns, is kept not less than 5 mm diameter

d. All the above.

23. If W is weight of a retaining wall and P is the horizontal earth pressure, the factor of safety against sliding, is

a. 1.25

b. 1.5

c. 2.0

d. 2.5

24. In the zone of R.C.C. beam where shear stress is less than 5 kg/cm2, nominal reinforcement is provided at a pitch of

a. one-half lever arm of the section

b. one-third lever arm of the section

c. lever arm of the section

d. one and half lever arm of the section.

25. The angle between skew back of a flat arch and the horizontal, is kept approximately equal to

a. 30°

b. 60°

c. 90°

d. 120°.

26. For plastering the exposed brick walls, the cement sand mortar should be

a. 1:3

b. 1:4

c. 1:6

d. 1:8.

27. The commonly used material in the manufacture of cement is

a. sand stone

b. slate

c. lime stone

d. graphite.

28. Pick up the correct statement from the following:

a. Continuous grading is not necessary for obtaining a minimum of air voids

b. The omission of a certain size of aggregate is shown by a straight horizontal line on the grading curve

c. The omission of a certain size of aggregate in concrete increases the workability but also increases the liability to segregation

d. All the above.

29. Demand for public uses in a city, does not include water required for

a. watering of public parks

b. watering of public gardens

c. sprickling on roads

d. drinking purposes.

30. De-chlorination is followed by

a. post-chlorination

b. pre-chlorination

c. double-chlorination

d. super-chlorination.

31. The self-cleansing velocity of water flowing through pipe lines, is

a. 2 metres/sec

b. 1 metre/sec

c. 0.5 metre/sec

d. 0.25 metre/sec.

32. For detecting the nitrates in sewage, the colour may be developed by adding

a. Pottassium permanganate

b. Sulphuric acid and napthamine

c. Phenol-di-sulphuric acid and pottassium hydroxide

d. None of these.

33. In rigid module, the discharge

a. is independent of water levels in the distributary and water course

b. depends upon the water level in distributary

c. depends upon the water level in the water course

d. depends upon the water levels of both, i.e. distributary and water course

34. If the optimum depth of kor watering for a crop is 15.12 cm, the outlet factor for the crop for four week period in hectares per cumec, is

a. 1200

b. 1400

c. 1600

d. 2000

35. The Lacey's regime velocity is proportional to

a. R1/2 S3/4

b. Q3/4 S1/3

c. R3/4 S1/3

d. R2/3 S1/2

36. The structure constructed to allow drainage water to flow under pressure through an inverted syphon below a canal, is called

a. super passage

b. aqueduct

c. super-aqueduct

d. syphon aqueduct.

37. The desirable camber for straight roads with water bound macadam or gravel surface, is

a. 1 in 40 to 1 in 33

b. 1 in 150 to 1 in 140

c. 1 in 160 to 1 in 140

d. none of these.

38. Hill roads are

a. provided camber along spur curves

b. provided camber along valley curves

c. seldom provided a camber

d. None of these.

39. In a braking test, a vehicle travelling at 36 km ph was stopped at a braking distance of 8.0 m. The average value of the vehicle's skid resistance (friction coefficient) is

a. 0.64

b. 6.25

c. 0.16

d. none of these

40. Border Roads Organisation for hilly regions, was formed in

a. 1954

b. 1958

c. 1960

d. 1962

41. Cost of fittings and their fixing is specified for the following sanitary fittings

a. Flushing pipes

b. Lavatary basins

c. Sinks

d. All the above.

42. Pick up the correct statement from the following:

a. The bent up bars at a support resist the negative bending moment

b. The bent up bars at a support resist the sharing force

c. The bending of bars near supports is generally at 45°

d. All the above.

43. Pick up the correct statement regarding the centre line method of estimating a building

a. Product of the centre line of the walls and area of cross-section of any item, gives total quantity of the item

b. The centre line is worked out separately for different sections of walls of a building

c. The centre line length is reduced by half the layer of main wall joining the partition wall

d. All the above.

44. The measurement is not made in square metres in case of

a. D.P.C. (Damp proof course)

b. Form works

c. Concrete Jaffries

d. R.C. Chhajja.

45. PERT is

a. limited of event oriented diagrams

b. used for research and development projects

c. based on three time estimates for activities linking up two events

d. all the above.

46. The first method invented for planning projects, was

a. Bar chart method

b. Milestone chart

c. Critical path method (CPM)

d. Programme Evaluation and Review Technique (PERT)

47. Pick up the correct statement from the following:

a. Programme Evaluation and Review Technique, is event oriented

b. Programme Evaluation and Review Technique is not event oriented

c. Critical Path Method is event oriented

d. Critical Path method is event oriented.

48. Pick up the incorrect statement from the following:

a. The difference between the maximum time available for the job and actual time it consumes, is called total float

b. The difference between the latest start time and earliest start time of an activity, is called total float

c. The difference between the earliest finish time of an activity and the earliest start time of its successor activity, is called free float of the activity

d. None of these.

49. International Civil Aviation Organisation (I.C.A.O.) was set up at Montreal (Canada), in

a. 1929

b. 1939

c. 1947

d. 1950

50. Beaufort scale is used to determine

a. strength of winds

b. direction of winds

c. height of air-crafts

d. none of these.