***Sub Engineer***

1. The bearing of C from A is N 30° E and from B, 50 metres east of A, is N 60° W. The departure of C from A is

a. 50 m

b. 50 3 m

c. 25 3 m

d. 25 m

2. An angles of 45° with a chain line may be set out with

a. optical square

b. open cross staff

c. Fench cross staff

d. prismatic square.

3. Designation of a curve is made by :

a. angle subtended by a chord of any length

b. angle subtended by an arc of specified length

c. radius of the curve

d. curvature of the curve.

4. To orient a plane table at a point P roughly south of the mid-point of two inaccessible conical hill stations A and B in the plains, a point C is selected in line with AB and table is oriented at C by bringing ab in line with AB. A ray is then drawn towards P and at P the table is oriented by back ray method. The orientation so obtained, is

a. unique and correct

b. incorrect

c. manifold and correct

d. not reliable.

5. Cast iron

a. is manufactured in required shapes

b. may contain 2 to 5 per cent of carbon with other impurities

c. is remelted in a cupola furnace

d. all the above.

6. Slump test for concrete is carried out, to determine

a. strength

b. durability

c. workability

d. water content.

7. Pick up the correct statement from the following:

a. Formaldehyde is prepared synthetically from methane

b. The melamine when reacted with formaldehyde forms the melamine-formaldehyde resin

c. The plastics made from melamine formaldehyde resin, are used for electrical insulators

d. All the above.

8. Pick up the correct statement from the following:

a. The bulking of sand is taken into account while volumetric proportioning of the aggregates

b. The dry sand and the sand completely flooded with water, have practically the same volume

c. The expansion and contraction joints are provided if concrete structures exceed 12 m in length

d. All the above.

9. The greatest load which a spring can carry without getting permanently distorted, is called

a. stiffness

b. prof resilience

c. proof stress

d. proof load.

10. Beams composed of more than one material, rigidly connected together so as to behave as one piece, are known as

a. Compound beams

b. Indeterminate beams

c. Determinate beams

d. Composite beams.

11. The slenderness ratio of a vertical column of a square cross-section of 2.5 cm sides and 300 cm length, is

a. 240

b. 360

c. 416

d. 500

12. The point of contraflexture occurs in

a. continuous beams only

b. over hanging beams only

c. all types of beams

d. both (a) and (b).

13. In two dimensional flow the components of velocity are given by u = ax; v = by. The stream lines will be

a. circular

b. parabolic

c. hyperbolic

d. elliptical.

14. The acceleration f required to accelerate a rectangular tank containing water horizontally so that the slope of its free surface is 45°, is

a. g

b. 2 g

c. 2.5 g

d. 3 g.

15. On an inclined plane, centre of pressure is located

a. at the centroid

b. above the centroid

c. below the centroid

d. anywhere.

16. For a most economical rectangular channel, the hydraulic mean depth, is equal to

a. the depth of flow

b. half the depth of flow

c. one-third depth of flow

d. None of these.

17. Geologic cycle for the formation of soil, is

a. Weathering → upheaval → transportation → deposition

b. Transportation → upheaval → weathering → deposition

c. Weathering → transportation → deposition → upheaval

d. None of these.

18. A pile is being driven with a drop hammer weighing 1800 kg and having a free fall of 1.00 m. If the penetration with last blow is 5 mm, the load carrying capacity of the pile, -according to the Engineering News formula, is

a. 100 tonnes

b. 50 tonnes

c. 20 tonnes

d. 10 tonnes.

19. Si particles

a. show dilatancy

b. swell when moist

c. possess high strength when dry

d. disintegrate easily.

20. Pick up the clay soil group which does not swell when wet from the following :

a. Kaolinite group

b. Illite group

c. Vermiculite group

d. Montrorillonite group.

21. For the design of a simply supported T-beam the ratio of the effective span to the overall depth of the beam is limited to

a. 15

b. 20

c. 25

d. 30

22. The self-weight of the footing, is

a. not considered for calculating the upward pressure on footing

b. also considered for calculating the upward pressure on footihg

c. not considered for calculating the area of the footing

d. both (b) and (c)

23. The maximum permissible size of aggregates to be used in casting the ribs of a slab, is

a. 7.5 mm

b. 10 mm

c. 15 mm

d. 20 mm

24. For M 150 mix concrete, according to I.S. specifications, local bond stress, is

a. 10 kg/cm2

b. 15 kg/cm2

c. 20 kg/cm2

d. 25 kg/cm2

25. To stagger vertical joints in successive courses of a wall, a piece of brick is generally used at the end of the course, which is known as

a. bat

b. header

c. stretcher

d. closer.

26. The opening provided in sloping roof with its top parallel to the roof surface, is called

a. dormer window

b. sky light window

c. lantern window

d. louvered window.

27. Pick up the correct statement from the following:

a. Water lubricates the mixture of gravel, sand and cement

b. Only a small quantity of water is required for hydration of cement

c. Strength of concrete structure largely depends upon its workability

d. All the above.

28. An excess of flaky particles in concrete aggregates

a. increases the quantity of water and sand

b. affects the durability of concrete

c. more than 15% are not desirable

d. all the above.

29. From the surface of reservoir, evaporation may be minimised by sprinkling

a. Hydrochloric acid

b. Cetyl alcohol

c. Methane

d. None of these.

30. Mud balls may be removed by

a. breaking and washing

b. washing the filter with a solution of caustic soda

c. removing, cleaning and replacing the damaged sand

d. all the above.

31. In 1000 kg of sewage, the total solids approximate

a. 0.5 to 1.0 kg

b. 1 kg to 2.0 kg

c. 5 kg to 7.5 kg

d. 7.5 kg to 10 kg.

32. The width of a settling tank with 2 hour detention period for treating sewage 378 cu m per hour, is

a. 5.5 m

b. 6 m

c. 6.5 m

d. 7 m.

33. If the straight sides of a triangular section of a lined canal with circular bottom of radius D, make an angle θ with horizontal, the hydraulic mean depth is

a. D/2

b. D/3

c. D/4

d. D/5

34. According to G.W. Pickles the effect of confining the flood water of a river between levee, is to increase

a. water surface elevation during floods

b. maximum discharge

c. velocity and scouring action

d. surface slope of streams above the leveed portion.

35. If the irrigation efficiency is 80%, conveyance losses are 20% and the actual depth of watering is 16 cm, the depth of water required at the canal outlet, is

a. 15 cm

b. 20 cm

c. 25 cm

d. 30 cm

36. Finally formed berms in canals are provided for

a. protection of banks erosion by the waves

b. control of seepage losses

c. strengthening of banks

d. all the above.

37. The absolute minimum radius of horizontal curve for a design speed 60 km ph is

a. 131 m

b. 210 m

c. 360 m

d. none of these.

38. Retaining walls are generally constructed with dry rubble stones with 60 cm top width and

a. 1 : 2 front batter

b. 1 : 3 front batter

c. 1 : 4 front batter

d. 1 : 5 front batter

39. The maximum comfortable retardation applied to moving vehicles, is

a. 3.42 m/sec2

b. 4.42 m/sec2

c. 5.56 m/sec2

d. 7.80 m/sec2

40. Pick up the incorrect statement from the following. The width of the right-of-way is decided so as to accommodate

a. side slopes

b. horizontal curve

c. vertical curve

d. provision of future widening.

41. While estimating a reinforced cement structure, the omitted cover of concrete is assumed

a. at the end of reminforcing bar, not less than 25 mm or twice the diameter of the bar

b. in thin slabs, 12 mm minimum or diameter of the bar whichever is more

c. for reinforcing longitudinal bar in a beam 25 mm minimum or diameter of the largest bar which is more

d. All the above.

42. If B is the width of formation, d is the height of the embankment, side slope S : 1, for a highway with no transverse slope, the area of cross-section is

a. B + d + Sd

b. Bd + Sd2

c. B x d - Sd1/2

d. (Bd + Sd2)

43. The brick work is not measured in cu m in case of

a. One or more than one brick wall

b. Brick work in arches

c. Reinforced brick work

d. Half brick wall.

44. The height of the sink of wash basin above floor level is kept

a. 60 cm

b. 70 cm

c. 75 cm to 80 cm

d. 80 cm

45. Completion of an activity on CPM network diagram, is generally known

a. Event

b. Node

c. Connector

d. All the above.

46. A dummy activity

a. is artificially introduced

b. is represented by a dotted line

c. does not consume time

d. all the above.

47. For completion of a project, the critical path of the network represents

a. minimum time

b. maximum time

c. maximum cost

d. minimum cost.

48. The three time estimates for the activities of the network shown in the given figure are shown above their arrows. The earliest expected time for the event 4, is

a. 19

b. 14

c. 24

d. None of these.

49. International Civil Aviation Organisation (I.C.A.O.) was set up at Montreal (Canada), in

a. 1929

b. 1939

c. 1947

d. 1950

50. Pick up the incorrect statement from the following:

a. L.M.M. is installed at 1.0 km from the threshold

b. Localizer antenna is installed at 300 m from the other end of the runway

c. Glide slope antenna is installed at the centre of the runway about 150 m on one side

d. None of these.