***Sub Engineer***

1. Two hill tops A and B 20 km apart are intervened by a third top C. If the top most contour of the three hill tops are of the same value, state whether the line of sight AB

a. passes clear of hill top C

b. passes below the hill top C

c. grazes the hill top C

d. none of these.

2. You have to observe an included angle with better accuracy than what is achievable by a vernier, you will prefer the method of

a. repetition

b. reiteration

c. double observations

d. exactness.

3. For true difference in elevations between two points A and B, the level must be set up

a. at any point between A and B

b. at the exact mid point of A and B

c. near the point A

d. near the point B.

4. The angle of intersection of a contour and a ridge line, is

a. 30°

b. 45°

c. 60°

d. 90°.

5. German silver is an alloy of

a. silver, gold and lead

b. copper, nickel and zinc

c. copper, brass and zinc

d. brass, silver and zinc.

6. Pick up the non-inflammable plastic from the following:

a. Cellulose acetate plastics

b. Polyvinyl chloride plastics

c. Phenol formaldehyde plastic

d. Urea formaldehyde plastic.

7. The most durable varnish is

a. spirit varnish

b. turpentine varnish

c. oil varnish

d. none of these.

8. Brittleness of cold is due to an excess of

a. sulphur

b. carbon

c. phosphorus

d. silicon.

9. A simply supported beam which carries a uniformly distributed load has two equal overhangs. To have maximum B.M. produced in the beam least possible, the ratio of the length of the overhang to the total length of the beam, is

a. 0.207

b. 0.307

c. 0.407

d. 0.508

10. The tangential component of stress on an plane inclined θ° to the direction of the force, may be obtained by multiplying the normal stress by

a. cos θ

b. tan θ

c. sin θ cos θ

d. sin2θ

11. The phenomenon of slow extension of materials having constant load, i.e. increasing with the time is called

a. creeping

b. yielding

c. breaking

d. none of these.

12. A shaft turning 150 r.p.m. is subjected to a torque of 150 kgm. Horse power transmitted by the shaft is

a. π

b. 10 π

c. π2

d. 1/π

13. Manometers are used to measure

a. pressure in water channels, pipes etc.

b. difference in pressure at two points

c. atmospheric pressure

d. very low pressure.

14. Hydraulic grade line

a. remains above the centre line of conduit

b. remains below the centre line of conduit

c. remains parallel to the centre line of conduit

d. may be above or below the centre line of conduit.

15. A rise or fall of liquid in a glass tube of a very small diameter when dipped is

a. directly proportional to the force per unit length of periphery

b. direcly proportional to the sine of the angle of contact

c. directly proportional to the specific weight of liquid

d. inversely proportional to the diameter of the glass tube.

16. Gauge pressure is

a. absolute pressure - atmospheric pressure

b. absolute pressure + atmospheric pressure

c. atmospheric pressure - absolute pressure

d. none of these.

17. According to Coulomb's wedge theory, the active earth pressure slides the wedge

a. down and outwards on a slip surface

b. up and inwards on a slip surface

c. horizontal upward and parallel to base

d. horizontal inward and parallel to base.

18. The compression index of a soil

a. decreases with an increase in the liquid limit

b. increases with an increase in the liquid limit

c. decreases with an increase in the plastic limit

d. is not related with plastic limit.

19. Over-consolidation of soils is caused due to

a. erosion of over burden

b. melting of ice sheets after glaciation

c. permanent rise of water table

d. all the above.

20. Determination of water content of a soil sample suspected to contain gypsum is made by drying the sample for longer period at a temperature not more than

a. 60°C

b. 80°C

c. 100°C

d. 110°C

21. Pick up the correct statement from the following:

a. Lateral reinforcement prevents the shearing of concrete on diagonal plane

b. Lateral reinforcement stops breaking away of concrete cover, due to buckling

c. Lateral reinforcement in R.C.C. columns, is kept not less than 5 mm diameter

d. All the above.

22. In a singly reinforced beam, if the permissible stress in concrete reaches earlier than that in steel, the beam section is called

a. under-reinforced section

b. over reinforced section

c. economic section

d. critical section.

23. The radius of a bar bend to form a hook, should not be less than

a. twice the diameter

b. thrice the diameter

c. four times the diameter

d. five times the diameter

24. The minimum head room over a stair must be

a. 205 cm

b. 210 cm

c. 200 cm

d. 230 cm

25. The sill of a common wooden partition is

a. vertical wooden member on either end

b. lower horizontal wooden member

c. upper horizontal wooden member

d. intermediate horizontal wooden member.

26. The under surface of an arch, is called

a. soffit

b. intrados

c. haunch

d. back.

27. The commonly used material in the manufacture of cement is

a. sand stone

b. slate

c. lime stone

d. graphite.

28. The datum temperature for maturity by Plowman, is

a. 23°C

b. 0°

c. - 5.6°C

d. - 11.7°C

29. P.V.C. pipes can withstand pressure bead of water upto

a. 50 m

b. 75 m

c. 100 m

d. 125 m.

30. Abyssinian tube well is a special type of

a. slotted type wells

b. cavity type wells

c. strainer type well

d. none of these.

31. The disintegrating pump in which solid matter is broken up before passing out, is

a. centrifugal pump

b. reciprocating pump

c. pneumatic ejector

d. none of these.

32. For providing an Indian type W.C., the R.C.C. slabs in the toilet portion

a. should be sunk by 20 cm

b. should be kept 20 cm above the adjacent portion

c. should be sunk by 50 cm

d. need not be sunk.

33. The top of the capillary zone

a. lies below the water table at every point

b. lies above the water table at every point

c. coincides the water table at every point

d. none of these.

34. Water-shed line is abondened for aligning an irrigation canal if

a. water shed forms a sharp loop

b. canal has to take off from a river

c. towns and villages are located on the water shed line

d. all the above.

35. Pick up the correct statement from the following:

a. Gravity water is harmful to crops

b. Hygroscopic water remains attached to soil molecules by chemical bond

c. Capillary moisture held in the soil pores against gravity by surface tension, is utilised by plants

d. All the above.

36. Lane's weighted creep theory assumes

a. double weightage to horizontal creep and one weightage to vertical creep

b. triple weightage to horizontal creep and one weightage to vertical creep

c. triple weightage to vertical creep and one weightage to horizontal creep

d. double weightage to vertical creep and one weightage to horizontal creep.

37. The ideal shape of a transition curve, is

a. clothoid

b. cubic spiral

c. cubic parabola

d. lamniscate

38. Design of horizontal curves on highways, is based on

a. permissible friction on the road surface

b. permissible centrifugal ratio

c. permissible maximum super-elevation

d. all the above.

39. The best compromise between the increase of the length of a highway and reduction in its load carrying capacity, is the ruling gradient

a. 1 in 15

b. 1 in 20

c. 1 in 25

d. l in 30

40. Roadway width for a National highways and State highways (two-lanes) is

a. 12 m

b. 9 m

c. 9.5 m

d. 15 m

41. The value of 'B' of Indian type W.C. shown in the given figure is :

a. 45 cm

b. 50 cm

c. 30 cm

d. 25 cm

42. The expected out turn of half brick partition wall per mason per day is

a. 1.5 m3

b. 2.0 m3

c. 4.0 m2

d. 5.0 m2

43. Pick up the incorrect statement from the following:

a. Dimensions are measured to the nearest 0.01 m

b. Areas are measured to the nearest 0.01 sq.m

c. Cubic contents are measured to the nearest 0.1 cm m

d. Weights are measured to the nearest 0.001 tonnes

44. Referring of given figure, pick up the correct statement from the following:

a. The total length of centre line of four walls is 20 m

b. Length of long wall out-to-out is 6.80 m

c. Length of short walls in-to-in is 3.20 m

d. All the above.

45. In the given figure, the network of a project represents

a. activity of an excavation of a footing

b. activity of an excavation which starts at event No. 1 and ends at even No. 2

c. activity of excavation which takes 8 units of time

d. none of these.

46. If the total float and duration of an activity are 5 and 10 days respectively, the particular activity can be

a. started 5 days later

b. completed 5 days later

c. performed at slower rate in 15 days

d. all the above.

47. The main principle of an organisation, is

a. cohegency

b. effective control at all levels

c. delegation of authority

d. all the above.

48. A Milestone chart

a. shows the interdependencies of various jobs

b. depicts the delay of jobs, if any

c. points outgoing ahead of schedule of jobs, if any

d. none of these.

49. Pick up the correct statement from the following:

a. The propellers which are driven by turbine engines, are technically called turboprops

b. The aircrafts which obtain the thrust directly from turbine engine, are called turbo-jets

c. The aircraft which is filled with a fan infront of turbo-jet engine, is called turbofan propelled aeroplane

d. All the above.

50. The lift off distance is the distance along the centre of the runway between the starting point and

a. end of the runway

b. end of stopway

c. point where air craft becomes air borne

d. point where air craft attains a height of 10.7 m