***Sub Engineer***

1. Pick up the incorrect statement from the following :

a. while measuring a distance with a tape of length 100.005 m, the distance to be increasing by 0.005 m for each tape length

b. an increase in temperature causes a tape to increase in length and the measured distance is too large

c. the straight distance between end points of a suspended tape is reduced by an amount called the sag correction

d. a 100 m tape of cross section 10 mm x 0.25 mm stretches about 10 mm under 5 kg pull.

2. An ideal vertical curve to join two gradients, is

a. parabolic

b. elliptical

c. hyperbolic

d. none of these.

3. A bearing of a line is also known as

a. magnetic bearing

b. true bearing

c. azimuth

d. reduced bearing

4. There are two stations A and B. Which of the following statements is correct :

a. the fore bearing of AB is AB

b. the back bearing of AB is BA

c. the fore and back bearings of AB differ by 180°

d. all the above.

5. In   the   method   of   condensation polymerization,

a. low-molecular substances are removed from the high molecular substance

b. the reaction proceeds with an evolution of ammonia

c. the reaction proceeds with an evolution of hydrogen chloride

d. all the above.

6. Soundness test of cement determines

a. quality of free lime

b. ultimate strength

c. durability

d. initial setting

7. Dextrin is

a. starch glue

b. albumin glue

c. rubber based adhesive

d. none to these.

8. The minimum compressive strength of Ilnd class bricks should be

a. 75 kg/cm2

b. 90 kg/cm2

c. 100 kg/cm2

d. 120 kg/cm2

9. For calculating the permissible stress  is the emprical formula, known as

a. Straight line formula

b. Parabolic formula

c. Perry's formula

d. Rankine's formula.

10. The forces acting normally on the cross section of a bar shown in the given figure introduce

a. compressive stress

b. tensile stress

c. shear stress

d. none of these.

11. When a rectangular beam is loaded longitudinally, shear develops on

a. bottom fibre

b. top fibre

c. middle fibre

d. every-horizontal plane.

12. The value of Poisson's ratio always remains

a. greater than one

b. less than one

c. equal to one

d. none of these.

13. Highest dam in India, is

a. Bhakra dam

b. Hirakund dam

c. Nagarjuna Sagar dam

d. Iddiki dam.

14. Which one of the following statements is true ?

a. The value of kinetic energy correction factor for turbulent flow lies between 1.03 to 1.06

b. The value of kinetic energy correction factor for laminar flow is 2

c. The practical value of kinetic energy correction factor for turbulent flow is unity

d. all the above.

15. On an inclined plane, centre of pressure is located

a. at the centroid

b. above the centroid

c. below the centroid

d. anywhere.

16. Barometres are used to measure

a. difference in pressure at two points

b. atmospheric pressure

c. very low pressure

d. very high pressure.

17. The slip at critical angle, is generally known

a. δ 1-line

b. rupture plane

c. slip plane

d. all the above.

18. The seepage force in a soil, is

a. perpendicular to the equipotential lines

b. proportional to the exit graident

c. proportional to the head loss

d. all the above.

19. Hydrometer readings are corrected for:

a. meniscus correction

b. dispersing agent correction

c. meniscus and dispersing agent corrections

d. temperature, meniscus and dispersing agent corrections.

20. Water content of a soil sample is the difference of the weight of the given sample at the given temperature and the weight determined after drying it for 24 hours at temperature ranging from

a. 90° to 95°C

b. 95° to 100°C

c. 103° to 105°C

d. 105° to 110°C

21. The minimum head room over a stair must be

a. 205 cm

b. 210 cm

c. 200 cm

d. 230 cm

22. The minimum cube strength of concrete used for a prestressed member, is

a. 150 kg/cm2

b. 250 kg/cm2

c. 350 kg/cm2

d. 400 kg/cm2

23. A pre-stressed concrete member is preferred because

a. its dimensions are not decided from the diagonal tensile stress

b. large size of long beams carrying large shear force need not be adopted

c. removal of cracks in the members due to shrinkage

d. all the above.

24. In a simply supported slab, alternate bars are curtailed at

a. 1/5th of the span

b. 1/6th of the span

c. 1/7th of the span

d. none of these.

25. A cut in frame of a door to receive the shutter, is called

a. louver

b. stop

c. horn

d. rebate.

26. Pick up the correct statement from the following:

a. In a king post truss, principal rafter and tie beams are jointed together with a bridle joint.

b. Joint between the principal rafter and the king post is made by making tenon and mortice respectively

c. Joint between strut and king post, is generally of mortice and tenon type

d. All the above.

27. Pick up the incorrect statement from the following:

a. Cement bags should preferably be piled on wooden planks

b. Cement bags should be placed such that bags of one layer does not touch the bags of the adjacent layer

c. Width and height of the pile should not exceed 3 m and 2.70 m respectively

d. None of these.

28. Grading of sand causes great variation in

a. strength of concrete

b. durability of concrete

c. handing and placing of concrete

d. all the above.

29. Demand for public uses in a city, does not include water required for

a. watering of public parks

b. watering of public gardens

c. sprickling on roads

d. drinking purposes.

30. Pick up the correct statement from the following :

a. Average consumption of commercial use of water is 25% of total consumption

b. Consumption of water on public use is 10% of total consumption

c. Waste water and leakage is 15% of total consumption

d. all the above.

31. For the survival of fish in a river stream, the minimum dissolved oxygen is prescribed

a. 3 PPm

b. 4 PPm

c. 5 PPm

d. 10 ppm.

32. The asbestos cement sewers are

a. light in weight

b. not structurally strong to bear large compressive stress

c. susceptible to corrosion by sulphuric acid

d. All the above.

33. In rigid module, the discharge

a. is independent of water levels in the distributary and water course

b. depends upon the water level in distributary

c. depends upon the water level in the water course

d. depends upon the water levels of both, i.e. distributary and water course

34. Which one of the following statements is correct ?

a. The canal system and the drainage system are complementary

b. The canal system runs on ridges, gets divided into smaller channels and spreads its water on the land

c. The drainage system runs along the lowest line, collects drainage water from the slopes and joins the main stream

d. All the above.

35. Pick up the incorrect statement from the following

a. Side walls of a venturi head flume are splayed out from the end of the throat at 1 : 10 for a length of 4.5 m

b. Length of side walls should be such that the width of the flume is made equal to 2/3rd the bed width of the distributary

c. Once the width of the flume becomes 2/3rd of the width of the distributary, the splayed walls are increased to 1 in 3 to get full bed width

d. None of these.

36. Borrow pits should preferably be located in

a. field on the left side of the canal

b. field on the right side of the canal

c. fields on both sides of the canal

d. central half width of the section of the canal.

37. While calculating the overtaking sight distance, the height of the object above road surface, is assumed

a. 50 cm

b. 75 cm

c. 100 cm

d. 120 cm

38. Extra widening required at a horizontal curve on a single lane hill road of radius 80 m for a design speed of 50 km ph and for a vehicle with wheel base 6.0 m is

a. 0.225 m

b. 0.589 m

c. 1.250 m

d. none of these

39. The width of different roads as recommended in Nagpur plan by the Indian Road Conference for hilly region, is

a. same for National Highways

b. different for National Highways

c. same for State Highways

d. same for Major District roads.

40. The minimum superelevation in rolling terrain in plains, is limited to

a. 5%

b. 6%

c. 7%

d. 10%

41. The value of 'B' of Indian type W.C. shown in the given figure is :

a. 45 cm

b. 50 cm

c. 30 cm

d. 25 cm

42. The order of booking dimensions is

a. Length, breadth, height

b. Breadth, length, height

c. Height, breadth, length

d. None of these.

43. The expected out turn of cement concrete 1 : 2 : 4 per mason per day is

a. 1.5 m3

b. 2.5 m3

c. 3.5 m3

d. 5.0 m3

44. The plinth area of a building not includes

a. area of stair cover

b. internal shaft for sanitary installations up to 2 sq m. in area

c. lift and wall including landing

d. area of cantilevered porch.

45. Which one of the following represents an event ?

a. concrete cured

b. fixing of door

c. plastering of walls

d. selecting sites

46. The artificial activity which indicates that an activity following it, cannot be started unless the preceding activity is complete, is known as

a. event

b. free float

c. dummy

d. constant

47. Construction team means

a. an architect

b. an owner

c. a contractor

d. all the above.

48. If t is the duration of an activity, t1 is the latest finish possible moment of its preceding activity and t2 is the earliest start possible moment, the independent float of the activity is

a. (t1 - t2) - t

b. t - (t1 - t2)

c. (t1 + t2) - t

d. t + (t1 - t2)

49. Pick up the incorrect statement from the following:

a. L.M.M. is installed at 1.0 km from the threshold

b. Localizer antenna is installed at 300 m from the other end of the runway

c. Glide slope antenna is installed at the centre of the runway about 150 m on one side

d. None of these.

50. Pick up the correct statement from the following:

a. The distance between the points of intersection of the extreme tangents to the transition curve is kept greater than 7500 m x sum of grade changes at the point of intersection

b. The rate of change of grade is limited to 0.3% per 30 m length of the curve

c. According to I.C.A.O. the maximum longitudinal gradient along a runway is limited to 1.5%

d. All the above.