***Sub Engineer***

1. From any point on the surface with a given inclination

a. only one contour gradient is possible

b. two contour gradients are possible

c. indefinite contour gradients are possible

d. all the above.

2. While measuring with a metallic tape of 30 m length pull should be applied

a. 1 kg

b. 2 kg

c. 3 kg

d. 4 kg

3. In quadrantal bearing system, back bearing of a line may be obtained from its forward bearing, by

a. adding 180°, if the given bearing is less than 180°

b. subtracting 180°, if the given bearing, is more than 180°

c. changing the cardinal points, i.e. substituting N for S and E for W and vice-versa

d. none of these.

4. The bearing of lines OA and OB are 16° 10' and 332° 18', the value of the included angle BOA is

a. 316° 10'

b. 158° 28'

c. 348° 08'

d. 43°  52'

5. Kaolin is chemically classified as

a. metamorphic rock

b. argillaceous rock

c. calcareous rock

d. silicious rock.

6. Durability of building stone is affected by its

a. texture

b. resistance to atmosphere

c. location in structure

d. all the above.

7. Quartzite, a metamorphic stone is

a. brittle

b. crystalline

c. compact

d. All the above.

8. The colour of statuary marble used for sculptor's work, is

a. blue

b. white

c. green

d. yellow

9. Stress may be expressed in Newtons

a. per millimetre square (N/mm2)

b. per centimetre square (N/cm2)

c. per metre square (N/m2)

d. All of the above.

10. A body is said to be in equilibrium if

a. it moves horizontally

b. it moves vertically

c. it rotates about its C.G.

d. none of these.

11. A beam of length L supported on two intermediate rollers carries a uniformly distributed load on its entire length. If sagging B.M. and hogging B.M. of the beam are equal, the length of each overhang, is

a. 0.207 L

b. 0.307 L

c. 0.407 L

d. 0.5 L.

12. A solid cube is subjected to equal normal forces on all its faces. The volumetric strain will be x-times the linear strain in any of the three axes when

a. x = 1

b. x = 2

c. x = 3

d. x = 4

13. An ideal fluid is

a. incompressible

b. compressible

c. compressible and non-viscous

d. slightly affected by surface torque.

14. While applying the Bernoulli's equation  = total head, the work any section done on the flow system, if any

a. is added on the right side of the equation

b. is added on the left side of the equation

c. is ignored

d. none of these.

15. For the most economical trapezoidal open channel,

a. half of the top width must be equal to one of the sloping sides

b. the hydraulic mean depth must be equal to half the depth of flow

c. the semicircle drawn with top width as diameter must touch the three sides of the channel

d. All of these.

16. Most economical section of a triangular channel, is

a. equilateral triangle

b. right angled triangle

c. isosceles triangle with 45° vertex angle

d. right angled triangle with equal sides.

17. The shearing strength of a cohesion-less soil depends upon

a. dry density

b. rate of loading

c. confining pressure

d. nature of loading.

18. The maximum net pressure intensity causing shear failure of soil, is known

a. safe bearing capacity

b. net safe bearing capacity

c. net ultimate bearing capacity

d. ultimate bearing capacity.

19. The compression index of a soil

a. decreases with an increase in the liquid limit

b. increases with an increase in the liquid limit

c. decreases with an increase in the plastic limit

d. is not related with plastic limit.

20. If G is specific gravity of sand particles, e is porosity, the critically hydraulic gradient

a. ic =

b. ic =

c. ic =

d. ic =

21. If the permissible compressive and tensile stresses in a singly reinforced beam are 50 kg/cm2 and 1400 kg/cm2 respectively and the modular ratio is 18, the percentage area At of the steel required for an economic section, is

a. 0.596%

b. 0.696%

c. 0.796%

d. none of these.

22. The length of the straight portion of a bar beyond the end of the hook, should be at least

a. thrice the diameter

b. four times the diameter

c. five times the diameter

d. seven times the diameter.

23. If diameter of a reinforcement bar is d, the anchorge value of the hook is

a. 8d

b. 12d

c. 16d

d. none of these.

24. The radius of a bar bend to form a hook, should not be less than

a. twice the diameter

b. thrice the diameter

c. four times the diameter

d. five times the diameter

25. The minimum strength of the mortar used in load bearing brick masonry, is

a. 50 N/cm2

b. 100 N/cm2

c. 150 N/cm2

d. 200 N/cm2.

26. Raft foundations are used for :

a. providing increased area of foundation over poor bearing capacity of soil

b. spanning over small soft or loose pockets

c. counter acting the hydrostatic effect

d. all the above.

27. The maximum amount of dust which may be permitted in aggregates is

a. 5% of the total aggregates for low workability with a coarse grading

b. 10% of the total aggregates for low workability with a fine grading

c. 20% of the total aggregates for a mix having high workability with fine grading

d. all the above.

28. The bulk density of aggregates does not depend upon :

a. specific gravity of aggregates

b. grading of aggregates

c. size and shape of the container

d. none of these.

29. The capability of a soil mass of full width and depth to transmit water, is known

a. porosity

b. permeability

c. transmissibility

d. none of these.

30. E. Coli bacterias die in water having pH greater than

a. 6.5

b. 7.5

c. 8.5

d. 9.5

31. The rainfall at any place may be determined by

a. its intensity

b. its direction

c. its frequency

d. all the above.

32. The sewage is pumped up

a. from flat areas

b. from basements

c. across a high ridge

d. all the above.

33. A fall in a canal bed is generally provided, if

a. ground slope exceeds the designed bed slope

b. designed bed slope exceeds the ground slope

c. ground slope is practically the same as the designed bed slope

d. none of these.

34. Useful soil moisture for plant growth, is

a. capillary water

b. gravity water

c. hygroscopic water

d. chemical water

35. A counter berm is

a. a horizontal benching provided on the inside slope

b. a horizontal benching provided on the outside slope

c. a vertical benching provided on the outer edge of the bank

d. a vertical benching provided on the inner edge of the bank.

36. The down stream expansion head of a guide bank is extended to subtend an angle at the centre, equal to

a. 30°

b. 45°

c. 60°

d. 90°

37. Passing zones are generally not provided on

a. summit curves

b. horizontal curves

c. two lane highways

d. all the above.

38. In ideal pavement is constructed with

a. hard soil

b. Portland cement concrete

c. tar

d. none of these.

39. Extra widening required at a horizontal curve on a single lane hill road of radius 80 m for a design speed of 50 km ph and for a vehicle with wheel base 6.0 m is

a. 0.225 m

b. 0.589 m

c. 1.250 m

d. none of these

40. The desirable camber for straight roads with water bound macadam or gravel surface, is

a. 1 in 40 to 1 in 33

b. 1 in 150 to 1 in 140

c. 1 in 160 to 1 in 140

d. none of these.

41. While preparing a detailed estimate

a. Dimension should be measured correct to 0.01 m

b. Area should be measured correct to 0.01 sqm

c. Volume should be measured correct to O.Olcum

d. All the above.

42. The concrete work for the following part of the building of specified thickness is measured in square metres

a. Floors

b. D.P.C.

c. Wall panels

d. All the above.

43. The most reliable estiamte is

a. Detailed estimate

b. Preliminary estimate

c. Plinth area estimate

d. Cube rate estimate

44. The floor area includes the area of the balcony up to

a. 100%

b. 75%

c. 50%

d. 25%

45. For completion of a project, the critical path of the network represents

a. minimum time

b. maximum time

c. maximum cost

d. minimum cost.

46. Critical path method

a. provides a realistic approach to daily problems

b. avoids delays which are very common in bar charts

c. was invented by Morgan R. Walker of Dupot and James E. Kalley or Remington U.S.A. in 1957

d. All the above.

47. The critical activity has

a. maximum float

b. minimum float

c. zero float

d. none of these.

48. For the network shown in the given figure, the expected time for the activity

a. 1-2 is 4

b. 2-3 is 7

c. 3-4 is 8

d. all the above.

49. Pick up the incorrect statement from the following:

a. Channelization of pavement is caused by constant use of tri-cycle gears of aircraft

b. Blast pads are used over cohesionless soils to resist erosion due to tremendous speed of the jet blast

c. Over-run areas of at least 300 m length on either side of the runway are provided

d. None of these.

50. The landing and take off of the air craft is made against the direction of wind. In no case the centre line of the runway should make an angle with the wind direction exceeding

a. 10°

b. 20°

c. 30°

d. 40°