***Sub Engineer***

1. The defect of a lens whereby rays of white light proceeding from a point get dispersed into their components and conveyed to various foci, forming a blurred and coloured image is known as

a. chromatic aberration

b. spherical aberration

c. astigmatism

d. coma.

2. The horizontal angle between true meridian and magnetic meridian, is known

a. bearing

b. magnetic declination

c. dip

d. convergence.

3. The best method of interpolation of contours, is by

a. estimation

b. graphical means

c. computation

d. all of these.

4. The intercept of a staff

a. is maximum if the staff is held truly normal to the line of sight.

b. is minimum if the staff is held truly normal to the line of sight.

c. decreases if the staff is tilted away from normal

d. increases if the staff is tilted towards normal.

5. Brittleness of cold is due to an excess of

a. sulphur

b. carbon

c. phosphorus

d. silicon.

6. Pick up the correct statement from the following:

a. The commercial plywoods are available upto 150 cm wide and upto 300 cm long

b. The plywoods possess uniform tensile strength in all directions

c. The plywoods are light in weight

d. All the above.

7. The cast iron when heated to red heat with powdered red haemetite in an oven for increasing its toughness, is converted to

a. grey cast iron

b. white cast iron

c. mottled cast iron

d. toughed cast iron.

8. Chemically, marble is known as

a. metamorphic rock

b. argillaceous rock

c. calcareous rock

d. silicious rock.

9. The shape factor of standard rolled beam section varies from

a. 1.10 to 1.20

b. 1.20 to 1.30

c. 1.30 to 1.40

d. 1.40 to 1.50

10. In case of principal axes of a section

a. sum of moment of inertia is zero

b. difference of moment inertia is zero

c. product of moment of inertia is zero

d. none of these.

11. The moment diagram for a cantilever carrying a concentrated load at its free end, will be

a. triangle

b. rectangle

c. parabola

d. cubic parabola.

12. The tensile force required to cause an elongation of 0.045 mm in a steel rod of 1000 mm length and 12 mm diameter, is (where E = 2 x 106 kg/cm2)

a. 166 kg

b. 102 kg

c. 204 kg

d. 74 kg

13. When two layers of a fluid separated by dy move over the other with a difference of velocity dv, causes a shearing stress , where μ is known as

a. absolute viscosity

b. dynamic viscosity

c. viscosity

d. all the above.

14. Bernoulli's equation assumes that

a. fluid is homogeneous

b. flow is steady

c. flow is along the stream line

d. all the above.

15. In a venturimeter, the divergent cone is kept

a. shorter than convergent cone

b. equal to covergent cone

c. longer than convergent cone

d. none of these.

16. For a most economical rectangular channel, the width of the channel must be

a. equal to depth of flow

b. twice the depth of flow

c. half the depth of flow

d. None of these.

17. The ratio of the volume of voids to the total volume of the given soil mass, is known

a. porosity

b. specific gravity

c. void ratio

d. water content.

18. The passive earth pressure of a soil, is proportional to (where φ is the angle of internal friction of the soil.)

a. tan (45° - φ)

b. tan (45° + φ)

c. tan2 (45° - φ

d. tan2 (45° + φ)

19. The weight of a pycnometer containing 400 g sand and water full to the top is 2150 g. The weight of pycnometer full of clean water is 1950 g. If specific gravity of the soil is 2.5, the water content is

a. 5%

b. 10%

c. 15%

d. 20%

20. According to the Indian Standards the specific gravity is the ratio of the unit weight of soil solids to that of water at a temperature of

a. 17°C

b. 23°C

c. 27°C

d. 30°

21. The pitch of the main bars in a simply supported slab, should not exceed its effective depth by

a. four times

b. five times

c. six times

d. seven times.

22. According to I.S. : 456, slabs which span in two directions with corners held down, are assumed to be divided in each direction into middle strips and edge strips such that the width of the middle strip, is

a. two-third of the width of the slab

b. three-fourth of the width of the slab

c. four-fifth of the width of the slab

d. three-fifth of the width of the slab.

23. Total pressure on the vertical face of a retaining wall of height h acts parallel to free surface and from the base at a distance of

a. h/4

b. h/3

c. h/2

d. 2h/3

24. The length of lap in tension reinforcement should not be less than the bar diameter x (actual tension / four times the permissible average bond stress) if it is more than

a. 18 bar diameters

b. 24 bar diameters

c. 30 bar diameters

d. 36 bar diameters

25. The inclined support at the ends of treads and rises of a stair, is known as

a. baluster

b. header

c. string

d. beam.

26. The taper of precast concrete pile should not be more than

a. 1 cm per metre length

b. 2 cm per metre length

c. 4 cm per metre length

d. 5 cm per metre length.

27. Pick up the correct statement from the following:

a. The maximum size of the fine aggregate, is 4.75 mm and minimum 0.075 mm

b. The material having particles of size varying from 0.06 mm to 0.002 mm, is known as silt

c. The material having particles of size less than 0.002 mm, is known as clay

d. All the above.

28. 1.S.I. has specified the full strength of concrete after

a. 14 days

b. 21 days

c. 28 days

d. 35 days

29. In slow sland filters, the turbidity of raw water can be removed only up to

a. 60 mg/litre

b. 75 mg/litre

c. 100 gm/litre

d. 150 mg/litre.

30. Rapid sand filter

a. should be preceeded by coagulation and sedimentation

b. uses rapid sand as filter media

c. is used after slow sand filtering has been done

d. can combine disinfection also.

31. The asbestos cement pipes are generally laid

a. horizontally

b. vertically

c. at an angle of 30°

d. at an angle of 60°.

32. You are asked to design sewer pipes of diameters 0.4 m to 0.9 m at maximum flow, you will assume the sewer flow running at

a. full depth

b. half full

c. two third full

d. three fourth full.

33. If the straight sides of a triangular section of a lined canal with circular bottom of radius D, make an angle θ with horizontal, the hydraulic mean depth is

a. D/2

b. D/3

c. D/4

d. D/5

34. The ratio of the discharge over a trapezoidal crest to a rectangular crest of Sarda falls of identical parameters, is

a. 1.084

b. 1.074

c. 1.064

d. 1.054

35. If the irrigation efficiency is 80%, conveyance losses are 20% and the actual depth of watering is 16 cm, the depth of water required at the canal outlet, is

a. 15 cm

b. 20 cm

c. 25 cm

d. 30 cm

36. The ratio of the rate of change of discharge of an outlet to the rate of change in level of water surface in a distributary at its normal depth, is known as

a. efficiency

b. sensitivity

c. flexibility

d. modular limit.

37. A single lane carriage way whenever changes to two-lane carriage way, is affected through a taper of

a. 1 in 10

b. 1 in 15

c. 1 in 20

d. 1 in 15 to 1 in 20

38. The minimum width of the pavement of a National Highway should be

a. 5.7 m

b. 6.7 m

c. 7.7 m

d. 8.0 m

39. If the radii of a compound curve and a reverse curve are respectively the same, the length of common tangent

a. of compound curve will be more

b. of reverse curve will be more

c. of both curves will be equal

d. none of these.

40. The minimum superelevation in rolling terrain in plains, is limited to

a. 5%

b. 6%

c. 7%

d. 10%

41. The measurement is made for stone work in square metre in case of

a. Columns, lintels, copings

b. Building work

c. Dressed stones in Chajjas

d. (a) and (d) of the above.

42. While estimating a reinforced cement structure, the omitted cover of concrete is assumed

a. at the end of reminforcing bar, not less than 25 mm or twice the diameter of the bar

b. in thin slabs, 12 mm minimum or diameter of the bar whichever is more

c. for reinforcing longitudinal bar in a beam 25 mm minimum or diameter of the largest bar which is more

d. All the above.

43. Berms are provided in canals if these are

a. fully in excavation

b. partly in excavtion and partly in embankment

c. fully in embankment

d. All the above.

44. In case of laying gullies, siphons, intercepting traps, the cost includes

a. Setting and laying

b. Bed concreting

c. Connection to drains

d. All of these.

45. The critical activity has

a. maximum float

b. minimum float

c. zero float

d. none of these.

46. The most popular type of organisation used for Civil Engineering Constructions, is

a. line organisation

b. line and staff organisation

c. functional organisation

d. effective organisation.

47. A dummy activity

a. is artificially introduced

b. is represented by a dotted line

c. does not consume time

d. all the above.

48. The main advantage of line organisation, is :

a. defined responsibilities at all levels

b. rigid discipline in the organisation

c. ability of quick decision at all levels

d. all the above.

49. Pick up the correct statement from the following:

a. Approximate geometric centre of the landing area, is called air port reference point

b. The boundaries of horizontal surface and conical surface are marked with reference to air port reference point

c. The location of the air port on a map is the position of air port reference point

d. All the above.

50. Pick up the correct abbreviation from the following:

a. V.H.F, (very high frequency)

b. L.O.M. (low powered outer marker)

c. L.M.M. (low powered middle marker)

d. All the above