***Sub Engineer***

1. Pick up the correct statement from the following :

a. a back sight is the first sight taken after setting up the instrument in any position

b. the first sight on each change point is a fore sight

c. the second sight on each change point is a back sight

d. all the above.

2. A clinometer is used for

a. measuring angle of slope

b. correcting line of collimation

c. setting out right angles

d. defining natural features.

3. Tacheometric formula for horizontal distances using horizontal sights can also suitable be employed for inclined sights through θ by multiplying

a. the constants by cos2 θ

b. the constants by cos θ

c. the constants by sin θ

d. the multiplying constant by cos2 θ and additive constant by cos θ.

4. In a lemniscate curve the ratio of the angle between the tangent at the end of the polar ray and the straight, and the angle between the polar ray and the straight, is

a. 3

b. 4

c. 5

d. 3/2

5. Pick up the metal refractory from the following:

a. Molyblendum

b. Tungsten

c. Zicronium

d. All of these.

6. The material generally not used as extender in paints, is

a. powdered silica

b. gypsum

c. talc

d. zinc white.

7. The wedging is adopted for quarrying costly stratified rock such as

a. marble

b. limestone

c. sandstone

d. All the above.

8. The sequence of refractory materials according to increasing melting points is :

a. Dolomite, Magnesia, Bauxite, Chromite

b. Bauxite, Chromite, Dolomite, Magnesia

c. Magnesia, Bauxite, Dolomite, Chromite.

9. The force in CD of the truss shown in given figure, is

a. 3t tension

b. zero

c. 1.5t compression

d. 1.5t tension

10. The forces in the members of simple trusses, may be analysed by

a. graphical method

b. method of joints

c. method of sections

d. all the above.

11. For a simply supported beam of length L, the bending moment M is described as M = a (x - x3/L2), 0 ≤ x < L; where a is a constant. The shear force will be zero at

a. the supports

b. x = L/2

c. x = L/3

d. x = L/3

12. Strain energy of any member may be defined as work done on it

a. to deform it

b. to resist elongation

c. to resist shortening

d. all the above.

13. While applying the Bernoulli's equation  = total head, the work any section done on the flow system, if any

a. is added on the right side of the equation

b. is added on the left side of the equation

c. is ignored

d. none of these.

14. The thickness of a sharp crested weir is kept less than

a. one-half of the height of water on the sill

b. one-fourth of the height of water on the sill

c. two-third of the height of water on the sill

d. none of these.

15. Manometers are used to measure

a. pressure in water channels, pipes etc.

b. difference in pressure at two points

c. atmospheric pressure

d. very low pressure.

16. In case of laminar flow through a circular pipe,

a. momentum correction factor is 1.33

b. energy correction factor is 2.00

c. both (a) and (b)

d. Neither (a) nor (b).

17. The zero atmospheric pressure is at

a. mean sea level

b. water table

c. phreatic surface

d. both (c) and (d) of the above.

18. The clay soil mainly consists of

a. Montomorillonite

b. Illites

c. Vermiculite

d. All the above.

19. The pressure that builds up in pore water due to load increment on the soil, is termed

a. excess pore pressure

b. excess hydrostatic pressure

c. hydrodynamic pressure

d. all the above.

20. Cohesive soils are generally

a. plastic and also compressible

b. elastic and also compressible

c. plastic but incompressible

d. none of these.

21. If the width of the foundation for two equal columns is restricted, the shape of the footing generally adopted, is

a. square

b. rectangular

c. trapezoidal

d. triangular.

22. A pre-stressed concrete member is preferred because

a. its dimensions are not decided from the diagonal tensile stress

b. large size of long beams carrying large shear force need not be adopted

c. removal of cracks in the members due to shrinkage

d. all the above.

23. A circular slab subjected to external loading, deflects to form a

a. semi-hemisphere

b. ellipsoid

c. parabolloid

d. none of these.

24. The diameter of longitudinal bars of a column should never be less than

a. 8 mm

b. 10 mm

c. 12 mm

d. none of these.

25. A wooden block fixed on back side of a door frame on its post, is known as

a. cleat

b. stop

c. horn

d. none of these.

26. The maximum permissible differential settlement, in case of foundations in clayey soil, is usually limited to

a. 20 mm

b. 30 mm

c. 40 mm

d. 50 mm.

27. Pick up the correct statement from the following:

a. The free water is the amount of water added while mixing and the amount of water held on the surface of the aggregates prior to mixing

b. The total water is the free water and the amount actually absorbed by the aggregates

c. Neither (a) nor (b)

d. Both (a) and (b).

28. Addition of pozzolana to cement causes

a. loss of heat of hydration

b. reduction in bleeding

c. increase in curing time

d. all the above.

29. Suction lift of a pump depends upon

a. water temperature

b. velocity of water in suction pipe

c. frictional losses

d. all the above.

30. If Q is discharge is cubicmetres per sec and D is the economical diameter of the pipe. According to Lea

a. D = 0.67 to 0.87 Q

b. D = 0.77 to 0.97 Q

c. D = 0.97 to 1.22 Q

d. D = 1.22 to 1.33 Q.

31. Maximum permissible velocity 1.5 m/sec, is adopted in drains

a. with beds of rocks and gravels

b. lined with stones

c. both (a) and (b)

d. neither (a) nor (b).

32. Hydraulic mean radius is

a. mean radius of sewer

b. difference in heads between two points in circular pipes

c. mean of radii in a pipe line of varying cross-sections

d. cross-sectional area/wetted perimeter.

33. The most suitable location of a canal head work, is

a. boulders stage of the river

b. delta stage of the river

c. rock stage of the river

d. trough stage of the river.

34. When a canal flowing under pressure is carried below a natural drainage such that its F.S.L. does not touch the underside of the supporting structure, the structure so provided, is called

a. syphon

b. aqueduct

c. super passage

d. syphon-aqueduct.

35. The length and width of a meander and also the width of the river, vary roughly as

a. square root of the discharge

b. discharge

c. square of the discharge

d. cube of the discharge.

36. In a canal syphon, the flow is

a. under atmospheric pressure

b. pipe flow

c. with critical velocity

d. under negative pressure.

37. The minimum design speed of various types of highways in plain terrain is the same as the ruling design speed of

a. rolling terrain

b. mountainous terrain

c. steep terrain

d. none of these.

38. Reconnaissance is best done with the help of

a. aerial photographic survey

b. condastral surveys

c. topographical surveys

d. triangulation surveys

39. Minimum thickness of a layer of fine sand required to cut off capillary rise of water completely, should be

a. 52 cm

b. 64 cm

c. 76 cm

d. 80 cm

40. Pick up the correct statement from the following:

a. Curve length should be at least 150 metres for a deflection angle of 5 degress

b. For every degree decrease in the deflection angle, 30 metre length of curve to be increased

c. If the deflection angle is less than 1°, no curve is designed

d. All the above.

41. The detention period in a septic tank is assumed

a. 20 minutes

b. 25 minutes

c. 30 minutes

d. 40 minutes

42. Pick up the correct statement regarding the centre line method of estimating a building

a. Product of the centre line of the walls and area of cross-section of any item, gives total quantity of the item

b. The centre line is worked out separately for different sections of walls of a building

c. The centre line length is reduced by half the layer of main wall joining the partition wall

d. All the above.

43. The expected out turn for earth work in excavation in ordinary soil per mazdoor per day is

a. 1.00 cubic meter

b. 2.00 cubic meter

c. 3.00 cubic meter

d. 4.00 cubic meter

44. As per Indian Standard Specifications, the peak discharge for domestic purposes per capita per minute, is taken

a. 1.80 litres for 5 to 10 users

b. 1.20 litres for 15 users

c. 1.35 for 20 users

d. All the above.

45. An Executive Engineer may have powers upto

a. Rs. 25,000

b. Rs. 50,000

c. Rs. 100,000

d. Rs. 200,000

46. The artificial activity which indicates that an activity following it, cannot be started unless the preceding activity is complete, is known as

a. event

b. free float

c. dummy

d. constant

47. Pre-tender stage requires

a. selection of site

b. formalisation of alignment of work

c. formalisation of designs and preparation of estimate

d. all the above.

48. A dummy activity

a. is artificially introduced

b. is represented by a dotted line

c. does not consume time

d. all the above.

49. The reduced level of the proposed site of an air port is 2500 m above M.S.L. If the recommended length by I.C.A.O. for the runway at sea level is 2500 m, the required length of the runway is

a. 2500 m

b. 3725 m

c. 3000 m

d. 3250 m

50. Pick up the correct statement from the following:

a. Minimum turning radius of aircrafts decides the size of the apron and the radius of the curves at taxi-ends

b. Take off and landing distances for an aircraft, determine the minimum runway length

c. The length of the normal haul of the air craft decides the frequency of operation

d. All the above.