***Sub Engineer***

1. The true meridian of a place is the line in which earth's surface is intersected by a plane through

a. east and west points

b. zenith and nadir points

c. north and south geographical poles

d. north and south magnetic poles.

2. The bearing of line AB is 152° 30' and angle ABC measured clockwise is 124° 28'. The bearing of BC is

a. 27° 52'

b. 96° 58'

c. 148° 08'

d. 186° 58'

3. A theodolite is said to be in perfect adjustment if

a. rotation axis is vertical to the transit axis

b. transit axis is perpendicular to line of collimation

c. line of collimation sweeps out a vertical plane while the telescope is elevated or depressed

d. all the above.

4. Which one of the following mistakes/errors may be cumulative + or - :

a. bad ranging

b. bad straightening

c. erroneous length of chain

d. sag.

5. In a mortar, the binding material is

a. cement

b. sand

c. surkhi

d. cinder.

6. When a brick is immersed in water for 24 hours and then dried, if

a. no grey or white deposits appear on the surface, the brick is free from soluble salts

b. 10 per cent surface is covered with grey or white deposits, the brick has slight efflorescence

c. 50 per cent surface is covered with grey or white deposits, the brick has serious efflorescence

d. All the above.

7. Strength of cement concrete primarily depends upon

a. quality of water

b. quantity of aggregate

c. quantity of cement

d. water-cement ratio.

8. Pick up the synthetic resin from the following:

a. Phenolic resin

b. Resorcinol resin

c. Urea resin

d. All of these.

9. For calculating the allowable stress of long columns  is the empirical formula, known as

a. Straight line formula

b. Parabolic formula

c. Perry's formula

d. Rankine's formula.

10. In the truss shown in the given figure, the force in member BC is

a. 100 t compressive

b. 100 t tensile

c. zero

d. indeterminate

11. Beams of uniform strength are preferred to those of uniform section because these are economical for

a. large spans

b. heavy weights

c. light weights

d. short spans.

12. When a rectangular beam is loaded longitudinally, shear develops on

a. bottom fibre

b. top fibre

c. middle fibre

d. every-horizontal plane.

13. The width of a weir with end contraction, is

a. equal to the width of the channel

b. less than the width of the channel

c. half the width of the channel

d. none of these.

14. Hydraulic ram is a device

a. for lifting water without an electric motor

b. for accelerating water flow

c. for lifting heavy loads

d. none of these.

15. With a clinging nappe of a weir, the excess discharge, is

a. 8% to 10%

b. 18% to 20%

c. 25% to 30%

d. 30% to 40%.

16. The acceleration f required to accelerate a rectangular tank containing water horizontally so that the slope of its free surface is 45°, is

a. g

b. 2 g

c. 2.5 g

d. 3 g.

17. Pick up the correct statement from the following:

a. A soil having pH value more than 7 is an acidic soil

b. A soil having pH value less than 7 is an acidic soil

c. A soil having pH value more than 7 is an alkaline soil

d. A soil containing chemicals for the manufacture of portland cement is preferred.

18. On wetting, cohesive soils,

a. loose permeability

b. gain shear strength

c. loose elasticity

d. decrease their shear strength.

19. A soil sample has passing 0.075 mm sieve = 60% liquid limit = 65% and plastic limit = 40%. The group index of the soil, is

a. 5

b. 20

c. 40

d. none of these.

20. The water content in a soil sample when it continues to loose weight without loosing the volume, is called

a. Shrinkage limit

b. Plastic limit

c. liquid limit

d. semi-solid limit.

21. A simply supported beam , 6 m long and of effective depth 50 cm, carries a uniformly distributed load 2400 kg/m including its self weight. If the lever arm factor is 0.85 and permissible tensile stress of steel is 1400 kg/cm2, the area of steel required, is

a. 15 cm2

b. 16 cm2

c. 17 cm2

d. 18 cm2

22. If Ac, Asc and A are areas of concrete, longitudinal steel and section of a R.C.C. column and m and σc are the modular ratio and maximum stress in the configuration of concrete, the strength of column is

a. σcAc + m σcAsc

b. σc(A - Asc) + m σcAsc

c. σc[A + (m - 1)ASC]

d. all the above.

23. Steel bars are generally connected together to get greater length than the standard length by providing

a. strainght bar splice

b. hooked splice

c. dowel splice

d. all the above

24. The minimum cube strength of concrete used for a prestressed member, is

a. 150 kg/cm2

b. 250 kg/cm2

c. 350 kg/cm2

d. 400 kg/cm2

25. The thickness of a reinforced brick partition wall, is generally kept

a. 10 cm

b. 15 cm

c. 20 cm

d. 25 cm.

26. The taper of precast concrete pile should not be more than

a. 1 cm per metre length

b. 2 cm per metre length

c. 4 cm per metre length

d. 5 cm per metre length.

27. Higher workability of concrete is required if the structure is

a. made with cement concrete

b. thick and reinfored

c. thin and heavily reinforced

d. thick and heavily reinfored.

28. For quality control of Portland cement, the test essentially done is

a. soundness

b. tensile strength

c. consistency

d. all the above.

29. Corrosion of a pipe

a. reduces its carrying capacity

b. adds colour to water

c. adds odour to water

d. all the above.

30. The maximum hourly consumption, is generally taken as

a. 120%

b. 130%

c. 140%

d. 150%.

31. The coagulant which is generally not used for treating the sewage, is

a. alum

b. ferric chloride

c. ferric sulphate

d. chlorinated copperas.

32. D is the diameter of a circular sewer and α is the side of a square section sewer. If both are hydraulically equivalent, the relationship which holds good, is

a. πD8/3 = 4 b8/3

b. πD3/8 = 4 b3/8

c. πD2/3 = 4 b2/3

d. πD3/2 = 4 b3/2

33. The level of the fioor of a syphon aqueduct can be obtained

a. by subtracting the depth of the culvert from the canal bed level

b. by subtracting the thickness of culvert plus the depth of the culvert from the canal bed level

c. both (a) and (b)

d. none of these.

34. The uplift pressure on the roof of an inverted syphon, is maximum when

a. drain in running dry

b. canal is running dry

c. canal is running with F.S.L.

d. drain is running with H.F.L.

35. If water table is comparatively high, the irrigation canal becomes useless, due to

a. water logging of the cultivated areas

b. uncertain water demand

c. spread of malaria

d. all the above.

36. Pick up the correct statement from the following

a. The capacity of escapes should not be less than the capacity of the canal at its location

b. The escapes must lead the surplus water to natural drainages

c. The escapes are aligned to take advantage of contours of lower values

d. All the above.

37. To prevent compressive stresses in a rigid concrete pavement, the joint prvoided, is

a. expansion joint

b. contraction joint

c. hinged joint

d. all the above.

38. If the coefficient of friction on the road surface is 0.15 and a maximum super-elevation 1 in 15 is provided, the maximum speed of the vehicles on a curve of 100 metre radius, is

a. 42.44 kg/hour

b. 52.44 km/hour

c. 62.44 km/hour

d. 72.44 km/hour

39. In a right angle bend of a road provided with a transition throughout, the maximum polar angle will be

a. 15°

b. 20°

c. 30°

d. 45°

40. According to Indian Road Congress, the width of carriageway, is

a. 7.0 m for two lanes without raised kerbs

b. 7.5 m for two lanes with raised kerbs

c. 10.5 m for three lanes pavements

d. All the above

41. Size, capacity and materials need be specified for

a. Stop-cocks

b. Ferrules

c. Ball valves

d. All the above.

42. According to ISI method of measurement, the order of the sequence is

a. Length, breadth, height

b. Breadth, length, height

c. Height, length, breadth

d. None of these.

43. The 'centre line method' is specially adopted for estimating

a. Hexagonal buildings

b. Octagonal buildings

c. Other geometrical shaped buildings

d. All the above.

44. The cross-sections for a highway is taken at

a. 30 metres apart

b. intermediate points having abrupt change in gradient

c. the starting end points of the curves

d. All the above.

45. The most popular type of organisation used for Civil Engineering Constructions, is

a. line organisation

b. line and staff organisation

c. functional organisation

d. effective organisation.

46. PERT is

a. limited of event oriented diagrams

b. used for research and development projects

c. based on three time estimates for activities linking up two events

d. all the above.

47. Pick up the incorrect statement from the following:

a. The difference between the maximum time available for the job and actual time it consumes, is called total float

b. The difference between the latest start time and earliest start time of an activity, is called total float

c. The difference between the earliest finish time of an activity and the earliest start time of its successor activity, is called free float of the activity

d. None of these.

48. While filling the tender for any work, the contractor considers

a. availability of construction materials

b. availability of labour

c. study of specifications

d. all the above.

49. The thickness design of the pavement, is decided on the load carried by

a. main gears

b. nose wheel

c. tail wheel

d. all the above.

50. The fuse large includes

a. passengers chamber

b. pilot's cabin

c. tail of aircraft

d. all the above.