***Sub Engineer***

1. Diurnal variation of magnetic declination is

a. greater at equator than nearer the poles

b. less at equator than nearer the poles

c. less in summer than in winter

d. same at all latitudes and during different months.

2. The properties of autogenous curve for automobiles are given by

a. true spiral

b. cubic parabola

c. Bernoulli's Lemniscate

d. clothoid spiral.

3. The smaller horizontal angle between the true meridian and a survey line, is known

a. declination

b. bearing

c. azimuth

d. dip.

4. The magnetic bearing of a line is 32° and the magnetic declination is 10° 15' W. The true bearing is

a. 21° 45'

b. 42° 15'

c. 42° 15' W

d. 21° 45' W

5. The property by virtue of which lime sets under water, is known as

a. slacking

b. setting

c. hydraulicity

d. calcining.

6. The stones obtained by blasting are used as

a. ballast in railways

b. aggregates for concrete

c. roadmetal

d. All the above.

7. Shingle is

a. decomposed laterite

b. crushed granite

c. water bound pebbles

d. air weathered rock.

8. Pick up the rock which is not  a sedimentary rock from the following:

a. sand stone

b. gypsum

c. dolerite

d. lignite.

9. Pick up the correct statement from the following:

a. If tensile stress is less than axial stress, the section experiences compressive stress

b. If tensile stress is equal to axial stress, the section experiences compressive stress

c. If tensile stress is more than axial stress, some portion of the section experiences a tensile stress

d. All the above.

10. Pick up the incorrect statement from the following : The torsional resistance of a shaft is directly proportional to

a. modulus of rigidity

b. angle of twist

c. reciprocal of the length of the shaft

d. moment of inertia of the shaft section.

11. Hooke's law states that stress and strain are

a. directly proportional

b. inversely proportional

c. curvilinearly related

d. none of these.

12. In a square beam loaded longitudinally, shear develops

a. on lower fibre along horizontal plane

b. on top fibre along vertical plane

c. equally on each fibre along horizontal plane

d. none of these.

13. 'Flow net' can be drawn only if the flow is

a. turbulent

b. rotational

c. distortion

d. none of these.

14. In pipe lines, a surge tank is provided

a. to relieve the pressure due to water hammer

b. to provide additional water head

c. to overflow the pipe line when suddenly closed

d. to remove the frictional loss in pipe.

15. In an inclined position, a venturimeter records

a. more reading

b. less reading

c. same reading

d. none of these.

16. Molecules of fluids get attracted due to

a. surface tension

b. adhesion

c. cohesion

d. none of these.

17. If the cohesive force, (c), is 1.5 t/m2, the density (γ) of the soil is 2.0 t/m3, factor of safety (F) is 1.5 and stability factor (Sn) is 0.05, the safe height of the §lope, is

a. 5 metres

b. 8 metres

c. 10 metres

d. 12 metres

18. Coulomb's wedge theory assumes that

a. slip surface is the plane which passes through the heel of the wall

b. sliding wedge itself acts as a rigid body and the value of earth pressure is obtained by considering the limiting equilibrium of the wedge

c. position and direction of the resultant earth pressure, are known

d. all the above.

19. A soil has bulk density 2.30 g/cm3 and water content 15 per cent, the dry density of the sample, is

a. 1.0 g/cm2

b. 1.5 g/cm3

c. 2.0 g/cm3

d. 2.5 g/cm3

20. The maximum load carried by a pile, when it continues to sink without further increase of load, is known as

a. ultimate load carrying capacity

b. ultimate bearing capacity

c. ultimate bearing resistant

d. all the above.

21. A flat slab is supported

a. on columns

b. on beams and columns

c. on columns monolithicaily built with slab

d. all the above

22. By over-reinforcing a beam, the moment of resistance can be increased not more than

a. 10%

b. 15%

c. 20%

d. 25%

23. The thickness of base slab of a retaining wall generally provided, is

a. one-third of the width of the stem at the bottom

b. one fourth of the width of the steam at the bottom

c. width of the stem at the bottom

d. twice the width of the steam at the bottom.

24. In the zone of R.C.C. beam where shear stress is less than 5 kg/cm2, nominal reinforcement is provided at a pitch of

a. one-half lever arm of the section

b. one-third lever arm of the section

c. lever arm of the section

d. one and half lever arm of the section.

25. The size of a floor tile commonly used, is

a. 15 cm x 15 cm x 1.8 cm

b. 20 cm x 20 cm x 2 cm

c. 22.5 x 22.5 cm x 2.2 cm

d. all the above.

26. For each storey of a building, the depth of exploration should be

a. 1 metre

b. 2 metres

c. 3 metres

d. 4 metres.

27. For road pavements, the cement generally used, is

a. rapid hardening cement

b. low heat cement

c. blast furnace slag cement

d. none of these.

28. Placing of concrete should preferably be done at a temperature of

a. 10°C

b. 20°C

c. 23°C

d. 27 ± 2°C

29. The maximum hourly consumption, is generally taken as

a. 120%

b. 130%

c. 140%

d. 150%.

30. Alum is a

a. coagulant

b. flocculant

c. catalyst

d. disinfectant.

31. If the flame of a miner's safety lamp in a manhole extinguishes within 5 minutes, the sewer certainly contains

a. Carbon dioxide

b. Methane

c. Oxygen

d. None of these.

32. If the diameter of a sewer is 150 mm, the gradient required for generating self cleansing velocity, is

a. 1 in 60

b. 1 in 100

c. 1 in 120

d. none of these.

33. Finally formed berms in canals are provided for

a. protection of banks erosion by the waves

b. control of seepage losses

c. strengthening of banks

d. all the above.

34. The saturation line is the line up to which banks get saturated after the canal runs for some time. The saturation gradient in ordinary loam soil, is generally

a. 3 : 1

b. 4 : 1

c. 5 : 1

d. 6 : 1

35. Cross regulators in main canals are provided

a. to regulate water supply in the distributaries

b. to increase water head upstream when a main canal is running with low supplies

c. to overflow excessive flow water

d. none of these.

36. If the straight sides of a triangular section of a lined canal with circular bottom of radius D, make an angle θ with horizontal, the hydraulic mean depth is

a. D/2

b. D/3

c. D/4

d. D/5

37. If the radius of a main curve is 300 m and length of the transition curve is 100 m, the angle with tangent to locate the junction point, is

a. 1° 11'

b. 2° 11'

c. 3° 11'

d. 4° 11'

38. According to IRC : 52-1973, for a single lane National Highway in hilly region,

a. width of the carriageway must be 3.75 m

b. shoulders on either side must be 1.25 m

c. total width of the road-way must be 6.25 m

d. total of the above.

39. Parapet walls along hill roads, are provided

a. to retain the back filling

b. to prevent the hill from sliding

c. to prevent the wheels of the vehicle from coming on the retaining wall

d. none of these.

40. If the width of carriage way is 12.5 metres, outer edge 50 cm higher than the inner edge, the required super elevation is

a. 1 in 25

b. 1 in 400

c. 1 in 40

d. 1 in 24

41. The correct prismoidal formula for volume is

a. D [first area + last area + ∑ Even area + 2 ∑ odd areas]

b.  [first area + last area + 4 ∑ Even area + 2 ∑ odd areas]

c.  [first area + last area + 2 ∑ Even area + 4 ∑ odd areas]

d.  [first area + last area + 2 ∑ Even area + 4 ∑ odd areas].

42. The slope of the oultet of 'P trap' below the horizontal is kept

a. 8°

b. 10°

c. 12°

d. 14°

43. The value of 'B' of Indian type W.C. shown in the given figure is :

a. 45 cm

b. 50 cm

c. 30 cm

d. 25 cm

44. Pick up the correct statement from the following:

a. In a gully trap, a water seal of 6 to 7.5 cm is provided

b. The gully trap collects waste water from the kitchen, sink, wash basins, etc.

c. The gully trap disconnects the sullage drain from the main drainage system

d. The grating provided over gully traps is 23 cm square.

45. Frederick W. Taylor introduced a system of working known as

a. line organisation

b. line and staff organisation

c. functional organisation

d. effective organisation.

46. Frequency distribution curves

a. having a single lump, are called uninodal curves

b. if symmetrical, are called normal curves

c. if not symmetrical, are called skew curves

d. all the above.

47. Critical Path Net Work helps an engineer

a. to concentrate his attention on critical activities

b. to divert the resources from non-critical advanced activities to critical activities

c. to be cautious for avoiding any delay in the critical activities to avoid delay of the whole project

d. all the above.

48. The Overall in-charge of an organisation at the site responsible for the execution of the works, is

a. Engineer

b. Junior Engineer

c. Sub overseer

d. Assistant Engineer.

49. Effective length of a runway is the distance between

a. ends of the runway

b. point of intersection of the obstruction clearance line and the extended plane of the runway surface, and the other end of the runway

c. point of intersection of the glide path and the extended plane of the runway surface and the other end of the runway

d. ends of the clear way on either side.

50. According to the International Civil Aviation Organisation (I.C.A.O.), the runway lengths of aerodromes, have been coded by

a. Seven English alphabets

b. Last Seven English alphabets

c. First Seven English alphabets

d. First seven natural numbers.