***Sub Engineer***

1. The main principle of surveying is to work

a. from part to the whole

b. from whole to the part

c. from higher level to the lower level

d. from lower level to higher level.

2. If a linear traverse follows a sharp curve round a large lake where it is difficult to have long legs, the accuracy of the traverse may be improved by

a. taking short legs

b. making repeated observations of angular and linear measurements

c. making a subsidiary traverse to determine the length of a long leg

d. all the above.

3. Planimeter is used for measuring

a. area

b. contour gradient

c. slope angle

d. none of these

4. The area of any irregular figure of the plotted map is measured with

a. sextant

b. clinometer

c. planimeter

d. optical square

5. A stone is rejected if it absorbs water more than

a. 10%

b. 15%

c. 20%

d. 25%.

6. Plastic

a. consists of natural or synthetic binders

b. finished products are rigid and stable at normal temperature

c. is capable of flow when necessary heat and pressure are applied

d. All the above.

7. Cast iron is used for

a. structural works in beams

b. small sized water pipes

c. columns and struts

d. none to these.

8. Pick up the correct statement from the following:

a. Blistering may be cured by applying water paint finished with oil paint dried with a little copal varnish

b. Cracked paints may be cured by removing paint and giving a fresh coat of paint

c. Crawling paints may be cured by sand preparing the surface and giving a fresh coat with plenty of turps

d. All the above.

9. A compound bar consists of two bars of equal length. Steel bar cross-section is 3500 mm2 and that of brass bar is 3000 mm2. These are subjected to a compressive load 100,000 N. If Eb = 0.2 MN/mm2 and Eb = 0.1 MN/mm2, the stresses developed are:

a. σb = 10 N/mm2, σs = 20 N/mm2

b. σb = 8 N/mm2, σs = 16 N/mm2

c. σb = 6 N/mm2, σs = 12 N/mm2

d. σb = 5 N/mm2, σs = 10 N/mm2

10. The ratio of tangential and normal components of a stress on an inclined plane through θ° to the direction of the force, is :

a. cos θ

b. tan θ

c. cos θ

d. sec θ

11. The slenderness ratio of a vertical column of square cross- section of 10 cm side and 500 cm long, is

a. 17.3

b. 173.2

c. 137.2

d. 13.72

12. The shear stress at any section of a shaft is maximum

a. at a distance r/2 from the centre

b. at the top of the surface

c. at a distance 3/4 r from the centre

d. none of these.

13. An ideal flow of a liquid obeys

a. Continuity equation

b. Newton's law of viscosity

c. Newton's second law of motion

d. dynamic viscosity law,

14. An error of 1% in measuring the head of water over the crest of a rectangular weir, produces an error in the discharge which is equal to

a. 1.25%

b. 1.5%

c. 1.75%

d. 2.25%

15. If velocities of fluid particles vary from point to point in magnitude and direction, as well as from instant to instant, the flow is said to be

a. laminar

b. turbulent flow

c. uniform flow

d. non-uniform flow.

16. Though angle of deviation of liquid is more in internal mouth piece, the contraction of the jet, is

a. more in the internal mouth piece

b. less in the internal mouth piece

c. equal to external mouth piece

d. none of these.

17. The shear strength in plastic undrained clay, is due to

a. inter-granular friction

b. internal friction

c. cohesion

d. none of these.

18. The property of a soil which permits water to percolate through it, is called

a. moisture content

b. permeability

c. capillarity

d. none of these.

19. A soil mass is said to be in plastic equilibrium if

a. it is stressed to maximum

b. it is on the verge of failure

c. it is in plastic stage

d. it starts flowing.

20. The zero atmospheric pressure is at

a. mean sea level

b. water table

c. phreatic surface

d. both (c) and (d) of the above.

21. If the length of an intermediate span of a continuous slab is 5m, the length of the end span is kept

a. 4.5 m

b. 4.0 m

c. 3.5 m

d. 3.0 m

22. For initial estimate for a beam design, the width is assumed

a. 1/20th of span

b. 1/25th of span

c. 1/30th of span

d. 1/40th of span.

23. To ensure that the hogging bending moment at two points of suspension of a pile of length L equals the sagging moment at its centre, the distances of the points of suspension from either end, is

a. 0.107 L

b. 0.207 L

c. 0.307 L

d. 0.407 L

24. The reinforced concrete beam which has width 25 cm, lever arm 40 cm, shear force 6t/cm2, safe shear stress 5 kg/cm2 and B.M. 24 mt,

a. is safe in shear

b. is unsafe in shear

c. is over safe in shear

d. needs redesigning.

25. Which one of the following rocks is used for monumental buildings :

a. granite

b. marble

c. sand stone

d. slate.

26. The type of bond in which every course contains both headers and stretchers, is called

a. English bond

b. Flemish bond

c. Russian band

d. Mixed bond.

27. For batching 1:3:6 concrete mix by volume, the ingredients required per bag of 50 kg cement, are:

a. 70 kg of sand and 140 litres of aggregates

b. 105 litres of sand and 140 litres of aggregates

c. 105 litres of sand and 210 litres of aggregates

d. none of these.

28. Saw dust can be rendered chemically inert by boiling it in water containing

a. ferrous sulphate

b. potassium chloride

c. ammonia

d. nitric acid

29. The average domestic consumption under normal conditions in an Indian city per day per person, is

a. 115 litres

b. 125 litres

c. 135 litres

d. 150 litres.

30. Water supply system includes

a. digging a well for water

b. construction of dams

c. construction of canals

d. entire arrangement from source to distribution.

31. The minimum diameter of a sewer is kept

a. 15 cm

b. 20 cm

c. 25 cm

d. 30 cm.

32. Lead acetate test in sewer manhole is done to test the presence of

a. Methane gas

b. Hydrogen sulphide

c. Carbondioxide gas

d. Diesel vapours.

33. Effective precipitation for a crop may be defined as

a. total precipitation minus the loss due to evaporation

b. total precipitation minus the loss due to infiltration

c. total precipitation during the crop period

d. available water stored in soil within root zone of the crop.

34. The optimum depth of kor watering for a rice crop, is

a. 19.0 cm

b. 17.5 cm

c. 13.5 cm

d. 12.0 cm

35. According to Lacey, in regime conditions

a. silt is kept in suspension by vertical components of eddies

b. entire cross-section of the channel is generated at all points by the forces normal to the wetted perimeter

c. both (a) and (b)

d. neither (a) nor (b).

36. Cross regulators in main canals are provided

a. to regulate water supply in the distributaries

b. to increase water head upstream when a main canal is running with low supplies

c. to overflow excessive flow water

d. none of these.

37. Parapet walls along hill roads, are provided

a. to retain the back filling

b. to prevent the hill from sliding

c. to prevent the wheels of the vehicle from coming on the retaining wall

d. none of these.

38. The shape of a vertical curve, is

a. parabolic

b. elliptical

c. circular

d. spiral

39. The desirable camber for straight roads with thin bituminous surfacing, is

a. 1 in 40 to 1 in 33

b. 1 in 150 to 1 in 140

c. 1 in 160 to 1 in 140

d. none of these.

40. Longitudinal pavement lines marked broken in white paint

a. are for the guidance of the drivers

b. are not to be crossed over

c. may be crossed over at the discretion of the driver

d. all the above.

41. The value of 'C' of Indian type W.C. shown in the given figure is :

a. 400 mm

b. 450 mm

c. 500 mm

d. 550 mm

42. For 12 mm thick cement plastering 1 : 6 on 100 sq.m new brick work, the quantity of cement required, is

a. 0.200 m3

b. 0.247 m3

c. 0.274 m3

d. 0.295 m3

43. The brick work is measured in sq metre, in case of

a. Honey comb brick work

b. Brick flat soling

c. Half brick walls or the partition

d. All the above.

44. The height of the sink of wash basin above floor level is kept

a. 60 cm

b. 70 cm

c. 75 cm to 80 cm

d. 80 cm

45. Time and progress chart of a construction, is also known as

a. Gantt chart

b. Modified Mile stone chart

c. Critical path method chart

d. All the above.

46. A dummy activity

a. is artificially introduced

b. is represented by a dotted line

c. does not consume time

d. all the above.

47. Modular co-ordination of construction means proper

a. planning

b. designing

c. execution

d. all the above.

48. The main principle of an organisation, is

a. cohegency

b. effective control at all levels

c. delegation of authority

d. all the above.

49. Pick up the correct statement from the following:

a. Landing speed is directly proportional to the wing loading

b. Wing loading remaining constant, the take off distance is directly proportional to the powder loading

c. Neither (a) nor (b)

d. Both (a) and (b)

50. Pick up the correct statement from the following:

a. Localizer indicates to the pilot his position with respect to the proposed alignment

b. The glide slope indicates the correct angle of descent

c. L.O.M. and L.M.M. help the pilot to judge his position for the runway

d. All the above.