***Sub Engineer***

1. The ratio of the linear displacement at the end of a line, subtended by an arc of one second to the length of the line, is

a. 1:206 300

b. 1:3440

c. 1:57

d. 1:100.

2. The area of any irregular figure of the plotted map is measured with

a. sextant

b. clinometer

c. planimeter

d. optical square

3. For indirect ranging, number of ranging rods required, is

a. 2

b. 3

c. 4

d. 5

4. Permanent adjustments of a level are

a. 2 in number

b. 3 in number

c. 4 in number

d. 6 in number

5. Asphalt is obtained from

a. petroleum distillation

b. bitumen distillation

c. plastic distillation

d. none of these.

6. For the manufacture of Portland cement, the proportions of raw materials used, are

a. lime 63% ; silica 22% ; other ingredients 15%

b. lime 22% ; silica 63% ; other ingredients 15%

c. silica 40% ; lime 40% ; other ingredients 20%

d. silica 70% ; lime 20% ; other ingredients 10%.

7. A good brick earth should contain :

a. about 50% to 60% of silica

b. not more than 5% of lime

c. about 5 to 6% of oxide of lime

d. All the above.

8. Plastic

a. consists of natural or synthetic binders

b. finished products are rigid and stable at normal temperature

c. is capable of flow when necessary heat and pressure are applied

d. All the above.

9. The degree of indeterminacy of the frame in the given figure, is

a. 1

b. 2

c. 3

d. zero

10. A composite beam is composed of two equal strips one of brass and other of steel. If the temperature is raised

a. brass experiences compressive force

b. composite beam gets subjected to a couple

c. composite beam bends

d. All the above.

11. A three-hinged arch is said to be :

a. statically determinate structure

b. statically indeterminate structure

c. a bent beam

d. none of these.

12. The property of a material by which it can be beaten or rolled into thin plates, is called

a. malleability

b. ductility

c. plasticity

d. elasticity.

13. An orifice is called a large orifice if water head, is

a. twice the diameter of the orifice

b. thrice the diameter of the orifice

c. four times the diameter of the orifice

d. five times the diameter of the orifice.

14. The ratio of inertia force of a flowing fluid and the viscous force of the liquid is called :

a. Renold's number

b. Froude's number

c. Euler's number

d. Weber's number.

15. The magnitude of water hammer in a pipe depends upon

a. length of the pipe line

b. elastic properties of the pipe material

c. elastic properties of the following liquid

d. all the above.

16. The length of hydraulic jump is roughly

a. 2 to 3 times its height

b. 3 to 5 times its height

c. 5 to 7 times its height

d. None of these.

17. The ratio of emax and emin of silty sand, is

a. 5

b. 3.0

c. 3.5

d. 4.0

18. The critical exist gradient of seepage water in soils, is

a. inversely proportional to the specific gravity

b. directly proportional to the specific gravity

c. inversely proportional to the voids ratio

d. none of these.

19. Pick up the correct statement from the following:

a. In hydrometer method, weight Wd per ml of suspension is found directly

b. In pipette analysis, weight Wd per ml of suspension is found indirectly

c. In pipette analysis, weight Wd per ml of suspension is found directly

d. None of these.

20. For determining the ultimate bearing capacity of soil, the recommended size of a square bearing plate to be used in load plate test should be 30 to 75 cm square with a minimum thickness of

a. 10 mm

b. 15 mm

c. 20 mm

d. 25 mm

21. The Young's modulus of elasticity of steel, is

a. 150 KN/mm2

b. 200 KN/mm2

c. 250 KN/mm2

d. 275 KN/mm2

22. A simply supported beam , 6 m long and of effective depth 50 cm, carries a uniformly distributed load 2400 kg/m including its self weight. If the lever arm factor is 0.85 and permissible tensile stress of steel is 1400 kg/cm2, the area of steel required, is

a. 15 cm2

b. 16 cm2

c. 17 cm2

d. 18 cm2

23. Thickened part of a flat slab over its supporting column, is technically known as

a. drop panel

b. capital

c. column head

d. none of these.

24. After prestressing process is completed, a loss of stress is due to

a. elastic shortening of concrete

b. creep of concrete

c. creep of steel

d. all the above.

25. A floor constructed with 3 mm marble chips, is known

a. mosaic floor

b. terrazo floor

c. chips floor

d. marble floor.

26. The type of stone masonry in which stones of same height are laid in layers, is called

a. random rubble masonry

b. course rubble masonry

c. uncoursed rubble masonry

d. ashlar masonry.

27. Workability of concrete is measured by

a. Vicat apparatus test

b. Slump test

c. Minimum void method

d. Talbot Richard test.

28. An aggregate is said to be flaky, if its least dimension is less than

a. 3/4 mean dimension

b. 3/5 mean dimension

c. 5/8 mean dimension

d. none of these.

29. The depth of the water table at a place is 45 m below the general ground level. To lift water from a deep tube well in such a locality, the type of pump to be installed is

a. Centrifugal pump

b. Reciprocating pump

c. Deep well turbine pump

d. None of these.

30. The maximum depth of sedimentation tanks is limited to

a. 3 m

b. 4 m

c. 5 m

d. 6 m.

31. To maintain aerobic biological activity, the moisture content of the compost mass should be about

a. 45%

b. 50%

c. 55%

d. 60%.

32. If n is the rugosity coefficient, r is the hydraulic depth, s is the bed slope of sewer, the velocity of flow in m/sec may be obtained by the formula  evolved by

a. Chezy

b. Manning

c. Bazin

d. Kutter.

33. Pick up the incorrect statement from the following

a. Side walls of a venturi head flume are splayed out from the end of the throat at 1 : 10 for a length of 4.5 m

b. Length of side walls should be such that the width of the flume is made equal to 2/3rd the bed width of the distributary

c. Once the width of the flume becomes 2/3rd of the width of the distributary, the splayed walls are increased to 1 in 3 to get full bed width

d. None of these.

34. The state of the soil when plants fail to extract sufficient water for their requirements, is

a. maximum saturated point

b. permanent wilting point

c. ultimate utilisation point

d. none of these.

35. Borrow pits should preferably be located in

a. field on the left side of the canal

b. field on the right side of the canal

c. fields on both sides of the canal

d. central half width of the section of the canal.

36. Canals constructed for draining off water from water logged areas, are known

a. drains

b. inundation canals

c. valley canals

d. contour canals

37. One-way streets are generally provided in crowded cities as, these

a. reduce delays to vehicles

b. permit higher speed

c. reduce the number of accidents

d. all the above.

38. Pick up the incorrect statement from the following. The super-elevation on roads is

a. directly proportional to width of pavement

b. directly proportional to velocity of vehicles

c. inversely proportional to acceleration due to gravity

d. inversely proportional to the radius of curvature.

39. The full width of land acquired before finalising a highway, alignment is known

a. width of formation

b. right of way

c. carriage way

d. roadway.

40. The desirable camber for straight cement concrete roads, is

a. 1 in 40 to 1 in 33

b. 1 in 150 to 1 in 140

c. 1 in 160 to 1 in 140

d. none of these.

41. The value of 'A' of Indian type W.C. shown in the given figure is :

a. 25 cm

b. 30 cm

c. 40 cm

d. 45 cm

42. Pick up the excavation where measurements are made in square metres for payment.

a. surface dressing up to 15 cm depths

b. Surface excavation up to 30 cm depths

c. Both (b) and (c)

d. Both (a) and (b)

43. The main factor to be considered while preparing a detailed estimate, is

a. Availability of materials

b. Transportation of materials

c. Location of site and local labour charges

d. All the above.

44. The inspection pit or chamber is a manhole provided in a base drainage system

a. at every change of gradient

b. at every 30 m intervals

c. at the point where vertical soil pipe joins the house drain

d. All the above.

45. PERT analysis is based on

a. optimistic time

b. pessimistic time

c. most likely time

d. all the above.

46. The main disadvantage of line organisation, is

a. rigid structure

b. extraordinary delay in communications

c. top level executions over work

d. all the above.

47. Railway projects are treated as

a. light construction

b. heavy construction

c. industrial construction

d. none of these.

48. A Milestone chart

a. shows the interdependencies of various jobs

b. depicts the delay of jobs, if any

c. points outgoing ahead of schedule of jobs, if any

d. none of these.

49. The air is blowing at a speed of 75 knots in the direction of the aircraft flying at a ground speed of 775 knots. The air speed of the aircraft, is

a. 775 knots

b. 75 knots

c. 850 knots

d. 675 knots

50. The lift off distance is the distance along the centre of the runway between the starting point and

a. end of the runway

b. end of stopway

c. point where air craft becomes air borne

d. point where air craft attains a height of 10.7 m