***Sub Engineer***

1. If + 0.8% grade meets - 0.7% grade and the rate of change of grade for 30 m distance is 0.05, the length of the vertical curve will be

a. 700 m

b. 800 m

c. 900 m

d. 1000 m

2. The distances AC and BC are measured from two fixed points A and B whose distance AB is known. The point C is plotted by intersection. This method is generally adopted in

a. chain surveying

b. traverse method of surveys

c. triangulation

d. none of these.

3. The magnetic bearing of a line is 32° and the magnetic declination is 10° 15' W. The true bearing is

a. 21° 45'

b. 42° 15'

c. 42° 15' W

d. 21° 45' W

4. Angles to a given pivot station observed from a number of traverse stations when plotted, the lines to the pivot station intersect at a common point

a. angular measurements are correct and not the linear measurements

b. linear measurements are correct and not the angular measurements

c. angular and linear measurements are correct and not the plotting of traverse

d. angular and linear measurements and also plotting of the traverse are correct.

5. Shingle is

a. decomposed laterite

b. crushed granite

c. water bound pebbles

d. air weathered rock.

6. Depending on the chemical composition and mechanical properties, iron may be classified as

a. cast iron

b. wrought iron

c. steel

d. all the above.

7. The silica is used for preparing

a. silica bricks

b. coke oven

c. lining for glass furnaces

d. all of these.

8. Upto a maximum of 72% of iron, is available in

a. Magnetite

b. Red haemetite

c. Limonite

d. Siderite

9. A load of 1960 N is raised at the end of a steel wire. The minimum diameter of the wire so that stress in the wire does not exceed 100 N/mm2 is :

a. 4.5 mm

b. 5.0 mm

c. 5.5 mm

d. 6.0 mm

10. For beams of uniform strength, if depth is constant,

a. width b ∝ M

b. width b ∝ M

c. width b ∝ 3 M

d. width b ∝

11. A member is balanced at its end by two inclined members carrying equal forces. For equilibrium the angle between the inclined bars must be

a. 45°

b. 60°

c. 90°

d. 120°

12. The tensile force required to cause an elongation of 0.045 mm in a steel rod of 1000 mm length and 12 mm diameter, is (where E = 2 x 106 kg/cm2)

a. 166 kg

b. 102 kg

c. 204 kg

d. 74 kg

13. Flow in pipes is laminar if Reynold number is

a. less than 2100

b. more than 3000

c. between 2100 and 3000

d. none of these.

14. The specific weight of water is 1000 kg/m3

a. at normal pressure of 760 mm

b. at 4°C temperature

c. at mean sea level

d. all the above.

15. If H is the difference of liquid levels on two sides of an orifice (width b, depth d), the discharge through the orifice will be

a. Cdbd22gH

b. Cdb2d2gH

c. Cdbd2gh

d. Cdbd.2g H1/2.

16. Gauge pressure is

a. absolute pressure - atmospheric pressure

b. absolute pressure + atmospheric pressure

c. atmospheric pressure - absolute pressure

d. none of these.

17. Through a point in a loaded soil mass, there exists n typical planes mutually orthogonal on which the stress is wholly normal and no shear stress acts, if n is

a. 1

b. 2

c. 3

d. 4

18. The internal molecular attraction of a soil, the cohesion

a. decreases as the moisture content increases

b. increases as the moisture content decreases

c. is more in well compacted clays

d. depends upon the external applied load.

19. A fundamental equation of void ratio(e), specific gravity (G), water content (ω) and degree of saturation (Sr) is

a. e =

b. ω =

c. G =

d. Sr =

20. Maximum size of clay particles, is :

a. 0.002 mm

b. 0.04 mm

c. 0.06 mm

d. 0.08 mm

21. The minimum cube strength of concrete used for a prestressed member, is

a. 150 kg/cm2

b. 250 kg/cm2

c. 350 kg/cm2

d. 400 kg/cm2

22. An R.C.C. roof slab is designed as a two way slab if

a. it supports live loads in both directions

b. the ratio of spans in two directions is less than 2

c. the slab is continuous over two supports

d. the slab is discontinuous at edges.

23. A foundation is called shallow if its depth, is

a. half of its width

b. three-fourth of its width

c. equal to its width

d. all the above.

24. In a combined footing if shear stress exceeds 5 kg/cm2, the nominal stirrups provided are:

a. 8 legged

b. 10 legged

c. 12 legged

d. none of these.

25. The service area in a building means the area occupied by

a. stairs

b. toilets

c. light and shafts

d. all the above.

26. The depth of excavation of foundations, is generally measured with a

a. ranging rod

b. steel tape

c. levelling staff

d. bonning rod.

27. For preparing a test-specimen, it is necessary

a. to mix coarse aggregates

b. to mix water to the cement, fine aggregates and coarse aggregates

c. to oil inner surface of the mould and the base plate, before placing concrete

d. all the above.

28. On a grading curve, the gap grading is represented by

a. a horizontal line

b. a vertical line

c. N.W. inclined line

d. N.E. inclined line

29. Pick up the correct statement from the following regarding radial flow centrifugal pumps :

a. In involute type of radial flow centrifugal pump, the impeller discharges into a gradually expanding spiral casing

b. In turbine type of radial flow centrifugal pumps, the impeller is surrounded by stationary guide vanes to reduce the velocity of water

c. The efficiency of turbine type of radial flow centrifugal pump, is always higher than that of volute type

d. All the above.

30. Pick up the incorrect statement from the following :

a. Iron salts produce heavy flocs and hence remove more suspended matter

b. Iron salts remove hydrogen sulphides

c. Iron salts can be used over a limited range of pH values

d. Iron salts impart corrosiveness to water.

31. Disposal to sewage in large cities, is done in

a. irrigation

b. dilution

c. oxidation

d. putrifaction.

32. If the diameter of a sewer is 100 mm, the gradient required for generating self cleansing velocity, is

a. 1 in 60

b. 1 in 100

c. 1 in 120

d. none of these.

33. In a canal syphon, the flow is

a. under atmospheric pressure

b. pipe flow

c. with critical velocity

d. under negative pressure.

34. If straight sides of a triangular section of a lined canal with circular bottom of radius R, make an angle θ with horizontal, the area of its cross-section, is

a. R(θ + cot θ)

b. R2(θ + tan θ)

c. R2(θ + cot θ)

d. R(θ + tan θ)

35. A minimum of 90 cm free board is provided if the discharge in the canal is between

a. 30 to 33 cumecs

b. 30 to 60 cumecs

c. Over 60 cumecs

d. Over 100 cumecs

36. In a syphon aqueduct

a. drainage passes over the canal and F.S.L. of the canal is below the bottom of the drainage trough

b. drainage passes over the canal and F.S.L. of the canal is above the bottom of the drainage trough

c. canal passes over the drainage and H.F.L. of the drainage is above the bottom of the canal trough

d. canal passes over the drainage and H.F.L. of the drainage is below the bottom of the canal trough.

37. First operation during the detailed survey of a hill road, is

a. adjustment of alignment along with curves

b. derivation of longitudinal and cross-sections

c. fixation of Bench Marks

d. staking the ground.

38. The traffic carrying capacity of a single lane, depends on

a. level crossings

b. road intersections

c. surface texture

d. all the above.

39. In an ideal transition curve, the radius of curvature

a. is constant

b. at any point is directly proportional to its distance from the point of commencement

c. is inversely proportional to the radius of main curve

d. is directly proportional to the radius of main curve

40. The minimum superelevation in rolling terrain in plains, is limited to

a. 5%

b. 6%

c. 7%

d. 10%

41. A portion of an embankment having a uniform up-gradient 1 in 500 is circular with radius 1000 m of the centre line. It subtends 180° at the centre. If the height of the bank is 1 m at the lower end, and side slopes 2:1, the earth work involved.

a. 26, 000 m3

b. 26, 500 m3

c. 27, 000 m3

d. 27, 500 m3

42. The value of 'B' of Indian type W.C. shown in the given figure is :

a. 45 cm

b. 50 cm

c. 30 cm

d. 25 cm

43. If B is the width of formation, d is the height of the embankment, side slope S : 1, for a highway with no transverse slope, the area of cross-section is

a. B + d + Sd

b. Bd + Sd2

c. B x d - Sd1/2

d. (Bd + Sd2)

44. The volume is measured correct to the nearest

a. 0.01 cum

b. 0.02 cum

c. 0.03 cum

d. 0.04 cum

45. Construction team means

a. an architect

b. an owner

c. a contractor

d. all the above.

46. Pick up the PERT event from the following:

a. Digging of foundation completed

b. Laying of concrete started

c. Laying of concrete completed

d. All the above.

47. For the execution of a project, a contractor is

a. a person

b. a firm

c. an agency

d. all the above.

48. CPM is

a. is built of activities oriented programme

b. is based on time estimate

c. is used for repetitive works

d. all the above.

49. Pick up the incorrect statement from the following:

a. In single engine aeroplanes, the engine is provided in the nose of the aircraft

b. In double engine aeroplanes, one engine on either wing is placed symmetrically

c. In three engine aeroplanes, two engines are placed on both wings and one engine is placed in the tail

d. None of these.

50. Pick up the correct statement from the following:

a. Minimum turning radius of aircrafts decides the size of the apron and the radius of the curves at taxi-ends

b. Take off and landing distances for an aircraft, determine the minimum runway length

c. The length of the normal haul of the air craft decides the frequency of operation

d. All the above.