***Sub Engineer***

1. A uniform slope was measured by the method of stepping. If the difference in level between two points is 1.8 m and the slope distance between them is 15 m, the error is approximately equal to

a. cumulative, + 0.11 m

b. compensating, ± 0.11 m

c. cumulative, - 0.11 m

d. none of these

2. Pick up the incorrect statement from the following :

a. while measuring a distance with a tape of length 100.005 m, the distance to be increasing by 0.005 m for each tape length

b. an increase in temperature causes a tape to increase in length and the measured distance is too large

c. the straight distance between end points of a suspended tape is reduced by an amount called the sag correction

d. a 100 m tape of cross section 10 mm x 0.25 mm stretches about 10 mm under 5 kg pull.

3. Pick up the correct statement from the following :

a. a back sight is the first sight taken after setting up the instrument in any position

b. the first sight on each change point is a fore sight

c. the second sight on each change point is a back sight

d. all the above.

4. In an adjusted level, when the bubble is central, the axis of the bubble tube becomes parallel to

a. line of sight

b. line of collimation

c. axis of the telescope

d. None of these.

5. Acrylic is the name of

a. cellulose resin

b. alkyd resin

c. methyl metha crylate

d. cumarone-indene.

6. For a 50 kg cement bag water required, is

a. 18.5 litres

b. 20.5 litres

c. 22.5 litres

d. 25 litres.

7. Initial setting time of cement for asbestos cement products should be not less than

a. 30 minutes

b. 50 minutes

c. 75 minutes

d. 90 minutes.

8. Sand stone is

a. sedimentary rock

b. metamorphic rock

c. igneous rock

d. volcanic rock.

9. Pick up the correct statement from the following:

a. If tensile stress is less than axial stress, the section experiences compressive stress

b. If tensile stress is equal to axial stress, the section experiences compressive stress

c. If tensile stress is more than axial stress, some portion of the section experiences a tensile stress

d. All the above.

10. A shaft subjected to a bending moment M and a torque T, experiences

a. maximum bending stress =

b. maximum shear stress =

c. both (a) and (b)

d. neither (a) nor (b)

11. A rectangular log of wood is floating in water with a load of 100 N at its centre. The maximum shear force in the wooden log is

a. 50 N at each end

b. 50 N at the centre

c. 100 N at the centre

d. none of these.

12. The bending moment at C of a portal frame shown in below figure is(1st distance 4m)

a. 4 t-m

b. 28 t-m

c. 16 t-m

d. zero.

13. For maximum horse power of a nozzle, the head supplied must be equal to

a. head loss in the pipe due to friction

b. twice the head loss in the pipe due to friction

c. thrice the head loss in the pipe due to friction

d. four times the head loss in the pipe due to friction.

14. Pick up the correct statement from the following :

a. For tranquil flow h > 2g or v <

b. For critical flow

c. For torrential flow h >  or v < gh

d. None of these.

15. The radius of gyration of the water line of a floating ship is 4 m and its metacentric height is 72.5 cm. The period of oscillation of the ship, is

a. 2π

b. 3π

c. 4π

d. π/2

16. Energy equation is usually applicable to

a. non-uniform flow

b. turbulent flow

c. laminar flow

d. steady flow.

17. The weight of a pycnometer containing 400 g sand and water full to the top is 2150 g. The weight of pycnometer full of clean water is 1950 g. If specific gravity of the soil is 2.5, the water content is

a. 5%

b. 10%

c. 15%

d. 20%

18. The ratio of the volume of voids to the volume of soil solids in a given soil mass, is known

a. porosity

b. specific gravity

c. void ratio

d. water content.

19. Pick up the correct statement from the following:

a. The void space between the soil grains, is filled partly with air and partly with water

b. In perfectly saturated soil, the voids are completely filled with water

c. In dry soil, the voids are completely filled with air

d. all the above.

20. A partially saturated sample of soil has a unit weight of 2.0 g/cm3 and specific gravity of soil particles is 2.6. If the moisture content in the soil is 20%, the degree of saturation is

a. 20%

b. 77%

c. 92%

d. none of these.

21. The thickness of the flange of a Tee beam of a ribbed slab is assumed as

a. width of the rib

b. depth of the rib

c. thickness of the concrete topping 0d) half the thickness of the rib

d. twice the width of the rib.

22. A part of the slab may be considered as the flange of the T-beam if

a. flange has adequate reinforcement transverse to beam

b. it is built integrally with the beam

c. it is effectively bonded together with the beam

d. all the above.

23. The load stress of a section can be reduced by

a. increasing the total perimeter of bars

b. replacing larger bars by greater number of small bars

c. replacing smaller bars by greater number of greater bars

d. none of these.

24. According to I.S.: 456, 1978 the thickness of reinforced concrete footing on piles at its edges, is kept less than

a. 30 cm

b. 40 cm

c. 50 cm

d. 75 cm

25. The exterior angle between outer faces of a wall, is known as

a. turn

b. junction

c. quion

d. all the above.

26. Pick up the correct specification of one-room quarters generally adopted from the following :

a. six quarters in a row

b. the size of room is either 3.5 m x 3 m or 4.2 m x 2.5 m

c. the front verandah is kept 2 m wide.

d. all the above.

27. For the construction of the retaining structures, the type of concrete mix to be used, is

a. 1 : 2 :4

b. 1 :  : 3

c. 1 : l : 2

d. 1 : 4 : 8.

28. Pozzolana cement is used with confidence for construction of

a. massive foundations

b. abutments

c. R.C.C. structures

d. all the above.

29. The U.C. (uniformity coefficient)  for the best filter media sand should be

a. 2

b. 3

c. 4

d. 5

30. For a city developed haphazardly, the layout of distribution pipes preferred to, is

a. ring system

b. radial system

c. grid iron system

d. dead end system.

31. In areas where rainy season is limited to a few months, the type of sewerage system recommended is

a. combined system

b. partially separate system

c. separate system

d. none of these.

32. The design period of sewage treatment works in normally

a. 5 - 10 years

b. 15 - 20 years

c. 30 - 40 years

d. 40 - 50 years.

33. When a canal and a drainage approach each other at the same level, the structure so provided, is

a. an aqueduct

b. a syphon

c. a level crossing

d. inlet and outlet.

34. In rigid module, the discharge

a. is independent of water levels in the distributary and water course

b. depends upon the water level in distributary

c. depends upon the water level in the water course

d. depends upon the water levels of both, i.e. distributary and water course

35. In a concrete canal the approximate permissible velocity of water should not exceed

a. 1 m/sec

b. 1.5 m/sec

c. 2 m/sec

d. 2.5 m/sec

36. If A is the area of the surface, x is the depth of its C.G. from the surface of the water and ω is the density of water, then

a. total pressure on the surface is equal to ω x

b. depth of the point at which total pressure acts is equal to its moment of inertia divided by A x

c. depth of the centre of pressure is 2/3H vertically below the surface

d. all the above.

37. Setting out of Lemniscate transition curves, is done with

a. perpendicular offsets

b. radial offsets

c. deflection angles

d. polar deflection angles.

38. Passing zones are generally not provided on

a. summit curves

b. horizontal curves

c. two lane highways

d. all the above.

39. California Bearing Ratio method of designing flexible pavements is more accurate as it involves

a. characteristics of soils

b. traffic intensities

c. character of the road making materials

d. none of these.

40. Reconnaissance is best done with the help of

a. aerial photographic survey

b. condastral surveys

c. topographical surveys

d. triangulation surveys

41. The following item of earth work is not measured separately.

a. Site clearence

b. dead men

c. Steps in deep exavaction

d. All the above.

42. The order of booking dimensions is

a. Length, breadth, height

b. Breadth, length, height

c. Height, breadth, length

d. None of these.

43. The expected out turn of cement concrete 1 : 2 : 4 per mason per day is

a. 1.5 m3

b. 2.5 m3

c. 3.5 m3

d. 5.0 m3

44. Pick up the correct statement from the following:

a. The earth work of cutting in trenches or borrow pits in fairly uniform ground is measured with the help of average depths of the dead men

b. The earth work in trenches or borrow pits in irregular ground is measured by taking the difference in levels before and after completion of work

c. The earth work in trenches or borrow pits, where neither a nor b is feasible, are measured from the fillings after deduction of voids

d. All the above.

45. The main advantage of line organisation, is :

a. defined responsibilities at all levels

b. rigid discipline in the organisation

c. ability of quick decision at all levels

d. all the above.

46. Various activities of a project, are shown on bar charts by

a. vertical lines

b. horizontal lines

c. dots

d. crosses.

47. Military organisation is known as

a. line organisation

b. line and staff organisation

c. functional organisation

d. none of these.

48. If t is the duration of an activity, t1 is the latest finish possible moment of its preceding activity and t2 is the earliest start possible moment, the independent float of the activity is

a. (t1 - t2) - t

b. t - (t1 - t2)

c. (t1 + t2) - t

d. t + (t1 - t2)

49. The height of the pilot's eye above the runway surface is assumed

a. 1 m

b. 3 m

c. 4 m

d. 5 m

50. Beaufort scale is used to determine

a. strength of winds

b. direction of winds

c. height of air-crafts

d. none of these.