***Sub Engineer***

1. A well conditioned triangle has no angle less than

a. 20°

b. 30°

c. 45°

d. 60°.

2. Pick up the correct statement from the following :

a. the principal axes of both the lenses coincide the optical axis of the telescope

b. the lens nearer the object to be viewed is convex and is called objective

c. the lens nearer the eye is called eyepiece

d. all the above.

3. Pick up the correct statement from the following :

a. the eyepiece plays no part in defining the line of sight

b. the diaphragm plays no part in defining the line of sight

c. the optical centre of the objective plays no part in defining the line of sight

d. none of these.

4. The theodolites used for making tacheometric observations by optical wedge system, are

a. provided with stadia hairs in front of eye piece

b. not provided with stadia hairs at all

c. fitted with a glass wedge inside the telescope

d. fitted with a glass wedge in front of telescope.

5. Pick up the correct statement regarding low heat cement from the following:

a. Its initial setting time is about one hour

b. Its final setting time is about 10 hours

c. Its mainly used for mass concrete work

d. All the above.

6. A pug mill is used for

a. moulding brick earth

b. tempering brick earth

c. providing brick earth

d. all the above.

7. Spirit varnish generally consists of

a. alcohol, wax and turpentine

b. pigment and synthetic resin

c. spirit and shellac

d. none of these.

8. Pick up the synthetic resin from the following:

a. Phenolic resin

b. Resorcinol resin

c. Urea resin

d. All of these.

9. The degree of indeterminacy of the frame in the given figure, is

a. zero

b. 1

c. 2

d. 3

10. A masonry dam (density = 20, 000 N/m3) 6 m high, one metre wide at the top and 4 m wide at the base, has vertical water face. The minimum stress at the base of the dam when the reservoir is full, will be

a. 75 N/m2

b. 750 N/m2

c. 7500 N/m2

d. 75000 N/m2

11. The value of Poisson's ratio always remains

a. greater than one

b. less than one

c. equal to one

d. none of these.

12. Along the neutral axis of a simply supported beam

a. fibres do not undergo strain

b. fibres undergo minimum strain

c. fibres undergo maximum strain

d. none of these.

13. A fluid particle may possess the displacement of

a. translation

b. rotation

c. distortion

d. all the above.

14. The pressure rise due to water hammer depends upon

a. the length of pipe

b. time taken to close the valve

c. the elasticity of the pipe material

d. All of above.

15. Flow of water in pipes of diameter more than 3 metres, can be measured by

a. pitot tube

b. venturimeter

c. orifice plate

d. rotameter.

16. Poise is the unit of

a. viscosity

b. velocity gradient

c. mass density

d. kinematic viscous.

17. Cohesionless soil is

a. sand

b. silt

c. clay

d. clay and silt.

18. The change of moisture content of soils, changes the

a. value of the angle of repose

b. amount of compaction required

c. cohesive strength of soil

d. all the above.

19. An unsaturated 100 cm3 sample of soil weighs 190 g. If its dried weight is 160 g, water content of the soil, is

a. 0.188

b. 0.288

c. 0.388

d. 0.488

20. The maximum pressure which a soil can carry without shear failure, is called

a. safe bearing capacity

b. net safe bearing capacity

c. net ultimate bearing capacity

d. ultimate bearing capacity.

21. For M 150 mix concrete, according to I.S. specifications, local bond stress, is

a. 10 kg/cm2

b. 15 kg/cm2

c. 20 kg/cm2

d. 25 kg/cm2

22. Columns may be made of plain concrete if their unsupported lengths do not exceed their least lateral dimension

a. three times

b. four times

c. five times

d. six times.

23. A pre-stressed concrete member

a. is made of concrete

b. is made of reinforced concrete

c. is stressed after casting

d. possesses internal stresses.

24. In a cantilever retaining wall without a heel slab

a. thickness of the stem is kept same throughout

b. base slab is made 10 cm thicker than the stem

c. width of the base slab is kept 0.7 time the total height of the wall

d. all the above.

25. Pick up the incorrect statement from the following :

a. No  timbering is  required  for  shallow trenches

b. Shallow foundations can be constructed on made-up soil

c. Grillage   foundation   is   classified as a shallow foundation

d. Black cotton soil is very good for foundation bed.

26. The depth of the ground water table may be ascertained by

a. looking through the well in the vicinity

b. standing on the well in the vicinity

c. measuring the depth of water in the well

d. none of the above.

27. Expansion joints are provided if the length of concrete structures exceeds

a. 15 m

b. 15 m

c. 35 m

d. 45 m

28. High temperature

a. increases the strength of concrete

b. decreases the strength of concrete

c. has no effect on the strength of concrete

d. none of these.

29. Carbonates in water produce

a. temporary hardness

b. permanent hardness

c. acidity

d. alkanity.

30. Underground water is obtained from

a. rivers

b. lakes

c. reservoirs

d. springs.

31. If the pH value of sewage is 7

a. it is acidic

b. it is alkaline

c. it is neutral

d. none of these.

32. The formula  used for determining flow velocities in sewers, is known as

a. Chezy's formula

b. Bazin's formula

c. Kutter's formula

d. Manning's formula.

33. A deficit of sediments in flowing water may cause a river

a. meandering type

b. aggrading type

c. degrading type

d. sub-critical type.

34. Bed bars in a canal are provided

a. to watch the general behaviour of canal

b. to measure the discharge

c. to raise the supply level

d. to control the silting

35. According to Lacey, depth of scour in a river depends upon the straightness of the reach. If D is the depth of scour in regime flow in a right angled bend, it is

a. 1.25 D

b. 1.50 D

c. 1.75 D

d. 2.00 D

36. Bligh's theory of seepage assumes

a. equal weightage to the horizontal and vertical creep

b. more weightage to horizontal creep than vertical creep

c. less weightage to horizontal creep than vertical creep

d. loss of head follows the sine curve.

37. Volume of traffic which would immediately use a new road or an improved one when opened to traffic, is known

a. development traffic

b. current traffic

c. general traffic

d. normal traffic growth.

38. The basic formula for determination of pavement thickness was first suggested by

a. Spanglar

b. Picket

c. Kelly

d. Goldbeck

39. The first stage of deciding the alignment of a hill road, is

a. reconnaissance

b. detailed survey

c. trace-out

d. preliminary survey.

40. For the administration of road transport, a Motor Vehicle Act was enacted in

a. 1934

b. 1939

c. 1947

d. 1950

41. Brick walls are measured in sq. m if the thickness of the wall is

a. 10 cm

b. 15 cm

c. 20 cm

d. None of these.

42. Pick up the item of work not included in the plinth area estimate

a. Room area

b. Verandah area

c. W.C. area

d. Courtyard area.

43. The slope of the oultet of 'P trap' below the horizontal is kept

a. 8°

b. 10°

c. 12°

d. 14°

44. Pick up the correct statement from the following:

a. In a gully trap, a water seal of 6 to 7.5 cm is provided

b. The gully trap collects waste water from the kitchen, sink, wash basins, etc.

c. The gully trap disconnects the sullage drain from the main drainage system

d. The grating provided over gully traps is 23 cm square.

45. If D is the duration, ES and EF are the earliest start and finish, LS and LF are latest start and latest finish time, then the following relation holds good

a. LS = LF - D

b. LF = LS + D

c. D = EF - ES

d. all the above.

46. Critical path lies along the activities having total float

a. positive

b. negative

c. zero

d. same.

47. For the execution of a project, a contractor is

a. a person

b. a firm

c. an agency

d. all the above.

48. Various activities of a project, are shown on bar charts by

a. vertical lines

b. horizontal lines

c. dots

d. crosses.

49. The depressions and undulations in the pavement, are caused due to

a. impact of heavy wheel loads

b. punching effect

c. flexible gears

d. all the above.

50. Pick up the correct statement from the following:

a. The basic length of a runway is increased at a rate of 7% per 300 m of elevation of M.S.L.

b. The standard temperature at the site is obtained by reducing the standard sea level temperature of 15°C at the rate of 6.5°C per 1000 m rise in elevation

c. The aerodrome reference temperature is the monthly mean of the mean daily temperature for the hottest month of the year

d. All the above.