***Sub Engineer***

1. The additional lines which are measured to show the correctness of the chain surveying are called:

a. check clines

b. proof lines

c. tie lines

d. all of these.

2. The ratio of the distances at which a stated length can be distinguished by the telescope and the human eye, respectively, is called

a. brightness of telescope

b. magnification of telescope

c. resolving power of telescope

d. none of these.

3. The bearings of two traverse legs AB and BC are N52° 45' E and N34° 30' E respectively. The deflection angle is

a. 18° 15' N

b. 18° 15' W

c. 18° 15' R

d. 18° 15' L

4. The tangent to the liquid surface in a level tube, is parallel to the axis of the level tube at

a. every point of the bubble

b. either end of the bubble

c. the mid-point of the bubble

d. no where.

5. Refractory bricks are

a. neutral refractory bricks

b. acid refractory bricks

c. basic refractory bricks

d. all the above.

6. Chlorite, a green colour mineral is mainly derived from the decomposition of

a. augite

b. biotite

c. horn blende

d. All of these.

7. Pick up the correct statement from the following:

a. Cast iron oxidises less

b. Steel oxidises most

c. Wrought iron oxidises moderately

d. All the above.

8. The pigment used in paints for corrosive resistance, is

a. ferrous oxide

b. zinc white

c. red lead

d. gypsum.

9. Pick up the correct statement from the following:

a. The pitch of a close coil spring, is very small

b. The angle made by the coil with horizontal, is called the angle of helix

c. In the open coil helical spring, the angle of helix is comparatively large

d. All the above.

10. The strain energy due to volumetric strain

a. is directly proportional to the volume

b. is directly proportional to the square of exerted pressure

c. is inversely proportional to Bulk modulus

d. all the above.

11. If the stress in each cross-section of a pillar is equal to its working stress, it is called

a. body of equal

b. body of equal section

c. body of equal strength

d. none of these.

12. A beam is said to be of uniform strength, if

a. B.M. is same throughout the beam

b. shear stress is same throughout the beam

c. deflection is same throughout the beam

d. bending stress is same at every section along its longitudinal axis.

13. Dimensions of the dynamic viscosity (μ) are

a. MLT-2

b. M-1L-1T-1

c. ML-1T-2

d. ML-1T-1.

14. Water displaced by a floating wooden block of density 0.75, 5 m long, 2 m wide and 3 m high, is

a. 17.5 m3

b. 20.0 m3

c. 22.5 m3

d. 25 km3.

15. The ratio of the maximum height to which a jet inclined through θ rises to its total horizontal span is

a.  sin θ

b.  cos θ

c.  tan θ

d.  cot θ.

16. The formula Q = m x L x 2g x H3/2 where  was suggested by :

a. Bazin

b. Francis

c. Cipolletti

d. None of these.

17. The weight of a pycnometer containing 400 g sand and water full to the top is 2150 g. The weight of pycnometer full of clean water is 1950 g. If specific gravity of the soil is 2.5, the water content is

a. 5%

b. 10%

c. 15%

d. 20%

18. A direct shear test possesses the following disadvantage:

a. A relatively thin thickness of sample permits quick drainage

b. A relatively thin thickness of sample permits quick dissipation of pore pressure developed during the test

c. As the test progresses the area under shear, gradually changes

d. none of these.

19. The plasticity of fine soils may be assessed by means of

a. dry strength test

b. toughness test

c. dilatancy test

d. all of these.

20. Pick up the incorrect statement from the following:

a. Compaction has no effect on the structure of a soil

b. Permeability decreases with increase in the dry density of a compacted soil

c. A wet side compacted soil is more compressible than a dry side compacted soil

d. Dry side compaction soils swell more when given access to moisture

21. After prestressing process is completed, a loss of stress is due to

a. elastic shortening of concrete

b. creep of concrete

c. creep of steel

d. all the above.

22. Pick up the correct statement from the following:

a. Lateral reinforcement prevents the shearing of concrete on diagonal plane

b. Lateral reinforcement stops breaking away of concrete cover, due to buckling

c. Lateral reinforcement in R.C.C. columns, is kept not less than 5 mm diameter

d. All the above.

23. As per IS : 456, the reinforcement in a column should not be less than

a. 0.6% and not more than 6% of cross-see-tional area

b. 0.7% and not more than 7% of cross-sec-tional area

c. 0.8% and not more than 8% of cross-sectional area

d. none of these.

24. A ribbed slab is provided for

a. a plain ceiling

b. thermal insulation

c. acoustic insulation

d. all the above.

25. The X-ray rooms are plastered with

a. Plaster of Paris

b. Barium plaster

c. Martin's cement

d. Keen's cement.

26. The bearing capacity of piles is determined by

a. dynamic formula

b. static formula

c. pile load tests

d. all the above.

27. Pick up the correct statement from the following:

a. Angular and rough aggregates reduce the workability of the concrete

b. Large size aggregates increase the workability due to lesser surface area

c. The slump of the concrete mix decreases due to an increase in temperature

d. All the above.

28. Pick up the correct statement from the following:

a. High percentage of C3S and low percentage of C2S make the cement less resistive to chemical attack

b. Low percentage of C3S and high percentage of C2S contribute to slow hardening

c. Low percentage of C3S and high percentage of C2S provide greater resistance to chemical attack

d. All the above.

29. If average volume of sediment deposits is one-tenth million cubic metres per year in a reservoir of total capacity 10 million cubic metres, the dead storage will be filled up, in

a. 15 years

b. 20 years

c. 25 years

d. 50 years.

30. The main disadvantage of hard water, is

a. scaling of boilers

b. corrosion and incrustation of pipes

c. making food tasteless

d. all the above.

31. The normal value of over flow rates for plain primary sedimentation tanks, ranges between

a. 40, 000 to 50, 000 litres/sqm/day

b. 50, 000 to 60, 000 litres/sqm/day

c. 80, 000 to 100, 000 litres/sqm/day

d. none of these.

32. The sewage treatment units in which anaerobic decomposition of organic matter is used, are called

a. imhoff tanks

b. trickling filters

c. sludge sedimentation tanks

d. none of these.

33. A hydraulic jump is generally formed when a stream moving with

a. a hyper-critical velocity meets a stream moving with a critical velocity

b. a hyper-critical velocity meets a stream moving with a hyper-critical velocity

c. A hyper-critical velocity meets a stream moving with a sub-critical velocity

d. a sub-critical velocity meets a stream moving with a hyper-critical velocity.

34. If d1 is the depth of cutting, d2 is the height of the bank from bed level r2 : 1 and r1 : 1 are the slopes in filling and cutting respectively, the horizontal distance n between the bed and bank, is

a. x = r1 d1

b. x = r2 d2

c. x = d1 / r1

d. x = d2 / r2

35. Canals taken off from ice-fed perennial rivers, are known

a. ridge canals

b. perennial canals

c. inundation canals

d. ice canals.

36. Pick up the correct statement from the following:

a. The full supply level of a canal should be above ground level

b. According to Lacey, regime conditions require a particular slope for a given discharge and silt factor

c. In case the ground slope is less than the required bed slope, the silt factor must be reduced by permitting the entry of coarse silt

d. All the above.

37. Retaining walls are generally constructed with dry rubble stones with 60 cm top width and

a. 1 : 2 front batter

b. 1 : 3 front batter

c. 1 : 4 front batter

d. 1 : 5 front batter

38. Alignment of highways in hilly regions, is decided on

a. number of river crossings

b. natural unstable areas

c. saddles or passes

d. all the above.

39. Minimum radius of a simple circular curve deflecting through 5°, is

a. 1618.9 m

b. 1816.9 m

c. 1718.9 m

d. 1817.9 m

40. If d is the economic designed depth of a slab, the thickness of the cement concrete pavement slab at interior, is kept

a. 1.125 d

b. 0.85 d

c. 0.75 d

d. 0.60 d

41. The measurement is made in square metre in case of

a. R.C.C. structure

b. Hollow concrete block wall

c. Concrete fencing posts

d. None of these.

42. Cost of fittings and their fixing is specified for the following sanitary fittings

a. Flushing pipes

b. Lavatary basins

c. Sinks

d. All the above.

43. As per Indian Standard Specifications, the peak discharge for domestic purposes per capita per minute, is taken

a. 1.80 litres for 5 to 10 users

b. 1.20 litres for 15 users

c. 1.35 for 20 users

d. All the above.

44. The item of steel work which is measured in sq.m, is

a. Rolling shutters

b. Steel doors

c. Ventilators and glazing.

d. All the above.

45. The difference between the time avail-to do a job and the time required to do the job, is known as

a. event

b. float

c. duration

d. constraint.

46. Pre-tender stage requires

a. selection of site

b. formalisation of alignment of work

c. formalisation of designs and preparation of estimate

d. all the above.

47. The salient feature of functional organisation is

a. separation of planning and design part

b. each individual maintains functional efficiency

c. work is properly planned and distributed

d. all the above.

48. Power stations are generally treated as

a. light construction

b. heavy construction

c. industrial construction

d. electrical construction.

49. Pick up the incorrect statement from the following:

a. Channelization of pavement is caused by constant use of tri-cycle gears of aircraft

b. Blast pads are used over cohesionless soils to resist erosion due to tremendous speed of the jet blast

c. Over-run areas of at least 300 m length on either side of the runway are provided

d. None of these.

50. According to the International Civil Aviation Organisation (I.C.A.O.), the runway lengths of aerodromes, have been coded by

a. Seven English alphabets

b. Last Seven English alphabets

c. First Seven English alphabets

d. First seven natural numbers.