***Sub Engineer***

1. The angle between two plane mirrors of optical square, is

a. 30°

b. 45°

c. 60°

d. 90°.

2. The longitudinal section of the surface of bubble tube is

a. straight

b. circular

c. parabolic

d. elliptic.

3. Magnetic declination at any place

a. remains constant

b. does not remain constant

c. fluctuates

d. changes abruptly.

4. If the whole circle bearing of a line is 180°, its reduced bearing is

a. S 0° E

b. S 0° W

c. S

d. N.

5. Elastomers can extend upto

a. five times their original dimensions

b. seven times their original dimensions

c. ten times their original dimensions

d. three times their original dimensions.

6. Pick up the correct statement from the following:

a. The distinct plane of division along which a stone can easily be split, is called natural bed of stone

b. The natural bed of sedimentary rocks is along the planes of stratification

c. The natural bed of igneous rocks is not defined

d. All the above.

7. Pick up the correct statement from the following:

a. The plastic sheets are made by the calendering process in which the plastic material is allowed to pass between cylindrical rollers

b. The application of thermo-setting resins on sheets of paper, is called laminating process

c. The plastic articles made by placing raw material in the desired moulds, is known as moulding process

d. All the above.

8. If water required for 1 bag of cement is 30 litres, the water cement ratio is :

a. 0.40

b. 0.50

c. 0.60

d. None of these.

9. In plastic analysis, the shape factor for a circular section, is

a. 1.5

b. 1.6

c. 1.7

d. 1.75

10. The equivalent length of a column of length L, having both the ends hinged, is

a. 2L

b. L

c.

d.

11. During a tensile test on a ductile material

a. nominal stress at fracture is higher than the ultimate stress

b. true stress at fracture is higher than the ultimate stress

c. true stress at fracture is the same as the ultimate stress

d. none of these.

12. The deflection of any rectangular beam simply supported, is

a. inversely proportional to its width

b. inversely proportional to the cube of its depth

c. directly proportional to the cube of its length

d. none of these.

13. The ratio of the percentagge error in the discharge and percentage error in the measurement of head over a triangular notch, is

a. 2/3

b. 3/2

c. 2/5

d. 5/2

14. If a pitot tube is placed with its nose facing down stream, the liquid

a. does not rise in the tube

b. rises in the tube to a height

c. falls in the tube to a depth

d. none of these.

15. Hydraulic ram is a device

a. for lifting water without an electric motor

b. for accelerating water flow

c. for lifting heavy loads

d. none of these.

16. If H is the difference of liquid levels on two sides of an orifice (width b, depth d), the discharge through the orifice will be

a. Cdbd22gH

b. Cdb2d2gH

c. Cdbd2gh

d. Cdbd.2g H1/2.

17. The reduction in volume of soil due to squeezing out of water from the voids, is termed

a. primary consolidation

b. primary compression

c. primary time effect

d. all the above.

18. The maximum value of effective stress in the past divided by the present value, is defined as over consolidation ratio (OCR). The O.C.R. of an over consolidated clay is

a. less than 1

b. 1

c. more than 1

d. None of these.

19. The maximum shear stress occurs on the filament which makes an angle with the horizontal plane equal to

a. 30°

b. 45°

c. 60°

d. 90°

20. Pick up the correct statement from the following:

a. When stress decreases, void, ratio decreases

b. When stress decreases, coefficient of permeability decreases

c. When stress decreases, coefficient of volume change decreases

d. When stress decreases void ratio, co-effi-cients of per-meability and volume change decrease.

21. In a singly reinforced beam, the effective depth is measured from its compression edge to

a. tensile edge

b. tensile reinforcement

c. neutral axis of the beam

d. longitudinal central axis.

22. An R.C.C beam of 25 cm width has a clear span of 5 metres and carries a U.D.L. of 2000 kg/m inclusive of its self weight. If the lever arm of the section is 45 cm., the beam is

a. safe in shear

b. is safe with stirrups

c. is safe with stirrups and inclined members

d. needs revision of the section.

23. Design of R.C.C. simply supported beams carrying U.D.L. is based on the resultant B.M. at

a. supports

b. mid span

c. every section

d. quarter span.

24. The effective span of a simply supported slab, is

a. distance between the centres of the bearings

b. clear distance between the inner faces of the walls plus twice the thickness of the wall

c. clear span plus effective depth of the slab

d. none of these.

25. The compaction of concrete in the drilled pile hole is done by compressed air in the case of

a. simplex pile

b. Franki pile

c. pressure pile

d. vibro pile.

26. A cut in frame of a door to receive the shutter, is called

a. louver

b. stop

c. horn

d. rebate.

27. The lower water cement ratio in concrete, introduces

a. greater density and smaller permeability

b. improved frost resistance

c. greater wear resistance and improved bond strength

d. all the above.

28. C.R.R.I. charts are used to obtain a relatioship between strength of concrete and

a. water cement ratio

b. workability

c. grading of aggregate

d. fineness modulus

29. Normal values of overflow rate for sedimentation tanks using coagulants in litres/hr/m2 , generally range between

a. 250 to 500

b. 500 to 750

c. 750 to 1000

d. 1000 to 1250

30. According to IS : 1172-1963, a minimum of 135 litres of water capita per day, is required for

a. Boarding schools

b. Nurses home and medical quarters

c. hostels

d. all the above.

31. The pH value of fresh sewage is usually

a. equal to 7

b. more than 7

c. less than 7

d. equal to zero.

32. The sewers

a. must be of adequate size to avoid over flow

b. must flow under gravity  to  full

c. must be laid at least 2 to 3 m deep to collect /water from the basements

d. all the above.

33. According to Lacey, depth of scour in a river depends upon the straightness of the reach. If D is the depth of scour in regime flow in a right angled bend, it is

a. 1.25 D

b. 1.50 D

c. 1.75 D

d. 2.00 D

34. The ratio of the rate of change of discharge of an outlet and parent channel, is known as

a. efficiency

b. sensitivity

c. flexibility

d. modular limit.

35. While deciding the alignment of a proposed canal taking off from a river at B, three alignments of approximately equal lengths are available. These cross a drainages at C1, C2 and C3 where drainage bed levels C1 > C2 > C3, you will provide at site C3

a. an aqueduct

b. a syphon aqueduct

c. a super passage

d. a syphon.

36. A land is said to be water-logged if its soil pores within

a. a depth of 40 cm are saturated

b. a depth of 50 cm are saturated

c. root zone of the crops are saturated

d. all the above.

37. On most smooth hard surfaced roads, rolling resistance to moving vehicles, ranges from

a. 7 kg to 9 kg/tonne

b. 9 kg to 11 kg/tonne

c. 11 kg to 13 kg/tonne

d. none of these.

38. Passing zones are generally not provided on

a. summit curves

b. horizontal curves

c. two lane highways

d. all the above.

39. Pick up the correct statement from the following:

a. Detailed survey is carried out for a strip of land about 30 m at sharp curves

b. Levels are taken along the trace cut at an interval of 20 m

c. Contour interval is generally adopted at 2 metres vertical interval

d. All the above.

40. The desirable camber for straight cement concrete roads, is

a. 1 in 40 to 1 in 33

b. 1 in 150 to 1 in 140

c. 1 in 160 to 1 in 140

d. none of these.

41. The weight of an item is measured correct to nearest

a. 0.50 kg

b. 0.75 kg

c. 1.00 kg

d. 5 kg

42. The measurement is made for stone work in square metre in case of

a. Columns, lintels, copings

b. Building work

c. Dressed stones in Chajjas

d. (a) and (d) of the above.

43. The following item of earth work is not measured separately.

a. Site clearence

b. dead men

c. Steps in deep exavaction

d. All the above.

44. A portion of an embankment having a uniform up-gradient 1 in 500 is circular with radius 1000 m of the centre line. It subtends 180° at the centre. If the height of the bank is 1 m at the lower end, and side slopes 2:1, the earth work involved.

a. 26, 000 m3

b. 26, 500 m3

c. 27, 000 m3

d. 27, 500 m3

45. Railway projects are treated as

a. light construction

b. heavy construction

c. industrial construction

d. none of these.

46. A goiden rule for the procurement of construction stones, suggests

a. 67% at the site and 33% under procurement

b. 50% at the site and 50% under procurement

c. 33% at the site and 67% under procurement

d. none of these.

47. For completion of a project, the critical path of the network represents

a. minimum time

b. maximum time

c. maximum cost

d. minimum cost.

48. Pick up the correct statement from the following:

a. Pessimistic time estimate refers to activities

b. Most likely time estimate refers to activities

c. Expected time estimate refers to activities

d. All the above.

49. Pick up the correct statement from the following:

a. The basic length of a runway is increased at a rate of 7% per 300 m of elevation of M.S.L.

b. The standard temperature at the site is obtained by reducing the standard sea level temperature of 15°C at the rate of 6.5°C per 1000 m rise in elevation

c. The aerodrome reference temperature is the monthly mean of the mean daily temperature for the hottest month of the year

d. All the above.

50. In instrument landing system

a. L.O.M. and L.M.M. are installed on one side and Localizer antenna on the other side

b. L.O.M. and Localizer are installed on one side and L.M.M. on the other side

c. Localizer and L.M.M. are installed on one side and L.O.M. on the other side

d. None of these.