***Sub Engineer***

1. The accuracy of measurement in chain surveying, does not depend upon

a. length of the offset

b. scale of the plotting

c. importance of the features

d. general layout of the chain lines.

2. The zero of the graduated circle of a prismatic compass is located at

a. north end

b. east end

c. south end

d. west end.

3.  is a conventional sign of

a. mosque

b. temple

c. church

d. idgah.

4. The rise and fall method of reduction of levels, provides a check on

a. back sights

b. fore sights

c. intermediate sights

d. all of these.

5. Pegmatite is a/an

a. intrusive igneous rock

b. extrusive igneous rock

c. sedimentary rock

d. metamorphic rock.

6. In a mortar, the binding material is

a. cement

b. sand

c. surkhi

d. cinder.

7. The minimum compressive strength of Ilnd class bricks should be

a. 75 kg/cm2

b. 90 kg/cm2

c. 100 kg/cm2

d. 120 kg/cm2

8. The rock generally used for roofing, is

a. granite

b. basalt

c. slate

d. pumice.

9. At yield point of a test piece, the material

a. obeys Hooke's law

b. behaves in an elastic manner

c. regains its original shape on removal of the load

d. undergoes plastic deformation.

10. A simply supported beam which carries a uniformly distributed load has two equal overhangs. To have maximum B.M. produced in the beam least possible, the ratio of the length of the overhang to the total length of the beam, is

a. 0.207

b. 0.307

c. 0.407

d. 0.508

11. Along the principal plan subjected to maximum principal stress

a. maximum shear stress acts

b. minimum shear stress acts

c. no shear stress acts

d. none of these.

12. A simply supported wooden beam 150 cm long and having a cross section 16 cm x 24 cm carries a concentrated load, at the centre. If the permissible stress ft = 75 kg/cm2 and fs = 10 kg/cm2 the safe load is

a. 3025 kg

b. 3050 kg

c. 3075 kg

d. 3100 kg.

13. The unit of the viscosity is

a. kg sec/m2

b. Newton sec per m2

c. Newton-sec2/m3

d. m2 per sec.

14. For a most economical rectangular channel, the hydraulic mean depth, is equal to

a. the depth of flow

b. half the depth of flow

c. one-third depth of flow

d. None of these.

15. The discharge through a 100 mm diameter external mouth piece fitted to the side of a large vessel is 0.05948 m3/s. The head over the mouth piece is

a. 2 m

b. 2.5 m

c. 3.0 m

d. 4.0 m.

16. An independent mass of a fluid does not posses

a. elevation energy

b. kinetic energy

c. pressure energy

d. none of these.

17. Through a point in a loaded soil mass, there exists n typical planes mutually orthogonal on which the stress is wholly normal and no shear stress acts, if n is

a. 1

b. 2

c. 3

d. 4

18. Pick up the correct statement from the following:

a. When water table is above the base of a footing, the dry weight m should be used for soil below water table

b. When water table is located somewhat below the base of a footing, the elastic wedge is partly of moist soil and partly of submerged soil, and a suitable reduction factor is used

c. When water table is just at the base of the footing, no reduction factor is used

d. None of these.

19. A soil sample of mass specific gravity 1.92, has a moisture content 30%. If the specific gravity of solids is 2.75, the void ratio, is

a. 0.858

b. 0.860

c. 0.862

d. 0.864

20. The density of soil can be increased

a. by reducing the space occupied by air

b. by elastic compression of soil grains

c. by expelling water from pores

d. All the above.

21. The weight of reinforced concrete, is generally taken as

a. 2300 kg/m3

b. 2400 kg/m3

c. 2500 kg/m3

d. 2800 kg/m3

22. Lapped splices in tensile reinforcement are generally not used for bars of size larger than

a. 24 mm diameter

b. 30 mm diameter

c. 36 mm diameter

d. 32 mm diameter

23. The toe projection of foundation slabs is taken

a. as one third of the base

b. as one sixth of overall height of the wall

c. equal to heel slab

d. below ground surface.

24. Distribution reinforcement in a simply supported slab, is provided to distribute

a. load

b. temperature stress

c. shrinkage stress

d. all the above.

25. The inner section of a cavity wall, is generally known as

a. butress

b. leaf wall

c. pillaster

d. pillar.

26. The alignment of a cross joint along the plumb line is

a. bed block

b. perpend

c. lintel

d. vertical line.

27. Specified compressive strengh of concrete is obtained from cube tests at the end of

a. 7 days

b. 14 days

c. 21 days

d. 28 days.

28. Gypsum is added for

a. colour

b. strength

c. controlling setting time

d. none of these.

29. Suction lift of a pump depends upon

a. water temperature

b. velocity of water in suction pipe

c. frictional losses

d. all the above.

30. The nitrate concentration in domestic water supplies, is generally limited to

a. 15 ppm

b. 25 ppm

c. 30 ppm

d. 45 ppm.

31. Which one of the following tests is used for testing sewer pipes :

a. water test

b. ball test

c. mirror test

d. all of these.

32. The first stage of neutral process of sludge digestion, is

a. acid fermentation

b. acid regression

c. alkaline fermentation

d. none of these.

33. The ratio of the discharge over a trapezoidal crest to a rectangular crest of Sarda falls of identical parameters, is

a. 1.084

b. 1.074

c. 1.064

d. 1.054

34. The length and width of a meander and also the width of the river, vary roughly as

a. square root of the discharge

b. discharge

c. square of the discharge

d. cube of the discharge.

35. The Lacey's regime velocity is proportional to

a. R1/2 S3/4

b. Q3/4 S1/3

c. R3/4 S1/3

d. R2/3 S1/2

36. Lacey's equation for calculating flood discharge in rivers, is

a. V = 10.8 R1/2 S1/2

b. V = 10.8 R2/3 S1/2

c. V = 10.8 R2/3 S1/3

d. V = 10.8 R1/3 S2/3

37. In case of cement concrete pavements, pick up the incorrect statement

a. initial cost of construction is high

b. initial cost of construction is low

c. visibility during nights is high

d. none of these.

38. The statement, regarding the size of stones used at the wearing surface. "The size of a stone used on a road must be in due proportion to the space occupied by a wheel of ordinary dimensions on a smooth level surface. The point of contact will be found to be longitudinal about 2.5 cm and every piece of stone put into the road which exceeds 2.5 cm in any of its dimension is mischievious" was made by

a. Telford

b. Macadam

c. Tresguet

d. Sully.

39. Side drains on both sides of a hill road, are essential when the road is

a. along the spur curves

b. along the re-entrant curves

c. in cutting

d. none of these.

40. The best compromise between the increase of the length of a highway and reduction in its load carrying capacity, is the ruling gradient

a. 1 in 15

b. 1 in 20

c. 1 in 25

d. l in 30

41. The following item of earth work is not measured separately.

a. Site clearence

b. dead men

c. Steps in deep exavaction

d. All the above.

42. Pick up the incorrect statement from the following:

a. Lead is the average horizontal straight distance between the borrow pit and the place of spreading soil

b. The lead is calculated for each block of the excavated area

c. The unit of lead is 50 m for a distance upto 500 m

d. The unit of lead is 1 km where the lead exceeds 2 km.

43. The minimum width of a septic tank is taken

a. 70 cm

b. 75 cm

c. 80 cm

d. 90 cm

44. The reduced levels of points, 30 metres apart along the longitudinal section of a road portion between chainages 5 and 9 are shown in the given figure. If there is a uniform up-gradient of the road 120 in 1, the chainage of the point with no filling or cutting is

a. (6 + 15) chains

b. (6 + 12) chains

c. (6 + 18) chains

d. None of these.

45. If t is the duration of an activity, t1 is the latest finish possible moment of its preceding activity and t2 is the earliest start possible moment, the independent float of the activity is

a. (t1 - t2) - t

b. t - (t1 - t2)

c. (t1 + t2) - t

d. t + (t1 - t2)

46. The most popular type of organisation used for Civil Engineering Constructions, is

a. line organisation

b. line and staff organisation

c. functional organisation

d. effective organisation.

47. The first stage of a construction, is

a. survey of the site

b. initiation of proposal

c. preparation of tender

d. allotment of funds.

48. Pre-tender stage requires

a. selection of site

b. formalisation of alignment of work

c. formalisation of designs and preparation of estimate

d. all the above.

49. Pick up the incorrect statement from the following:

a. Channelization of pavement is caused by constant use of tri-cycle gears of aircraft

b. Blast pads are used over cohesionless soils to resist erosion due to tremendous speed of the jet blast

c. Over-run areas of at least 300 m length on either side of the runway are provided

d. None of these.

50. Two single runways may be arranged so as to have

a. L-shape

b. T-shape

c. X-shape

d. All the above.